December 2009 District Office Summaries for Land Reclamation, ABS BF Discharges and Potential ABS Discharges

ABS BF Land Reclamation

Cambria DMO

Permit no. (BF Date):  56960111 (2/27/01)

Big J Mining - Sorber Mine

Available Collected Bond:  $116,877 (hold for discharge)

Land Reclamation Estimate:  $409,105 (Conventional Bond)

Supplemental ABS $ needed:  $409,105

Status:  A Sub F Remine permit application with a contiguous Act 181 reclamation proposal received from Coal Loaders, Inc-the contract was awarded in October 2009.  The permit is close to being issued-the bond has been requested.  The collected bond was transferred to the Primacy ABS Site Legacy account.

************************************************************************ Greensburg DMO

Permit no. (BF Date):  26840202 (3/15/88)

Grandstone Coal Co – Grandstone Site

Available Collected Bond:  $29,716

Land Reclamation Estimate:  $48,124.42

Supplemental ABS $ needed:  $18,408

Status:  The reclamation cost estimate is based upon only the area affected/disturbed under Primacy.  90% chance that this site will be mined in future for fuel for the proposed Nemacolin power plant.  Work began on an Act 181 (Landowner Reclamation) project in the Fall of 2009.

***********************************************************************

Permit no. (BF Date):  26931601 (11/1/96)

Global Coal Recovery Inc – Isabella Site

Available Collected Bond:  $1,194,780 (being held for discharge)

Land Reclamation Estimate:  $1,200,000 (Conventional Bond)

Supplemental ABS $ needed:  $1,200,000

Status:  Reclamation required and discharge exists.  $1,194,780 of collected bond was transferred to the Primacy ABS Site Legacy account.  Mining of the refuse pile is dependent on the Nemacolin power plant being built.

**********************************************************************

Permit no. (BF Date):  63793203 (12/18/02)

Empire Fuel Corp. – Cherry Refuse Pile

Available Collected Bond:  $1,600

Land Reclamation Estimate:  $9,635

Supplemental ABS $ needed:  $8,035

Status:  Reclamation required.  Refuse pile on one side of the road will require removal of pond (pond release letter?) and revegetation of affected area (could be done in conjunction with a BAMR project).  Refuse pile on the other side of the road was not disturbed and is eligible for Title IV funding.  It is difficult to reach the landowner.

************************************************************************

Permit no. (BF Date):  65860201 (8/25/89)

Aspenergy Inc – Wilpen Operation, SOAP #248

Available Collected Bond:  $27,390

Land Reclamation Estimate:  $396,000 (Conventional Bond)

Supplemental ABS $ needed:  $368,610

Status:  Reclamation required.  About two-thirds of the pile was affected.  The remaining area is eligible for Title IV.  There are documented landowner issues which may prevent the reclamation from proceeding soon.

***********************************************************************

Permit no. (BF Date):  65920108 (5/3/00)

Bituminous Proc. Co Inc – Wyano Mine

Available Collected Bond:  $71,160

Land Reclamation Estimate:  $84,827.97 (Conventional Bond)

Supplemental ABS $ needed:  $13,667.18

Status:  Reclamation required.  This is a combined refuse and surface mine.  The property owner is the permittee.  $8,777 of the bond was spent on the project for mapping.  A contract has been signed.  Reclamation is due to be completed in the Fall of 2010.

************************************************************************

Permit no. (BF Date):  65920201 (9/25/96)

Ebony Coal Co – Claridge Sub F Refuse pile

Available Collected Bond:  $13,000

Land Reclamation Estimate:  (see comments below)

Supplemental ABS $ needed:  $0

Status:  Projects pending.  This is a coal refuse reprocessing permit.  BAMR is currently designing a reclamation project for this site and adjacent AML lands.  Project estimate is $250,000 which will use the $13,000 bond money and will be supplemented with AML Title IV funds.  No ABS Bond Deficit funding is anticipated.  Anticipated bid date is 3/30/10.

************************************************************************

The Knox DMO has completed all ABS Land Reclamation within their district

************************************************************************

Moshannon DMO

*******************************************************************

Permit no. (BF Date):  14663004 (6/23/00)

Power Opr Co Inc - Dugan 4 Mine

Available Collected Bond:  $394, 420 (held for discharge)

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status: Reclamation to be completed by repermittee but AMD discharge exists.  Minimum $10,000 bond posted on new permit.  Awaiting posting of the full-cost bond by the successor operator, however, that is tied to the completion of the Karthaus cogen plant.  Remaining collected bond was transferred to Primacy ABS Legacy Site Trust Account.

*********************************************************************

Permit no. (BF Date):  17673057 (6/23/00)

Power Opr Co Inc – Rosemary Mine

Available Collected Bond:  $162,700 (collection of the other bond to be waived upon completion of the reclamation under the CO & A)

Land Reclamation Estimate:   $0

Supplemental ABS $ needed:  $0

Status:  All remaining reclamation will be completed under a CO& A. with Jr. Coal.  --The work is done, but a lingering landowner (land swap) issue remains-awaiting property transfer documents from the courthouse.  The Rockwood Surety bond ($180,700) was forfeited and collected.  Some of that bond was spent.  The remaining $162,700 of the collected Rockwood bond will be transferred to the SCMRA Fund – Released Bond account.

***********************************************************************

Permit no. (BF Date):  17793121 (12/19/88)

RL Maney Coal Co – Maney mine

Available Collected Bond: $63,796

Land Reclamation Est:  $0

Supplemental ABS $ needed: $0

Status:  Partially repermitted by Sky Haven.  Remaining reclamation by Sky Haven via Bond Credit CO&A.  Reclamation value $335,968.00.  Completion due date is 10/31/2010.  Should be substantially completed by 12/31/09.  Work is on schedule.  RES Coal LLC succeeded Sky Haven and will complete the reclamation.  Excess bond of $63, 796 will be transferred to SMCRA Fund – Released Bond account.

**********************************************************************

Permit no. (BF Date):  17803034 (10/8/93)

Cambria Mills Coal Co Inc – Lyncroft mine

Available Collected Bond: $656,937

Land Reclamation Estimate:  $850,597

Supplemental ABS $ needed:  $193,660

Status:  Reclamation work in progress.  Notice to Proceed with Act 181 contract for issued 12/19/08.  Work began 3/12/09.  All backfilling and regrading work complete as of 6/17/09.  Planting (ground cover) completed 9/14/09.  Tree planting deferred until May 2010.

***********************************************************************Permit no. (BF Date):  17803139 (7/15/87)

Glen Irvan Corp. – Ogden #2

Available Collected Bond:  0

Land Reclamation Estimate:  0

Supplemental ABS $ needed:  0

Status:  Reclamation completed.  Land was reclaimed by BAMR, but impacted water supplies remain.  BAMR will be extending water supply line to homes (in 2011).  Department will pay for tap into line for one of the water supplies.  Cost estimates are not known at this time.  Cost will most likely be taken from the Released Bond account since this type of expenditure does not qualify for ABS Bond Deficit Close-Out funds.

*********************************************************************

Permit no. (BF Date):  17804004 (9/18/89)

Benjamin Coal Co – Ecklund mine

Available Collected Bond:  $152,700

Land Reclamation Estimate:  $33,600

Supplemental ABS $ needed:  $0

Status:  Reclamation required.  Waiting on Act 181 proposal from adjacent operator-to be done in conjunction with the Johnson-Sinclair.  Met with operator’s engineer on 9/9/09.  Cost estimates are being finalized.  Engineer’s estimate:  $33,600.  There may be excess bond of $119,100.

************************************************************************

Permit no. (BF Date):  17820114 (9/26/00)

Power Op Co – Vought mine

Available Collected Bond: $203,306 (hold for discharge)

Land Reclamation Estimate:  $1,056,100 (conventional bond calculation)

Supplemental ABS $ needed:  $1,056,100.

Status:  Reclamation required.  Possible remining by Jr. Coal (application submitted 6/08, correction letter sent 11/08, second correction letter sent 6/12/09).  $500,000 GGII fund remining incentive to purchase high rate alkaline addition lime to address ABS MDI discharge approved by Governor’s Office 8/27/08,  Sole Source Grant approval complete 12/08.  Grant Center contract approval 3/2/09.  Sent to Junior for signature 3/17/09.  Bond credit funding has been reserved.  Collected bond was transferred to the ABS Legacy Site Trust Account for the discharge.

************************************************************************

Permit no. (BF Date):  17840105 (9/18/89)

Benjamin Coal – Johnston-Sinclair

Available Collected Bond: $72,500

Land Reclamation Estimate:  $300,000 (Conventional Bond)

Supplemental ABS $ needed:  $227,500

Status:  Projects pending.  Forcey Coal Co. remining permit application on part of site, interested in Act 181, in conjunction with Ecklund described above.  District is assessing reclamation needs and researching landowners.  Act 181 negotiations began in Spring 08.  Further investigation has indicated that the project may cost $300,000 (rather than the $40,000 previously reported).

**********************************************************************

Permit no. (BF Date): 17841604 (6/22/00)

Power Op Co – Leslie Tipple

Available Collected Bond:  $189,817

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status:  Jr. Coal was issued permit number 17051601 on August 13, 2008.  The permit was accompanied by 2 CO & As-one for Sub-F like protection and another allowing for less bond to be posted by Jr. (for a period of five years) based on the collection of Power’s bond.  There are also two GFCC’s to remove the coal refuse on site.  Potential excess bond of $189,817, after permit renewal when Jr. Coal will post the full bond in August 2013.

*********************************************************************

Permit no. (BF Date):  17841605 (9/18/89)

Benjamin Coal Co - 3 Prep Plant

Available Collected Bond: $54,004

Land Reclamation Estimate:  $288,665 (Engineering Cost Estimate)

Supplemental ABS $ needed:  $234,661

Status:  Reclamation required.  Twelve electrical transformers were removed from the site via Waste Management contract.  $7,671.52 from bond was spent.  The revised technical specifications, scope of work, mapping and landowner release letters are being prepared.  Landowner letters for alternate AOC and pond release sent July 2007, with no response.

**********************************************************************

Permit no. (BF Date):  17921605 (4/4/01)

Thomas Coal Sales Inc - Greenwood Tipple

Available Collected Bond: $92,785 (being held for discharge)

Land Reclamation Estimate:  $299,625 (Engineering Cost Estimate)

Supplemental ABS $ needed:  $299,625

Status:  Reclamation required and discharge exists.  Permit application received 9/30/08 from Victor, LLC, Victor Tipple.  The applicant withdrew the application on 5/23/09 due to the poor coal market.  They plan to resubmit in the future.  Note – because of the discharge, this would be a Sub F permit.  Collected bond was transferred to the ABS Legacy Site Trust Account.

*********************************************************************

Permit no. (BF Date):  17930115 (4/18/06)

K&J Coal Co – Beaver #1

Available Collected Bond:  $0

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status: Projects pending. Repermitted under River Hill SMP 17020103 Thompson site. Also being reclaimed under River Hill Reclamation CO&A.  Due date was 9/30/08, third time extension pending, backfilling 9/15/09, trees 4/20/10, pond removal 9/30/10 includes $7500 civil penalty and stipulated penalties of $30,000 for future missed deadlines.

*********************************************************************

Permit no. (BF Date):  17960121 (4/18/06)

K&J Coal Co – Beaver #2

Available Collected Bond:  $0

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status: Projects pending. Repermitted under River Hill SMP 17020103 Thompson site. Also being reclaimed under River Hill Reclamation CO&A.  Due date was 9/30/08, third time extension pending, backfilling 9/15/09, trees 4/20/10, pond removal 9/30/10 included $7500 civil penalty and stipulated penalties of $30,000 for future missed deadlines.

************************************************************************

Permit no. (BF Date):  57813001 (12/18/02)

Bernice Mining – Bliss Mine

Available Collected Bond: $260,100

Land Reclamation Estimate:  $719,417.77 (Approved contract cost)

Supplemental ABS $ needed:  $459,317.87

Status:  Conventional Bond Estimate is $1,063,038.  Reclamation started 11/12/09.  Scheduled completion date is 9/20/10.

***********************************************************************

Permit no. (BF Date):  57830101 (1/2/02)

Bernice Mining – Lewis Mine

Available Collected Bond:  $192, 181 (held for discharge)

Land Reclamation Estimate:  $364,786 (Act 181 bid)

Supplemental ABS $ needed:  $364,786

Status:  Final grading completed 6/09.  Reclamation completed.  Warranty period expires 9/15/10.

***********************************************************************

Pottsville DMO

************************************************************************

Permit no. (BF Date):  35850201 (5/26/92)

American Silt Processing Co – American Bank #1 mine

Available Collected Bond:  $12,000

Land Reclamation Estimate:  $46,522 (Conventional Bond)

Supplemental ABS $ needed:  $34,522

Status:  This is a coal refuse reprocessing permit.  This site will be included in a BAMR Title IV reclamation project.  Anticipated bid date is March 2010.

************************************************************************

Permit no. (BF Date):  40920101 (12/16/94)

Laurel Run Corp - Laurel Run Mine

Available Collected Bond:  $29,800

Land Reclamation Estimate:  $43,748 (Conventional Bond)

Supplemental ABS $ needed:  $13,948

Status:  A site visit was conducted 3/12/09.  Access to the site is currently being evaluated since the mine operator’s access consisted of a bridge that has washed out and no longer exists.  A detailed engineering assessment will be made to develop actual reclamation costs.  Exploring the option of having the BAMR AD crew complete the work.

*********************************************************************

Permit no. (BF Date):  49851605 (2/22/07)

Twin Creek Coal Inc – Swift Breaker Mine

Available Collected Bond:  $40,973.23

Land Reclamation Estimate:  $40,973.23

Supplemental ABS $ needed:  $0

Status:  Project was partially reclaimed under a CO&A.  As of June 2008, the breaker had been removed.  The operator did not fulfill the remainder of the reclamation obligation.  Working to contract for the remaining work (using the collected bond) via Act 181 project.

*******************************************************************

Permit no. (BF Date):  49860201 (11/2/05)

Rosini Coal Co, Inc – Rosini Mine

Available Collected Bond:  $0

Land Reclamation Estimate:  $313,350 (Conventional Bond)

Supplemental ABS $ needed:  $245,031

Status:  Permit Transfer in Process.  Rosini Enterprises submitted a transfer of permit application on 3/2/09.  The permit has gone through the review process and a bond request was sent June 12, 2009.   Bond (LC) was approved 9/30/09.  All features requiring reclamation will be addressed through Special Conditions in the new permit.  In fact, the demolition of the breaker has been substantially completed.  Surety bond ($32,310) not collected yet.  Site also had $36,009 in conversion assistance.  If transfer occurs, and the bond is collected, then it may be transferred to the Released Bond Account.  Complications (relating to waste disposal by the landowner) have come up relating to the pond across the road from the breaker area.

***********************************************************************

Permit no. (BF Date):  54813040 (2/17/04)

Sunray Coal Inc – Hammond Mine

Available Collected Bond:  $0

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status:  To be reclaimed under an SMP with Keystone Anthracite.  Keystone will be submitting an application that will encompass the Sunray permit.  A CO&A with the surety company is near completion to settle penalties and bonding issues.  Conventional bond cost estimate is $252,226.

***********************************************************************

Permit no. (BF Date):  54813225 (8/13/98)

Shamrock Coal Co Inc – Reber 4 Mine

Available Collected Bond:  $57,729

Land Reclamation Estimate:  $146,518 (Conventional Bond)

Supplemental ABS $ needed:  $88,789

Status:  The intent is to approach this site in phases.  There is a pending grant agreement with the Schuylkill Headwaters Association to reclaim a portion of the site, Phase 1.  The property owner expressed an interest in repermitting most of the site which will be Phase 2.  Met with consultant December 15, 2008 to discuss reclamation options and repermitting interests.  May only need the available collected bond of $57,729 to complete reclamation.

************************************************************************

Permit no. (BF Date):  54830110 (7/1/87)

J&W Coal Co – Lake Run 1 Mine

Available Collected Bond: $11,260

Land Reclamation Estimate:  $155,676

Supplemental ABS $ needed:  $144,416

Status:  Land ownership recently changed and the new landowner expressed possible mining interest.  He requested that no reclamation be done until prospecting of a potential mining operation is conducted.  The feature requiring reclamation is only a small portion of the overall abandoned mine features on the property.

********************************************************************

Permit no. (BF Date):  54850203 (8/14/95)

Tamaqua Anthracite – Bell Colliery Bank

Available Collected Bond: $11,900

Land Reclamation Estimate:  $54,500 (Grant Request Amount)

Supplemental ABS $ needed:  $42,600

Status:  There is a total of approximately 13 acres in need of minor grading and planting. Grant agreement with Schuykill Headwaters Association to complete reclamation.  Grant documents are circulating for approval.

*********************************************************************

Permit no. (BF Date):  54970105 (3/5/04)

ANTHRACO LTD – Primrose Mine

Available Collected Bond:  $0

Land Reclamation Estimate:  $0

Supplemental ABS $ needed:  $0

Status:  Area was repermitted to Blaschak Coal under permit no. 54060101 on 8/24/06.  The area where reclamation is needed is included in the operational area of this permit.  Pottsville DMO working on a CO&A for revegetation of a portion of the BF area to be completed by St. Clair & Associates.

***********************************************************************

Primacy ABS Bond Forfeiture Permits and Discharges

Moshannon DMO

Permittee			Power Operating Co

Permit no.(BF Date) 		14663003 (6/23/00)

Site Name			Dugan 2

Township County		Rush/Centre

Discharge id (SIS MP No)	PD’ (40953) & PO3-12 (40954)

Passive treatment system design was completed under contract with a cost of $40,000.  The two discharges will be treated together.  Notice to Proceed letter 9/19/07 to with a completion date of 7/19/2008.  Construction contract is under development.  Construction of facilities will be competitively bid with Dugan 4.  Treatment cost estimate:  Initial cost $297,810; O & M $861; Recap $67,977.  Bond of $344,593 was deposited in the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Power Operating Co

Permit no. (BF Date) 		14663004(6/23/00)

Site Name			Dugan 4

Township County		Rush/Centre

Discharge id (SIS MP No)	PA’ (40952)

Passive treatment system rehabilitation design with a cost of $19,000.  Design includes an existing Vertical Flow Pond (VFP) and a Manganese removal bed.  Construction contract is under development.  Treatment cost estimate:  Initial cost:  $194,362; O & M $10,359; Recap $85,186.  $394,420 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			R S Carlin Inc

Permit no. (BF Date) 		14663010 (4/18/06)

Site Name			Mine 26

Township County		Rush/Centre

Discharge id (SIS MP No)	TSN0 (85603)

This discharge is a lower priority to treat because there is a large deep mine discharge upstream that degrades the stream.  An alternate surety reclamation project was completed under Travelers Surety Reclamation COA # 074060 dated 11/21/07.  Treatment cost estimate (based on caustic and pond):  Initial cost $86,120; O & M $71,831; Recap $30,731.

************************************************************************

Permittee			Avery Coal Co Inc

Permit no. (BF Date) 		14803008 (6/30/93)

Site Name			Pine Glen

Township County		Burnside/Centre

Discharge id (SIS MP No)	SP3 (59998)

Discharge was conveyed to an already degraded stream (6 Mile Run) and away from a high quality stream (Sterling Run).  BAMR built a VFP on Sterling Run unrelated to this discharge.  SP3 is severe AMD to already degraded stream.  Initial cost $44,481; O & M $13,782; Recap $60,021.  Bond was spent.

************************************************************************

Permittee			Thompson Bros Coal Co

Permit no. (BF Date) 		17803054 (1/27/93)

Site Name			Alder Run

Township County		Boggs/Clearfield

Discharge id (SIS MP No)	36 (40744) & 37 (40745)

BAMR completed surface reclamation and piped the discharges to the permit boundary.  Recommended consultant to design project 5/22/09 ($36,100).  Allocation request approved 6/8/09.  Landowner wants changes to Landowner Agreement and pond location before allowing construction.  Task agreement notice to proceed issued 10/23/09.  For 36 & 37 Combined, Initial cost $301,100; O & M $31,200; Recap $12,773.  $39,557 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Thompson Bros Coal Co

Permit no. (BF Date) 		17810154 (7/31/97)

Site Name			001 Strip

Township County		Morris/Clearfield

Discharge id (SIS MP No)	25 (7956)

Previous Growing Greener passive treatment design/permitting project to treat mine discharge S8-14(ER-13) (inactive MDI MP: 25) in the Emigh Run headwaters completed 11/07.  Act 181 project to install conveyance ditch to proposed treatment system is complete.

2008 GG construction grant award of $414,945 to Emigh Run Watershed Assoc includes this discharge and others.  Additional funds ($40,000 from the 290 fund) used to supplement shortfall of OSM match because the primacy discharges are ineligible for federal money.  However, pre-primacy discharges from MDP 3268BSM31 are eligible.  The grant agreement is working its way through the approval process.  One setback was the need for updated landowner access forms.  Initial cost $154,858; O & M $12,021; Recap $83,531.  Bond in the amount of $4,560 was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Chews Contracting

Permit no. (BF Date) 		17813143 (4/20/90)

Site Name			Little D

Township County		Boggs/Clearfield

Discharge id (SIS MP No)	LD1(100205)

Not treated.  Discharge comes from a pipe from area reclaimed by BAMR.  Morgan Run Watershed Association submitted 2009 GG application for design, permitting, and construction of a passive treatment system.  Initial site visit on 5/12/09.  The grant request was ranked in December 2009.  Initial cost $8,185; O & M $3,166; Recap $2,517. Cost based on ALD and settling pond.  $16,040 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Power Operating Co Inc

Permit no. (BF Date) 		17820114 (9/26/00)

Site Name			Vought

Township County		Decatur/Clearfield

Discharge id (SIS MP No)	9(13052)

Remining permit (17080107) to include Sub-F protection submitted in June 2008 by Jr. Coal using bond credits (will address the land reclamation), correction letter sent 11/08, second correction letter sent 6/12/09.  Bond Credit Agreement (from Vought and Rosemary bonds) to reclaim old highwall/spoil is ready.  $500,000 GGII fund remining incentive to purchase high rate alkaline addition lime to address ABS MDI discharge approved by Governor 8/27/08, Sole Source Grant approval complete 12/08, Grant Center contract approval 3/2/09, sent to Junior for signature 3/17/09.  Abandoned chemical treatment system was providing some residual treatment.  Cost based on caustic and pond system.  Initial cost $14,093; O & M $27,815; Recap $12,773.  Discharge is low flow.  It comes from on old drift through the crop in an area that has been backfilled.  Remining alone is not likely to improve the discharge-however copious alkaline addition is expected to improve the water quality.  $203,307 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Benjamin Coal Co, Inc

Permit no. (BF Date) 		17820132 (1/10/90 & 3/6/95)

Site Name			Little Beaver 1

Township County		Decatur/Clearfield

Discharge id (SIS MP No)	RLB2 (89523)

Near this discharge there is severe AMD that needs chemical treatment to already degraded stream.  Initial cost $14,093; O & M $26,320; Recap $12,773.  $242,478 was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Avery Coal Co

Permit no. (BF Date) 		17820143 (6/30/93)

Site Name			Victoria

Township County		Boggs/Clearfield

Discharge id (SIS MP No)	MP8 (100203) & MP9 (100204)

2008 GG I Grant for $47,465 (Dimeling Discharge AMD Design Project Grant # MO080049), to design a passive treatment facility awarded to Clearfield County Conservation District.  Conceptual design, surveying and wetland delineation have been completed.  Permitting in progress.  For MP8, there is natural iron removal before entering the stream.  MP9 was reported as “dry” (most recently in April 2009) and will be monitored.  Assessment and design scheduled for 2010, construction 2011.  Combined treatment:  Initial cost $34,914; O & M $29,537; Recap $10,630.  $136,976 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Benjamin Coal Co

Permit no. (BF Date) 		17860105 (1/10/90)

Site Name			Marshall

Township County		Brady/Clearfield

Discharge id (SIS MP No)	MP3 (104484) & MP4 (104483)

This discharge is base flow to a stream.  The discharge could be passively treated (quality-wise), but it emerges at the edge of the stream.  Cost estimate based on wetland and settling pond.  Initial cost $37,233; O & M $3,430; Recap $30,455.  $100,100 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Thomas Coal Sales

Permit no. (BF Date) 		17921605 (4/4/01)

Site Name			Greenwood Tipple

Township County		Greenwood/Clearfield

Discharge id (SIS MP No)	1S (60393) and 2 (57270)

Permit application received 9/30/08 from Victor, LLC, Victor Tipple. The applicant withdrew the application on 5/23/09 due to the poor coal market.  They plan to resubmit in the future.  The discharges would be handled as Subchapter F points for a new permittee.  For 1S:  Initial cost $20,768; O & M $3,281; Recap $14,532.  For 2:  Initial cost $14,093; O & M $26,193; Recap $12,773.  $92,784 in bond was transferred to the ABS Legacy Sites Trust Account.

************************************************************************

Permittee			Ed Hanslovan Coal Co, Inc

Permit no. (BF Date) 		18840101 (7/1/05)

Site Name			Keating 1

Township County		West Keating/Clinton

Discharge id (SIS MP No)	K5(13141), K8(40782), K14(13137), K6(13142), K29 (13140)

K5 has a functioning passive treatment system (VFP) in place.  K8, K6 and K14 are treated together-this system (VFP) is also functional.  VFP needs $40,000 in embankment/plumbing repairs.  K29 is treated via an ALD-raw water no longer available.  Total for K8, K6 and K14:  Initial cost $56,505; O & M $3,038; Recap $65,812.  For K5:  Initial cost $40,463; O & M $2,476; Recap $27,726.  For K29:  Initial cost $2,236; O & M $78; Recap $3,090.  $75,880 in bond placed in trust (not fully funded).  DMO is doing O&M.  New trustee (Community Foundation of the Alleghenies) took over 12/1/09.

************************************************************************

Permittee			Lobb, Inc

Permit no. (BF Date) 		18860101 (3/22/96)

Site Name			Narco 1

Township County		Bald Eagle/Clinton

Discharge id (SIS MP No)	8 (9227)

This has a functioning passive treatment system.  ALD constructed by surety company as part of forfeiture CO&A, bypasses storm flows over 150 gpm.  Initial cost $20,151; O & M $3,635; Recap $20,026.  Bond collection was waived.  DMO is doing O&M.

************************************************************************

Permittee			Bernice Mining

Permit no. (BF Date) 		57830101 (1/2/02)

Site Name			Lewis

Township County		Cherry/Sullivan

Discharge id (SIS MP No)	102 (59444)

Passive treatment system (VFP) constructed-funded by Growing Greener II in 2007.  Initial cost $164,613; O & M $5,686; Recap $107,863.  $192,181 of collected bond remains.  DMO is doing O&M.

************************************************************************

Cambria DMO

Permittee			K & J Coal Co

Permit no. (BF Date) 		17880129 (4/29/04)

Site Name			Gaber Brown

Township County		Chest/Cambria

Discharge id (SIS MP No)	GB6 (51741), GB4 (51744)

GB4 is mixed with alkaline water from the GB6 pond and piped to a wetland where maintenance is minimal.  This site has a set of two VFPs that require quarterly inspection, flushing, sampling, and minimal other maintenance.  There is a trust fund in place partially funded (March ’09 about $750K).  Surety completed the land reclamation and facility construction (Actual cost to construct for this permit $756,400).  Treatment cost estimate:  O & M Cost calculated in combination with the Westover site listed below.  The Recap cost for GB6 is $599,512.  Total present value of the recap is $1,942,418.  O & M contract awarded ($17,440).

************************************************************************

Permittee			L & B Coal Co

Permit no. (BF Date) 		05830101 (8/6/99)

Site Name			No 5

Township County		Broadtop/Bedford

Discharge id (SIS MP No)	SP4A (12163)

The township is active in operating and maintaining facilities in the area.  The discharge is targeted for treatment by them.  There is a landowner hold up.  For caustic-- Initial cost:  $11,864; O & M $12,530; Recap $8,646.  Bond was transferred to the ABS Legacy Sites Trust Account ($85,487.00).

************************************************************************

Permittee			K & J Coal Co

Permit no. (BF Date) 		11693000 (8/22/03)

Site Name			Westover

Township County		Chest/Cambria

Discharge id (SIS MP No)	4 (58948), 23 (58947) & P-1 (98484)

Waiting for O & M contract to be finalized.  This site has three functioning VFPs.  They require quarterly inspection, flushing, sampling, and minimal maintenance.  Land reclamation and facility construction was completed by the Surety.  Treatment facilities for this permit cost about $1.3 M to build.  Treatment cost estimate:  O & M $17,573 Combined; Recap for 4, $599,512; Recap for 23, $143,883; Recap for P-1, $599,512.  There is a trust fund in place, including the Gaber Brown site.  O & M contract awarded ($21,380).

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		11783035 (7/18/05)

Site Name			Bell Woodcock/Blandburg Plant

Township County		Reade/Cambria

Discharge id (SIS MP No)	SBG32 (22596)

Treatment plant (caustic) was in place.  The in-stream quality is highly degraded by adjacent pre-act AMD.  Clearfield Creek Watershed Association has an interest in this site and is developing a GG grant application for a comprehensive treatment approach which includes the nearby pre-act AMD.  Treatment cost estimate:  Initial cost $33,853; O & M $183,578; Recap $26,914.  Partially funded trust.  O & M is included in CSF grant.  Site survey was completed at a cost of about $4,200.  The next step is to get a system design-the funding request was approved for this.

***********************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		11823002 (7/18/05)

Site Name			Stroud

Township County		Reade/Cambria

Discharge id (SIS MP No)	DEP-1(98486)

Waiting for O & M contract to be finalized.  Functioning passive treatment system (Mn bed and wetland).  The site requires little maintenance other than occasional stirring of the limestone Mn bed.  Clearfield Creek Watershed Association has an interest in this site.  Initial cost $14,401; O & M $1,655; Recap $38,585.  Partially funded trust.  Included in CSF grant proposal.  CSF has signed up an O & M contractor.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		11850106 (7/18/05)

Site Name			Cambria 51

Township County		Reade/Cambria

Discharge id (SIS MP No)	DER-1(22671) & DER-2 (22673)

A GG grant has been awarded to Clearfield Creek Watershed Association in order to rehabilitate this passive treatment system (VFPs, Mn bed and Wetland).  For caustic (combined):  Initial cost $16,748; O & M $76,953; Recap $12,206.  Partially funded trust.  Included in CSF grant proposal.  CSF has signed up an O & M contractor.

************************************************************************

Permittee			PA Energy Corp

Permit no. (BF Date) 		11813019 (3/21/86)

Site Name			Horse Hill

Township County		Lower Yoder/Cambria

Discharge id (SIS MP No)	MD1(88823)

MD1 discharge has acidity and 20 mg/l Al.  A large caustic treatment system operated by M.B. Energy is immediately below this discharge.  For caustic: Initial cost $13,664; O & M $19,423; Recap $9,958.  BAMR did surface reclamation.  O & M contract awarded ($26,272).  Design is pending.

***********************************************************************

Permittee			Acme Drilling

Permit no. (BF Date) 		32823035 (2/27/92)

Site Name			Broom

Township County		Burrell/Indiana

Discharge id (SIS MP No)	BS15 (89806)

Discharge impact cannot be measured due to low flow, distance to stream, and pre-act AMD.  PACD is planning to do the design.  For caustic: Initial cost $13,664; O & M $29,866; Recap $9,958.  Forfeited bond spent on land reclamation.

************************************************************************

Permittee			Delta Mining Inc.

Permit no. (BF Date) 		56763022 (1/28/93 & 2/5/96)

Site Name			Maust

Township County		Elk Lick/Somerset

Discharge id (SIS MP No)	RAW(47597) and OLDDH (59915)

Surety constructed Passive treatment system for RAW.  OLDDH bypassed the system.  The O & M needs are minimal.  This VFP system has functioned well since construction in 1998 with no flushing, and the first of two VFPs still produces 60 mg/l alkalinity.  For VFP/wetland:  RAW, Initial cost $ 117,940; O & M $3,370; Recap $302,343.  For OLDDH Initial Cost $52,344; O & M $2,088; Recap $120,553.  No bond left.  Construction completed to collect OLDDH.  O & M Contract awarded ($15,908).

************************************************************************

Permittee			D & E Construction Co

Permit no. (BF Date) 		56773084 (6/28/89)

Site Name			Moore

Township County		Summit/Somerset

Discharge id (SIS MP No)	PIPE (89089), SEEP (89090) & SEEP2 (89091)

There are no treatment facilities for these three seeps.  Space is available but the area is a cattail wetland and construction will be difficult at SEEP and unfeasible at the PIPE location.  For passive: PIPE:  Initial cost $80,425; O & M $2,131; Recap $161,307.  For SEEP:  Initial cost $30,328; O & M $1,757; Recap $86,492.00.  For SEEP2:  Initial cost $33,454; O & M $1,820; Recap $61,297.00.  The collected bond ($22,829.00) was transferred to the ABS Legacy Sites Trust Account.  The plan is to use the passive treatment design contractor list-funding allocation approved.  Funding was approved for the O & M contract.  Progress has been delayed while issues are worked out with the landowner.

************************************************************************

Permittee			H & H Coal Co

Permit no. (BF Date) 		56783046 (4/6/88)

Site Name			James E Long

Township County		Brothersvalley/Somerset

Discharge id (SIS MP No)	MD1 (82890)

Surety reclamation completed the land reclamation.  No treatment in place.  Two small sed ponds could be used for treatment.  Initial cost for Limestone Bed: $38,503; O & M $1,782; Recap $78,011.  Bond was waived for Surety reclamation.  This site is included in the pending O & M contract.  The plan is to use the passive treatment design contractor list-funding allocation approved.  Rehab project is in the contracting process.

************************************************************************

Permittee			Delta Mining Inc

Permit no. (BF Date) 		56793053 (10/14/92)

Site Name			Hay 2

Township County		Brothersvalley/Somerset

Discharge id (SIS MP No)	MD2 (88287) & MD3 (88288)

MD2 (2 gpm) will need some alkaline addition due to Fe of 100 mg/l.  Access will be difficult in the woods; collection along stream bank may be difficult.  For passive (wetland): MD2:  Initial cost $20,113; O & M $1,679; Recap $44,908.  For MD3:  Initial cost $8,928; O & M $1,552; Recap $12,003.  The collected bond ($86,380) was transferred to the ABS Legacy Sites Trust Account.  The plan is to use the passive treatment design contractor list-funding allocation approved.  This site is included in the pending O & M contract.  Treatment system design is underway.

************************************************************************

Permittee			Windber High Standard Coal Co

Permit no. (BF Date) 		56793078 (5/11/94 & 9/26/00)

Site Name			Brant

Township County		Stonycreek/Somerset

Discharge id (SIS MP No)	MD101 (34930)

Passive treatment system of five wetland ponds never worked well and caustic was added.  For caustic option:  Initial cost $13,664; O & M $20,453; Recap $11,950.  $109,300 in bond collected.  Treatment system design is completed.  This site is included in the pending O & M contract.  Rehabilitation work is in the contracting process.

************************************************************************

Permittee			Delta Mining Inc

Permit no. (BF Date) 		56803014 (5/12/92)

Site Name			Bashore

Township County		Summit/Somerset

Discharge id (SIS MP No)	NSEEP (89086) & NNSEEP (89085)

BAMR did surface reclamation.  Existing ponds could be used as part of a treatment system.  Discharges flow through old treatment ponds.  Lead waste was disposed of on site.  For caustic option: NSEEP, Initial cost $13,664; O & M $31,659; Recap $9,958.  For NNSEEP, Initial cost $13,664; O & M $30,159; Recap $9,958.  $326,507.00 in bond was transferred to the ABS Legacy Sites Trust Account.  OSM funds approved for this site.  The plan is to use the active treatment design contractor list-funding allocation approved.  System design is being worked on by consultant.

************************************************************************

Permittee			L & L Mining Inc

Permit no. (BF Date) 		56813054 (6/23/87)

Site Name			Berkey

Township County		Somerset/Somerset

Discharge id (SIS MP No)	SP30 (87383) & SP31 (87484)

Existing facility needs some work.  Landowner issues have resulted in delays.  SP31 gets adequate treatment via a wetland and pond.  SP30 short circuits which should be repaired.  Receiving tributary is not degraded by Fe or Mn.  Treatment options include more detention time.  For SP30:  Initial cost $5,426; O & M $1,565; Recap $5,932.  For SP31:  Initial cost $5,426; O & M $321; Recap $5,932.  $65,000.00 in bond was moved to the Legacy account.  This site is included in the pending O & M contract.  The plan is to use the passive treatment design contractor list-funding allocation approved.  Treatment system design is being worked on by consultant.

************************************************************************

Permittee			Metco Mining & Minerals Inc

Permit no. (BF Date) 		56840103 (3/10/95)

Site Name			Ankey

Township County		Jenner/Somerset

Discharge id (SIS MP No)	MM1 (87389) MM5 (87863) & MM9 (87864)

Separate functional treatment is in place for each discharge.  These alkaline-iron discharges flow 1-3 gpm through aerobic wetland ponds.  There are minimal O & M needs since the system has been working for many years with no maintenance.  For MM1, Initial cost $13,730; O & M $871; Recap $ 41,347.  For MM5, Initial cost $2,711; O & M $562; Recap $10,835.  For MM9, Initial cost $ 525; O & M $1,516; Recap $1,407.  Bond was waived for surety reclamation.  This site is included in the pending O & M contract.

************************************************************************

Permittee			American Development Corp

Permit no. (BF Date) 		56840112 (12/16/94)

Site Name			Job 33

Township County		Black/Somerset

Discharge id (SIS MP No)	BG156 (88846)

Small intermittent discharge diverted through ALD that eventually failed, very little impact.  Initial cost $16,748; O & M $31,478; Recap $12,206.  Bond was waived for surety reclamation.  The plan is to use the active treatment design contractor list-funding allocation approved.  Treatment system design in progress by consultant.  O & M is included in a contract.

************************************************************************

Permittee			Big J Mining Inc

Permit no. (BF Date) 		56960111 (2/27/01)

Site Name			Sorber

Township County		Quemahoning/Somerset

Discharge id (SIS MP No)	MD1 (87223)

High probability for remining with an application is in process.  Proposed Sub-F abatement plan requires the permittee to construct the treatment system.  Initial cost $13,664; O & M $23,064; Recap $9,958.  Bond was transferred to the Legacy Account - $116,877.00.  This site is included in the pending O & M contract.

************************************************************************Permittee			Bituminous Coals Inc.

Permit no. (BF Date) 		56773136 (2/16/06)

Site Name			Addison Strip

Township County		Addison/Somerset

Discharge id (SIS MP No)	TOS1 (23472), TOS2 (23473), TOS3 (23474)

Caustic system and ponds are in-place, but may need updating.  For MD1 caustic system:  Initial cost $13,664; O & M $37,578; Recap $9,958.  Bond declared forfeited but not collected, remaining bond is $280,240.00.  Alternate enforcement actions may result in the operator upgrading the treatment and providing a bond/trust.  Commonwealth Court action filed in November 2009.

************************************************************************

Permittee			Big Mack Leasing

Permit no. (BF Date) 		32950104 (8/20/07)

Site Name			Iselin No. 18

Township County		Young/Indiana

Discharge id (SIS MP No)	MD11 (23085)

Upwelling limestone ponds are in-place, but need additional caustic treatment.  Raw water cannot be sampled as it up-wells through limestone pond.  MD11 (caustic) Initial cost:  $52,784; O & M $52,357; Recap $63,443.  Bond declared forfeited but not collected ($125,000.00).  Consultant for the permittee is developing treatment system design.  Consent order with the operator was signed December 22, 2009.  The agreement requires a trust to be established (funded through monthly payments for a couple of years).

************************************************************************

Greensburg DMO

Permittee			Darmac Coal Inc

Permit no. (BF Date) 		03840112 (5/11/94)

Site Name			Darmac 14

Township County		East Franklin/Armstrong

Discharge id (SIS MP No)	D-1 (73563) & D-2 (98329)

D-1 has a functioning passive treatment system (ALD, Sed Pond and Natural Wetland).  It is possible that D-2 (headwater to a trib) could be diverted to this system.  A limestone ditch is in place for D-2.  Treatment cost estimate:  For D-1, Initial cost $34,529; O & M $3,465; Recap $177,729.  For D-2 (Pond and Mn Removal Bed), Initial cost $32,931; O & M $3,251; Recap $62,409.  Bond ($491,000) has been used for land reclamation.  Treatment system design proposal received.  There are space constraints (stream, stormwater pond and sewer) and other issues (property owner, wetland, Development).  Treatment system design proposal received from consultant ($28,989.30).  Treatment system design is underway.

***********************************************************************

Permittee			Darmac Coal Inc

Permit no. (BF Date) 		03890108 (11/15/93)

Site Name			Silver Rock

Township County		Washington/Armstrong

Discharge id (SIS MP No)	Highwall Drain (93827) and Main Discharge/SZ (93828)

The highwall drain has a functioning passive treatment system (limestone & cattail ponds).  The main discharge is in a seep zone which has no treatment.  The seep zone is alkaline with Fe and Mn.  The flow could be collected and directed to a large existing sed pond, which could be modified to treat the water.  Treatment cost estimate (Combined):  Initial cost  $127,097; O & M $7,128; Recap $120,911.  Bond collection was waived ($633,140) for land reclamation.  Treatment system design proposal received from consultant ($45,318.00).  Treatment system design is underway.

***********************************************************************

Permittee			Arthur Brooks Coal Co

Permit no. (BF Date) 		26830108 (6/1/93)

Site Name			Honsaker

Township County		German/Fayette

Discharge id (SIS MP No)	4 (80184) & 9A (80186)

Sample points 4 and 9A are Sub F points for an adjacent Amerikohl permit.  This site has no treatment system.  Treatment cost estimate:  4 is low flow discharge (0-2 gpm).  9A has good quality at higher flows and bad quality at low flows.  March 2008 Cost estimate (Combined):  Initial Cost $52,839; O & M $13,652; Recap $5,714.  There are potential property issues.  System design underway ($48,645).

***********************************************************************

Permittee			James Rumble & William Bane

Permit no. (BF Date) 		26840110 (2/26/92)

Site Name			Rumble & Bane

Township County		Luzerne/Fayette

Discharge id (SIS MP No)	1S (40087)

The discharge flows into a sed pond that doesn’t discharge at this time.  Treatment cost estimate:  Initial cost (Pebble Quick lime) $40,857; O & M $25,180; Recap $14,542.  System design is underway.

***********************************************************************

Permittee			Global Coal Recovery Inc

Permit no. (BF Date) 		26931601 (11/1/96)

Site Name			Isabella

Township County		Luzerne/Fayette

Discharge id (SIS MP No)	37 (20914) & 62 (20318)

Data sheet notes that the discharges may be eliminated by removing the refuse.  The site is involved in litigation.  Combined system (Hydrated lime and Mn removal bed):  Initial cost $266,049; O & M $40,885; Recap $214,140.  Sampling underway.

***********************************************************************

Permittee			Frank Kowalski

Permit no. (BF Date) 		65723055 (6/6/94)

Site Name			Strip # 3

Township County		Murrysville Boro/Westmoreland

Discharge id (SIS MP No)	SP8 (40221) & SP8A (40222)

There are fish in the stream above and below the discharges.  The discharges were pre-existing and have improved.  For SP8, a VFP is planned and for SP8A, a VFP and Mn removal bed are planned.  Site access will most likely require a stream crossing.  Combined system costs:  Initial cost $80,301; O & M $5,635; Recap $196,833.  $128,696 in bond collected spent for land reclamation.  Hydrogeologic investigation is pending.

***********************************************************************

Permittee			Delta Penn Corp

Permit no. (BF Date) 		65830202 (7/6/87)

Site Name			Hostetter

Township County		Unity/Westmoreland

Discharge id (SIS MP No)	BS-9 (86385), BS-10 (86384) & BS-11 (86386)

Discharges emanate at the edge of a stream.  Stream bank is made up of coal refuse.  Low flow discharges (1-2 GPM).  Combined system (caustic):  Initial cost $26,398; O & M $36,513; Recap $14,754.  $72,561.31 in bond collected and spent.  $550,000 spent on surface reclamation.  There are space issues (stream, sewer).  Scope of work for design has been drafted.

***********************************************************************

Permittee			Bulldog Excavation

Permit no. (BF Date) 		65940108 (1/22/01)

Site Name			Andrews

Township County		Sewickley/Westmoreland

Discharge id (SIS MP No)	MD1 (53255)

Low flow discharge (1 gpm).  It is a 50 foot long seep zone along the stream bank.  Initial cost (soda ash, Mn removal bed and ponds) $11,924; O & M $24,354; Recap $29,044.  $11,000 in bond collected was spent on land reclamation.  Will likely require a cut-off trench to collect the water.  Treatment system design is underway.

***********************************************************************

Permittee			Greene County Coal Co

Permit no. (BF Date) 		30840102 (5/18/94 & 4/4/01)

Site Name			McNatt

Township County		Monongahela/Greene

Discharge id (SIS MP No)	S-1B (17228), S-2B (17230) and S-2C (17231)

Other discharges associated with this permit either meet effluent (S-1A) or is Alkaline Manganese (S-2A).  This permit was on the ABS inventory and removed and then returned.  There is an existing treatment system (surety installed) for S-2B.  The data sheet indicates that the ALD for S-2B may be plugging.  Specific maintenance requirements need to be identified.  Discharge S-2C sometimes is slightly acidic with elevated manganese concentrations, but at other times it is alkaline with low metal concentrations.  Recent sampling shows that the in-stream manganese concentration immediately above the entry of S-2A is 2.6 mg/land the in-stream manganese concentration below the entry of S-2A is 2.7 mg/l.  Further evaluation is needed.  Total (S-2A, S-2B and S-1B/S-2C) Initial cost $60,628; O & M $5,089; Recap $28,093.  Bond was waived.  Abatement may be possible.  Design for S2C is underway.

************************************************************************

Knox DMO

Permittee			Pengrove Coal Co

Permit no. (BF Date) 		10820121 (6/23/94)

Site Name			Ruth

Township County		Venango/Butler

Discharge id (SIS MP No)	12.2 (22387), 9B (101663) & 9F (100206)

9B and 9F were added to the inventory.  Treatment system needs to be built.  Treatment cost estimate (12.2 and 9F):  Initial cost $70,579; O & M $65,349; Recap $64,297.  Bond has been  transferred to the Legacy Account ($259,700).  January 2008 samples show that 9F is highly acidic with Mn > 50 and Al >100.  9B is low flow (was frozen at that time) and is next to the back yard (adjacent to a dog house) of a home that was built after the mining.  Additional samples of 9B are needed in order to do a cost estimate for treating it.  The costs are based on a caustic system.  Initial landowner contact was negative.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		10860118 (7/18/05)

Site Name			Snyder Sertik

Township County		Allegheny/Butler

Discharge id (SIS MP No)	D1 (24739)

This discharge has a functioning passive treatment system.  Treatment cost estimate:  Initial cost $21,631; O & M $2,264; Recap $11,825 (Trust rate of return of 8.43% used).  No bond was collected.  Partially funded trust.  No corrective work has been required yet.  DEP doing O & M.  Grant agreement has been executed with CSF to cover any work needed.

************************************************************************

Permittee			Doverspike Bros Coal Co Inc

Permit no. (BF Date) 		10940105 (2/7/00)

Site Name			Emrick

Township County		Fairview/Butler

Discharge id (SIS MP No)	MD1 (46414), MD1A (52649), MD2 (46415), MD3 (46417), MD4 (46418), MD5 (46419)

This was added to the list in 2008.  Sampling is needed to determine current status of the seeps.  Initial sampling conducted in June 2008 shows that the discharges are alkaline.  Some met effluent limits.  Others are alkaline-manganese.  Cost estimates will follow.  Treatment will most likely result in negative environmental benefit.  Sampling continues.  This site was included on the MRAB tour on October 2009.

***********************************************************************Permittee			C & K Coal Co

Permit no. (BF Date) 		16713004 (7/18/05)

Site Name			Hill Estate

Township County		Limestone/Clarion

Discharge id (SIS MP No)	D2 (27664)

Treatment cost estimate:  Initial cost $69,601; O & M $4,249; Recap $ 38,048 (Trust rate of return of 8.43% used).  Partially funded trust.  Inspected quarterly.  The system consists of cattail ditches and a pond.  Since C&K quit chemically treating, the final effluent has been Alkaline-Manganese.  The receiving stream is degraded for Mn above the treatment system.  A re-evaluation is needed.  DEP doing O & M.  Grant agreement has been executed with CSF to cover any work needed.

************************************************************************

Permittee			Glacial Minerals Inc

Permit no. (BF Date) 		16803011 (1/5/95)

Site Name			Vosburg

Township County		Clarion/Clarion

Discharge id (SIS MP No)	GVO-1 (62827)

There is a small functioning passive treatment system in place.  Low flow alkaline seep with iron.  Initial cost $8,654; O & M $1,810; Recap $9,460.  DEP doing O & M.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		16803030 (7/18/05)

Site Name			Smith Heasley

Township County		Toby/Clarion

Discharge id (SIS MP No)	D1 (25040)

This needs to be monitored.  Initial cost $61,292; O & M $3,652; Recap $33,506 (Trust rate of return of 8.43% used).  No bond was collected.  Partially funded trust.  The treatment system consists of cattail bog and pond.  The pond discharge has been Alkaline Manganese since the chemical treatment was stopped.  Monitoring data shows that the receiving stream is degraded for Mn.  To meet for Mn, either needs chemical treatment or a Mn Bed, but room is an issue.  Treatment system discharge has met effluent limits since 2005.  DEP doing O & M.  Grant agreement has been executed with CSF to cover any work needed.

************************************************************************

Permittee			REM Coal Co Inc

Permit no. (BF Date) 		16820107 (6/20/91)

Site Name			Truittsburg

Township County		Redbank/Clarion

Discharge id (SIS MP No)	RT12(58208)

Needs a treatment system built.  Aluminum requires treatment.  Manganese shows up in the stream.  There is little room for a facility.  TIP is pending with consultant for system design.  Initial cost $35,051; O & M $25,746; Recap $31,931.  $30,976 in collected bond remains.  Caustic system was used to get the numbers.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		16830114 (7/18/05)

Site Name			Tremba Horner

Township County		Toby/Clarion

Discharge id (SIS MP No)	SCF37(28175)

This site has a functioning Passive Treatment System.  Initial cost $82,778; O & M $4,557; Recap $45,251 (Trust rate of return of 8.43% used).  Partially funded trust.  DEP doing O & M (inspections and sampling).  Grant agreement has been executed with CSF to cover any work needed.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		16840103 (7/18/05)

Site Name			Kriebel

Township County		Perry/Clarion

Discharge id (SIS MP No)	Z1(25072)

This discharge has a functioning passive treatment system.  Initial cost $26,866; O & M $2,447; Recap $14,687 (Trust rate of return of 8.43% used).  No bond was collected.  Partially funded trust.  Some corrective work was done to a ditch through the CSF in 2007.  DEP doing O & M.  Grant agreement has been executed with CSF to cover any work needed.

************************************************************************

Permittee			Glacial Minerals, Inc

Permit no. (BF Date) 		16850101 (1/5/95)

Site Name			Blair

Township County		Porter/Clarion

Discharge id (SIS MP No)	Z-1 (28334) [aka GBL8], SPA1(43029) & SPA2 (45232)

Z-1/GBL8 has a passive treatment system in place.  For SPA1/SPA2:  Initial cost $78,270; O & M $29,011; Recap $71,304.  For Z-1:  Initial cost $111,994; O & M $4,773; Recap $102,025.  $294,100 in collected bond remains.  GB1 and GBL8 are BAMR id’s.  Z-1 and SPA1 are DMO SIS id’s.  SPA2 goes to the same pond as SPA1 except from the other direction so any added treatment to SPA1 would apply to SPA2 too.  The BD crew completed enhancements (move the emergency spillway to eliminate short-circuiting) to the pond in Summer 2008.  Additional treatment may be needed-monitoring continues.  For Z-1, treatment consists of the bog and pond.  For SPA1 (and SPA2) pebble quick lime 1 ton bin system was used.

************************************************************************

Permittee			REM Coal Co, Inc

Permit nos. (BF Date)		33803040/33743044 (5/22/91)

Site Name			Smail Orcutt

Township County		Union/Jefferson

Discharge id (SIS MP No)	RAW (81789) and ALD (81785)

BAMR constructed a passive treatment system.  O &M (flushing) is being conducted by the Mill Creek Coalition.  O & M is a high priority.  For RAW:  Initial cost $17,264; O & M $46,864; Recap $15,728.  For ALD:  Initial cost $14,864; O & M $24,160; Recap $13,541.  A total of $953.51 in bond is left.  For the O&M costs AMDTreat was used.  The last samples show the treatment of RAW not meeting for Mn (>50) and acidity (pH is 7.0) and the treatment for ALD not meeting for Mn (>40).  When the existing system fails, treating chemically will probably be the best option.  Grant agreement for O & M has been executed with Headwaters Trust for O & M and potential system redesign.

************************************************************************

Permittee			Gurosik Coal Co, Inc

Permit no. (BF Date) 		33830117 (7/26/04)

Site Name			King

Township County		Pine Creek/Jefferson

Discharge id (SIS MP No)	BOG (30326)

This has a passive treatment system in place that no longer functions.  Initial cost $69,276; O & M $3,983; Recap $63,110.  $71,700 in bond collected.  The “raw” water can’t be sampled since it comes up in the bog/pond but the water quality of BOG is highly variable.  Sometimes it’s alkaline but most of the time it’s acid.  The Fe is less than 7 but is high enough that it probably would cause a problem in a Mn removal bed.  The cost estimates were based on an aerobic wetland (to eliminate the Fe) and a Mn removal bed.  Scope of work completed.  Design is underway-should be completed in February 2010.

************************************************************************

Permittee			Doverspike Bros. Coal Co, Inc

Permit no. (BF Date) 		33840111 (2/7/00)

Site Name			Mowery

Township County		Porter/Jefferson

Discharge id (SIS MP No)	MD1 (30341)

This is a low flow discharge with a functioning passive treatment system in place.  Initial cost $28,553; O & M $2,340; Recap $26,011.  $22,455 in bond was transferred to the ABS Legacy Sites Trust Account.  The O & M amounts to being inspected quarterly by DEP.  “Raw” water can’t be sampled.  The discharge goes through a limestone trench to a pond.  MD1 is out of the trench.  The pond hasn’t discharged recently and the rarely discharged before.  Has met effluent limits when it does discharge.

************************************************************************

Permittee			C & K Coal Co

Permit no. (BF Date) 		61783001 (7/18/05)

Site Name			Racic

Township County		Scrubgrass/Venango

Discharge id (SIS MP No)	1C (72126)

Discharge treatment is ongoing-funded by the C & K trust.  Initial cost $79,497; O & M $35,055; Recap $43,458 (Trust rate of return of 8.43% used).  The C&K trust fund treats 6 sites.  Under the contract for 2007, the trust pays $6400/month to the contractor for all of the sites.  The trust spent $171,000 (including chemicals and maintenance) in 2007.  The O & M for this site is now covered under a grant agreement with the Clean Streams Foundation.  About $28,000 was spent for O & M from October 2008 through September 2009.

************************************************************************

Permittee			H & D Coal Co

Permit no. (BF Date) 		61820102 (12/22/93)

Site Name			Merola

Township County		Irwin/Venango

Discharge id (SIS MP No)	4 (40609), S10 (40611) & S10A (44741)

For discharge 4, BAMR constructed a passive treatment system which is functioning.  For Discharges S10 and S10A, treatment must be built and aluminum is an issue.  For discharge 4:  Initial cost $74,657 (BAMR Cost plus additional ponds); O & M $4,120; Recap $81,614.  For S10/S10A:  Initial cost $63,311; O & M $36,649; Recap $57,676.  $327,908.42 in bond remains.  Inspected quarterly.  For discharge 4, a combination of what BAMR spent and AMDTreat was used to get the treatment costs.  For discharges S10 and S10A, AMDTreat with caustic was used.  Discharge S10A was not flowing in January 2008.  Sampling being done for design.

************************************************************************

Permittee			Pengrove Coal Co

Permit no. (BF Date) 		61820105 (10/1/93)

Site Name			Martin

Township County		Clinton/Venango

Discharge id (SIS MP No)	29W (44998)

No treatment in place.  Initial cost $43,669; O & M $41,215; Recap $39,782.  The cost is based on using a caustic system.  $135,247 in bond remains.  Landowner notices sent.  Two of 4 replied favorably-other no response yet.  Land reclamation of pre-act pit and spoil may complicate (or facilitate) this project.  This site was included on the MRAB field trip in October 2009.

***********************************************************************

Potential ABS Discharge sites

Cambria DMO

Trusts/Bonds not finalized

ZUBEK INC

Permit Number:	56663135(WALKER MINE)[1]

Bond Amount		$165,665.00

Target Trust Amount	$253,708.00

Total Annual O & M Cost Estimate:  $10,798

A draft CO & A was sent for legal review in February 2006.  Work is underway on the participation agreement.  Awaiting legal approval.  The water quality of the discharge has improved.  Costs should be recalculated.

************************************************************************

EP BENDER COAL CO/EP BENDER COAL CO INC

Permit numbers:	11793025(SGL 108A)[2] & 11930102(54 JOB SGL 184)[1]

Bond amount:		$984,800.00 & $429,500.00 & $11,200 (11850111)

Target Trust Amount	$2,371,299.70

Total Annual O & M Cost Estimate:  $96,624

The treatment facility is located on permit no. 11850111.

Finalized and funded 10/1/09.

************************************************************************

MARQUISE MINING CORP

Permit Number:	56950105(MF LAND OPR)[1]

Bond Amount:		$11,600.00

Target Trust Amount	$150,873.00

Required Bond	$317,744.00

Total Annual O & M Cost Estimate:  $6,771

A draft CO & A was sent for legal review in September 2007.  Bond is a CD which will be deposited in the trust.  Draft CO & A was sent to the operator on 11/7/08.

************************************************************************

SVONAVEC INC

Permit Number:	56890102(OHLER STRIP)[8]

Bond Amount		$134,900.00

Target Trust		$402,321.00

Required Bond	$686,681.00

Total Annual O & M Cost Estimate:  $16,000 (based on Target trust divided by 25)

A draft CO & A was sent for legal review in July 2006.  Legal review has been reassigned to Pittsburgh.  CO & A was sent to Operator in February.  Bond demand letter was sent in December 2009.

************************************************************************

ACTION MINING INC

Action 2 Trust

Permit Numbers:	5679123(ROMESBURG)[4], 56860104(SCHROCK)[2], 56880103(JOPA NO 1)[1] & 56950101(POORBAUGH)[1]

Total Bonds		$756,847.00

Target Trust		$1,064,038.00

Required Bond	$2,084,568.00

Total Annual O & M Cost Estimate:  $37,740 (about $24,000 is for primacy)

Jopa No 1 is issued to Future Industries and is listed as active in eFACTS.  It has the most bond ($570,285.00), but no treatment costs.  MDI indicator is on.  Romesburg is pre-primacy.

Consent order is being drafted.  This will be handled as an amendment to the existing agreement for the finalized trust for Action Mining.  Draft CO & A was sent to Action.  Action continues to pay into the trust in anticipation of the amendment.

************************************************************************

COONEY BROS COAL CO

Permit numbers:	11773037(CAROFF)[1], 11803038(POT RIDGE 2)[1], 11813039(FELLER NO 2)[3], 11813040(DUNLO NO 1)[1], 11860104(BETHLEHEM)[2], 4270BSM1(DEAN NO 3)[1] & 56743138(POT RIDGE LASKY)[1]

Total Bonds		$6,290,490.00

Target Trust		$14,336,322.00

Required bond		$25,511,805.00

Total Annual O & M Cost Estimate:  $618,380

Dean No 3 is pre-primacy.  Pot Ridge 2 and Pot Ridge Lasky are listed as Regraded in eFACTS but the MDI indicator is on.  The costs for the Dean No. 3 are included above.  Surety participation is anticipated to facilitate system upgrades.  Funding has begun through an escrow agreement with CSF as the escrow agent.  $921,914 deposited in escrow as of November 2009.  Improvements to the treatment systems are underway.

************************************************************************

SHADE MINING CO

Permit numbers:	4173SM6(SHAULIS)[1] & 56813050(SHADE NO 3)[1]

Total bonds		$355,345.00

Target Trust		$705,141.00

Required Bond	$1,315,528.00

Total Annual O & M Cost Estimate:  $28,169

Shaulis is pre-primacy.

A draft CO & A was sent for legal review in March 2006.  Legal review has been reassigned to Pittsburgh.  Proposed CO & A has been sent to the company.

************************************************************************

C & O COAL CO

Permit Number	56823108(BURKHOLDER)[1]

Bond			$12,570.00

Target Trust		$49,055.00

Required bond		$92,559.00

Total Annual O & M Cost Estimate:  $1892

A bond demand letter was sent 2/20/08.  There is a desire to deposit the collateral bond in a trust.  Treatment continues, but it is possible that this site may be forfeited.

************************************************************************

BLAIRSVILLE ASSOC

Permit Numbers	32823005 (EAGLE)[2] & 39A77SM4(ST CLAIR JOB NO 18)[3]

Total bond		$563,095.00

Target Trust		$277,418.95

Required bond		$603,654.32

Total Annual O & M Cost Estimate:  $12,283

St Clair Job No 18 is pre-primacy.  Blairsville Assoc. is a general partnership.  The partners are in other companies.  Reconstruction of Route 22 infringed on the previous treatment system.

************************************************************************

KMP ASSOC INC

Permit number		32940109(EHENGER)[1] & 32990110 (PARK)[0]

Bond			$124,500.00 & $37,041.00

Target Trust		$178,397.00

Required bond		$306,825.00

Total Annual O & M Cost Estimate:  $7,423

Trust executed 10/21/09.  To be paid over three years.

************************************************************************

PBS COALS INC

Permit numbers	3366BSM2(MAGNETO)[3], 4074SM28(GARRETT)[2], 40A77SM12(JOB 21)[1], 56663098(JOLIN (JOB NO 12))[1], 56703124(LAMBERT) & 56813104(ROBERTS)[1]

Total bond		$2,616,845.00

Target Trust		Not Calculated

Required bond		Not Calculated

PBS has provided treatment cost numbers.  These calculations are still a moving target.

************************************************************************

Greensburg DMO

Trusts/Bonds not finalized

Patriot Mining

Permit Number:		30850103 (VIABLE MINE)[2]

Bond Amount			$117,900.00

Target Trust Amount		$567,049.00 (or $589,500)

Required bond Amount	$963,346.00

Total Annual O & M Cost Estimate:  $24,637

The operator is still active.  Consent order was signed and partial payment made.  Agreement specifies 10 payments.  As of April 2009 the trust balance is $241,531.02.

***********************************************************************

MINERALS TECH INC

Permit number:	02860201 (HARMAR)[2]

Total bonds		$174,000.00

Target Trust		$2,426,760.00

Trust CO & A executed in April 2009—allows payment over time.

************************************************************************

MB Energy

Permit Numbers:	11830102(GRABIAK)[2], 32810135(DIETRICH)[1], 32820134(CBC)[3], 32880108 (BRUSH VALLEY)[2], 3473SM8 (HALL TALENT)[4], 65810113(CAMPBELL SKOVIRA)[5] & 65860105(RIDGE ROAD)[3]

Bond Amount:		$1,862,312.00

Target Trust Amount	 $1,291,548.00

Required Bond	$1,843,186.00

Total Annual O & M Cost Estimate:  $96,047

Hall Talent is pre-primacy.

The operator is consolidating operations and is no longer mining.  Installation of passive treatment is pending.  Permit transfers in process.  A draft CO & A has been prepared.  Progress is being made.  Calculations are being redone based on the passive systems.  Easements were requested in the Fall of 2009.

***********************************************************************

The Knox DMO has no remaining potential ABS Legacy Discharge sites

***********************************************************************

Moshannon DMO

Partially Funded Trusts

***********************************************************************

RIVER HILL COAL CO INC

Permit numbers:	1779145(MCCLOSKEY MINE)[0], 17793044(MCCLOSKEY[1] MINE) & 17910114(DUTCH HOLLOW)[1]

Total bond Held	$146,322.50

Target Trust Value	Full funding as of 2008 is $509,104

Total Annual O & M Cost Estimate:  $19,651

$360,000 paid in as of 9/08

March 2008 balance was $359,175.

1779145 is pre-primacy

CO & A dated January 18, 2006

Some additional AMD may need to be included under this trust.  A study is underway.  September 30, 2008 balance $371,168.70.

December 2008 balance $359,204.05.

March 31, 2009 balance     $365,666.51.

September 2009 balance   $437,551.47

***********************************************************************

Trusts/Bonds not finalized

M&M Const Trust Fund

Permit number:	17870114(LATHEROW)[2]

Bond amount:		$124,880

Target Trust Amount	$140,298

Negotiations on hold pending additional data collection.

***********************************************************************

Enercorp-Forcey Opr

Permit Number:	17970102 (FORCEY)[2]

Bond Amount:		$110,880

Target Trust Amount	Preliminary trust calc. is $777,485

Meeting was held 8/19/08.  Amount recalculated to be $442,219.  Reclamation of a pond eliminated a discharge.  Annual O & M $18,688.  The draft CO & A was prepared in December 2009.

***********************************************************************

Pottsville DMO

Payment Plan Trust

Lehigh Coal & Navigation

Permit Number:		54733020 (Tamaqua)[1]

Bond Amount			$7,473,163.46 + 285,837.00 (Phased Deposit)

Required Bond Amount	$

Target Trust Amount		Current estimates of annual costs are $500,000

Estimated Annual O & M (from the treatment trust database) is $26,254.  Treatment costs are estimated at $500,000/year which would result in a $11.7 million Trust or a bond of $21.7 million.  Evaluation of the system suggests that the costs can be reduced substantially.  An escrow account has been established and it had a balance of $971,000 in December 2009.

Bond agreement amounts total $14,722,207.

An additional bond amount of $192,946 applies to this area for the Coaldale permit (54070202-issued 9/24/09) for Great Lakes silt area which overlaps.

***********************************************************************