CHECKLIST FOR THE REVIEW OF

RCRA PART B PERMIT APPLICATIONS

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION A. PART A GENERAL INFORMATION REQUIREMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| A-1 Description of Activities Conducted which Require Facility to Obtain a Permit under the Resource Conservation and Recovery Act (RCRA), and Brief Description of Nature of the Business | 270.13(a),(m) | Form 2510-PM-BWM0004 - relating to Instructions for Completing the Hazardous Waste Treatment, Storage and Disposal Permit Application (Part B Permit Application). See Form HW-C relating to History - 2540-FM-BWM0058. |  |  |
| A-2 Name, Mailing Address, and Location of Facility for which the Application is Submitted, including a Topographic Map | 270.13(b),(l) |  |  |  |
| A-3 Up to Four Standard Industrial Classification Codes which Best Reflect the Products or Services Provided by the Facility | 270.13(c) |  |  |  |
| A-4 Operator/Owner's Name, Address, Telephone Number, and Ownership Status | 270.13(d),(e) | Ownership status must include status as federal, state, private, public, or other entity. |  |  |
| A-5 Facility is New, Existing, or Located on Indian Lands | 270.13(f),(g) | Description must include information on whether this is a first or revised application with date of last signed permit application. |  |  |
| A-6 Description of Processes to be Used for Treating, Storing, and Disposing of Hazardous Waste | 270.13(i) | Description must include design capacity for these items. |  |  |
| A-7 Specification of the Hazardous Wastes Listed or Designated Under 261 | 270.13(j) | Specifications must include estimate on quantity of waste to be treated, stored, or disposed of. |  |  |

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION A. PART A GENERAL INFORMATION REQUIREMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| A-8 Listing of all Permits or Construction Approvals Received or Applied for | 270.13(k) | Permits include the following programs: Hazardous Waste Management under RCRA; Underground Injection Control under the Solid Waste Disposal Act; Prevention of Significant Deterioration, Nonattainment Program, and National Emissions Standards for Hazardous Pollutants under the Clean Air Act; ocean dumping permits under the Marine Protection Research and Sanctuaries Act; dredge and fill permits under Section 404 of the Clean Water Act; or other relevant environmental permits including state permits. See Form HW-E relating to Contractual Consent of Landowner - 2510-FM-BWM0229. |  |  |
| A-9 Complying with siting criteria in Chapter 269a (relating to siting) | 264a.18, 270a.13 |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code). Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION B. FACILITY DESCRIPTION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| B-1 General Description | 270.14(b)(1) |  |  |  |
| B-2 Topographic Map | 270.14 | Show a distance of 1,000 feet around the unit at a scale of 1 inch to not more than 200 feet (multiple maps may be submitted at this scale), and should be similar to Part A topographic map. |  |  |
| B-2a General Requirements | 270.14(b)(19) |  |  |  |
| Scale and Date | 270.14(b)(19)(i) | Other scales may be used if justified. |  |  |
| The 100-Year Flood Plain Area | 270.14(b)(19)(ii) |  |  |  |
| Surface Waters | 270.14(b)(19)(iii) |  |  |  |
| Surrounding Land Use | 270.14(b)(19)(iv) |  |  |  |
| Wind Rose | 270.14(b)(19)(v) |  |  |  |
| Map Orientation | 270.14(b)(19)(vi) |  |  |  |
| Legal Boundaries | 270.14(b)(19)(vii) |  |  |  |
| Access Control | 270.14(b)(19)(viii) |  |  |  |
| Injection and Withdrawal Wells (On Site and Off Site) | 270.14(b)(19)(ix) |  |  |  |
| Buildings and Other Structures | 270.14(b)(19)(x) | 270.14(b)(19)(x) for example list. |  |  |
| Drainage and Flood Control Barriers | 270.14(b)(19)(xi) |  |  |  |
| Location of the Treatment or Disposal Unit(s) and Decontamination Areas | 270.14(b)(19)(xii) |  |  |  |
| Location of Solid Waste Management Units | 270.14(d)(1)(i) |  |  |  |
| B-2b Additional Information on the Topographic Map for Land Disposal Facilities | 270.14(c)(3) |  |  |  |
| Uppermost Aquifer and Hydraulically Connected Aquifers Beneath Facility Property | 270.14(c)(2) |  |  |  |
| Groundwater Flow Direction | 270.14(c)(2) |  |  |  |
| Waste Management Areas | 270.14(c)(3) |  |  |  |
| Property Boundaries | 270.14(c)(3) |  |  |  |
| Point of Compliance Location | 270.14(c)(3); 264.95 | Point of compliance is defined in 264.95. |  |  |
| Location of Groundwater Monitoring Wells | 270.14(c)(3); 264.97 |  |  |  |
| Extent of any Groundwater Contaminant Plume | 270.14(c)(4)(i) |  |  |  |
| B-3 Facility Location Information | 270.14(b)(11); 264.18 |  |  |  |
| B-3a Seismic Requirements | 270.14(b)(11)(i), (ii); 264.18(a) | Seismic requirements applicable only to new facilities. |  |  |
| Political Jurisdiction in which Facility is Proposed to be Located | 270.14(b)(11)(i) |  |  |  |
| Indication of Whether Facility is Listed in Appendix VI of 264 (New Facilities) | 270.14(b)(11)(i) |  |  |  |
| New Facility must be Located at Least 200 feet from a Fault which has had Displacement in Holocene Time | 270.14(b)(11)(ii); 264.18(a) | If facility location is listed in Appendix VI of 264, this information is required. |  |  |
| B-3b Flood Plain Requirements | 270.14(b)(11)(iii), (iv); 264.18(b) |  |  |  |
| Copy of Federal Insurance Administration or other Flood Map | 270.14(b)(11)(iii) | Reference source used to determine whether facility is located in 100-year flood plain. |  |  |
| B-3b(1) Demonstration that Facility is Designed, Constructed, Operated, and Maintained to Prevent Washout, or Detailed Description of Procedures to be Followed to Remove Hazardous Waste to Safety before Facility is Flooded | 270.14(b)(11)(iv);264.18(b) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| B-3b(1)(a) Engineering Analysis to Indicate the Various Hydrodynamic and Hydrostatic Forces Expected to Result from the 100-Year Flood Plain | 270.14(b)(11)(iv); 264.18(b) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Demonstration that no Adverse Effects will Result from Failure to Remove Waste by Providing: | 270.14(b)(11)(iv); 264.18(b)(ii) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Volume and Physical and Chemical Characteristics of the Waste in the Facility | 270.14(b)(11)(iv); 264.18(b)(ii)(A) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Concentration of Hazardous Constituents that Would Potentially Affect Surface Waters as a Result of Washout | 270.14(b)(11)(iv); 264.18(b)(ii)(B) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Impact of such Concentration on Current or Potential uses of, and Water Quality Standards Established for, the Affected Surface Waters | 270.14(b)(11)(iv); 264.18(b)(ii)(C) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Impact of Hazardous Constituents on the Sediments of Affected Surface Waters, or the Soils of the 100-Year Flood Plain, that could Result from Washout | 270.14(b)(11)(iv); 264.18(b)(ii)(D) | Flood plain requirements applicable if facility is located in 100-year flood plain. |  |  |
| Plan and Schedule for Future Compliance | 270.14(b)(11)(v) | Flood plain requirements applicable if facility is located in 100-year flood plain and not in compliance with 264.18(b). |  |  |
| B-4 Traffic Patterns | 270.14(b)(10) | Show turns across traffic lanes and stacking lanes, if appropriate. |  |  |
| Estimate of Number and Types of Vehicles around the Facility | 270.14(b)(10) |  |  |  |
| Traffic Control Signs and Signals | 270.14(b)(10) |  |  |  |
| Road Surface Composition and Load-Bearing Capacity | 270.14(b)(10) |  |  |  |

Notes:

a.	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION C. WASTE CHARACTERISTICS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| C-1 Chemical and Physical Analyses | 270.14(b)(2); 264.13(a); 264a.13 | Data generated by testing the waste, published data on the waste, or data gathered from similar processes may be used. |  |  |
| C-1a Containerized Waste | 270.15(b)(1); 264.172 | Demonstrate that waste is compatible with container construction materials. |  |  |
| C-1b Waste in Tank Systems | 270.16(a); 264a.191; 264.190(a); 264.191(b)(2); 264.192(a)(2) | Demonstrate that tank construction materials are compatible with waste stored in tank. |  |  |
| C-1c Waste in Piles | 270.18(a); 264.250(c)(1),(4) |  |  |  |
| C-1d Landfilled Wastes | 270.21(a); 264.13(c)(3); 264.314; | Demonstrate that sorbent materials are non-biodegradable. |  |  |
| C-1e Wastes Incinerated and Wastes used in Performance Tests | 270.19(c); 270a.62; 270.62(b);264.341 |  |  |  |
| C-1f Wastes to be Land Treated | 270.20(b)(4); 264.271(a)(1), (2); 264.272; 264.276, Part 261 Appendix VIII; 264a.276 | If food-chain crops will be grown in or on treatment zone, identify hazardous constituents reasonably expected to be in or derived from waste. |  |  |
| C-1g Wastes in Miscellaneous Treatment Units | 270.23(d) |  |  |  |
| C-1h Wastes in Boilers and Industrial Furnaces | 270.66(c); 270a.66; 266.102(b); |  |  |  |
| C-1i Wastes on Drip Pads | 270.26; 264.570; 264a.570 |  |  |  |
| C-2 Waste Analysis Plan | 270.14(b)(3); 264.13(b),(c) |  |  |  |
| C-2a Parameters and Rationale | 270.14(b)(3); 264.13(b)(1) |  |  |  |
| C-2b Test Methods | 270.14(b)(3); 264.13(b)(2) |  |  |  |
| C-2c Sampling Methods | 270.14(b)(3); 264.13(b)(3) | If a sampling method described in 261 Appendix I is not used, facility must provide detailed description of proposed method and demonstrate its equivalency. |  |  |
| C-2d Frequency of Analyses | 270.14(b)(3); 264.13(b)(4) |  |  |  |
| C-2e Additional Requirements for Wastes Generated Off Site | 270.14(b)(3); 264.13 (b)(5), (c); 264.73(b) | Describe statistical method used to determine a representative sample of incoming waste. |  |  |
| C-2f Additional Requirements for Ignitable, Reactive, or Incompatible Wastes | 270.14(b)(3); 264.13(b)(6); 264.17 |  |  |  |
| C-2g Additional Requirements Pertaining to BIF Facilities | 270.22; 266.102(e)(6)(ii) (C),(e)(6)(iii) |  |  |  |
| C-2h Additional Requirements Pertaining to Containment Buildings | 270.14(b)(3); 264.1100; 264a.1100 |  |  |  |
| C-3 Waste Analysis Requirements Pertaining to Land Disposal Restrictions | 270.14(b)(3); 264.13; 264.73; Part 268; 264a.13; 268a.1 |  |  |  |
| C-3a Waste Analysis | 270.14(a); 264.13(a)(1); 264a.13;268a.1 268.1; 268.7; 268.9; 268.32 - 268.37; 268.41 - 268.43 | Waste that was newly identified or newly listed as hazardous after 11/08/84 for which the U.S. Environmental Protection Agency has not promulgated land disposal prohibitions or treatment standards are not subject to land disposal provisions. |  |  |
| C-3a(1) Spent Solvent and Dioxin Wastes | 270.14(a);264.13(a)(1); 268.2(f)(1); 268.7; 268.30; 268.31 |  |  |  |
| C-3a(2) California List Wastes | 270.14(a);264.13(a)(1); 268.7; 268.32; 268.42(a); RCRA Section 3004(d) |  |  |  |
| C-3a(3) Listed Wastes | 270.14(a);264.13(a)(1); 268.7; 268.33 - 268.36; 268.41 - 268.43; 268a.1 | Arsenic-containing nonwastewater may use the extraction procedure (EP) toxicity test to determine compliance with treatment standards. |  |  |
| C-3a(4) Characteristic Wastes | 270.14(a); 264.13(a)(1); 268.7, 268.9; 268.37; Part 268 Appendix I, IX | Characteristic D008 lead nonwastewater and D004 arsenic nonwastewater may use EP toxicity test to determine compliance with treatment standards. |  |  |
| C-3a(5) Radioactive Mixed Waste | 270.14(a); 264.13(a); 268.7; 268.35(c),(d); 268.36(d); 268.42(d) | Hazardous debris containing radioactive waste must comply with treatment standards specified in 268.45. |  |  |
| C-3a(6) Leachates | 270.14(a); 264.13(a);268.35(a) | Leachate that originates from newly identified waste is not coded as F039 waste, but is labeled with newly listed waste code from which it is derived. |  |  |
| C-3a(7) Lab Packs | 270.14(a); 264.13(a); 268.7(a)(7),(8); 268.42(c); Part 268 Appendix IV | Lab packs containing California list polychlorinated biphenyls (PCB) or dioxins must be treated according to special incineration requirements detailed in 268.42(a). |  |  |
| C-3a(8) Contaminated Debris | 270.13(n); 268.2(g); 268.7; 268.9; 268.36; 268.45 |  |  |  |
| C-3a(9) Waste Mixtures and Wastes with Overlapping Requirements | 270.14(a); 264.13(a)(1); 268.7; 268.9; 268.41; 268.43; 268.45(a) | Waste that carries more than one characteristic or listed waste code must be treated to the most stringent treatment requirement for each hazardous waste constituent of concern. |  |  |
| C-3a(10) Dilution and Aggregation of Wastes | 270.14(a); 268.3 |  |  |  |
| C-3b Notification, Certification, and Recordkeeping Requirements | 270.14(a); 264.13; 264.73; 268.7; 268.9(d);264a.13 |  |  |  |
| C-3b(1) Retention of Generator Notices and Certifications | 270.14(a); 264.13; 268.7(a) |  |  |  |
| C-3b(2) Notification and Certification Requirements for Treatment Facilities | 270.14(a); 264.13; 268.7(b);264a.13 |  |  |  |
| C-3b(3) Notification and Certification Requirements for Land Disposal Facilities | 270.14(a); 264.13; 268.7(c)(1)264a.13 |  |  |  |
| C-3b(4) Wastes Shipped to Subtitle C Facilities | 270.14(a); 264.13; 268.7(a),(b)(6); 264a.13 |  |  |  |
| C-3b(5) Wastes Shipped to Subtitle D Facilities | 270.14(a); 264.13; 268.7(d); 268.9(d) |  |  |  |
| C-3b(6) Recyclable Materials | 270.14(a); 264.13; 268.7(b)(7) |  |  |  |
| C-3b(7) Recordkeeping | 270.14(a); 264.13; 264.73; 268.7(a) (5),(a)(6),(a)(7), (d) | Recycling facilities must keep records of name and location of each entity receiving hazardous waste-derived product. |  |  |
| C-3c Requirement Pertaining to the Storage of Restricted Wastes | 270.14(a); 264.73; 268.50 |  |  |  |
| C-3c(1) Restricted Wastes Stored in Containers | 270.14(a); 264.73; 268.50(a)(2)(i) |  |  |  |
| C-3c(2) Restricted Wastes Stored in Tanks | 270.14(a); 264.73; 268.50(a)(2)(ii) |  |  |  |
| C-3c(3) Storage of Liquid PCB Wastes | 270.14(a); 264.73; 268.50(f) |  |  |  |
| C-3d Exemptions, Extensions, and Variances to Land Disposal Restrictions |  |  |  |  |
| C-3d(1) Case-by-Case Extensions to an Effective Date | 270.14(b)(21) |  |  |  |
| C-3d(2) Exemption from Prohibition | 270.14(b)(21); |  |  |  |
| C-3d(3) Variance from a Treatment Standard | 270.14(a); 264.73; 268.7 |  |  |  |
| C-3d(4) Requirements for Surface Impoundments Exempted from Land Disposal Restrictions | 270.14(a); 264.13(b)(7); 268.4; 268.14 |  |  |  |
| C-3d(4)(a) Exemption for Newly Identified or Listed Wastes | 270.14(a); 264.13; 268.14 | If owner/operator continues to treat newly listed or characteristic hazardous waste after 48 months from promulgation of new waste listing or characteristic, surface impoundment must be in compliance with 268.4. |  |  |
| C-3d(4)(b) Treatment of Wastes | 270.14(a); 264.13; 268.4(a)(1),(b); 264a.13 |  |  |  |
| C-3d(4)(c) Sampling and Testing | 270.14(a); 264.13(b)(6); 268.4(a)(2)(i),(iv) |  |  |  |
| C-3d(4)(d) Annual Removal of Residues | 270.14(a); 264.13(b)(7)(iii); 268.4(a)(2)(ii) |  |  |  |
| C-3d(4)(e) Design Requirements | 270.14(a); 264.13; 268.4(a)(3),(4); 264a.13 |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - CONTAINERS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-1 Containers | 270.15; 264.170 |  |  |  |
| D-la Containers with Free Liquids | 270.15; 264.175(a),(b) | Containers storing waste with free liquids must meet secondary containment requirements of 264.175(b). |  |  |
| D-1a(1) Description of Containers | 270.14(b)(1); 264.171,172 | Specify numbers of containers, sizes, and specifications. |  |  |
| D-1a(2) Container Management Practices | 270.14(a); 264.173 264a.173 | Containers must be kept closed and must not be handled in any manner, which could cause them to rupture or leak. Specify aisle space and stacking height. |  |  |
| D-1a(3) Secondary Containment System Design and Operation | 270.15(a)(1); 264.175(a),(d) | Provide detailed design and profile drawings showing container storage areas. |  |  |
| D-1a(3)(a) Requirement for the Base or Liner to Contain Liquids | 270.15; 264.175(b)(1) | Demonstrate that base is impervious to waste stored and precipitation. |  |  |
| D-1a(3)(b) Containment System Drainage | 270.15(a)(2); 264.175(b)(2) | Containment system must be designed and operated to remove liquids resulting from leaks, spills, or precipitation. |  |  |
| D-1a(3)(c) Containment System Capacity | 270.15(a)(3); 264.175(b)(3) | Containment system must have capacity to hold 10 percent of container volume or volume of the largest container, whichever is greater. |  |  |
| D-1a(3)(d) Control of Runon | 270.15(a)(4); 264.175(b)(4) | Runon from storm water must be prevented unless containment system has sufficient excess capacity. |  |  |
| D-1a(3)(e) Removal of Liquids from Containment System | 270.15(a)(5); 264.175(b)(5) | Accumulated liquids must be removed in timely manner to prevent containment system from overflowing. |  |  |
| D-1b Containers without Free Liquids |  |  |  |  |
| D-1b(1) Test for Free Liquids | 270.15(b)(1) | Documentation that waste does not contain free liquids must be provided by test results or other information. |  |  |
| D-1b(2) Description of Containers | 270.14(a); 264.171; 264.172 | Describe numbers, sizes, and specifications of containers. |  |  |
| D-1b(3) Container Management Practices | 270.14(a); 264.173; 264a.173 | Same comment as D-1a(2). |  |  |
| D-1b(4) Container Storage Area Drainage | 270.15(b)(2); 264.175(c) | Same comment as D-1a(3)(b). |  |  |
| D-1b(15) Weighing or Measuring Facilities | 264a.180 | Describe facilities if required. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - TANKS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-2 Tank Systems | 270.16; 264.191 -194; 264a.191, 193, 194 |  |  |  |
| D-2a Tank Systems Descriptions | 270.14(b)(1) | Describe type (aboveground, underground) and specific location of each tank. |  |  |
| D-2a(1) Dimensions and Capacity of each Tank | 270.16(b) |  |  |  |
| D-2a(2) Description of Feed Systems, Safety Cutoff, Bypass Systems, and Pressure Controls | 270.16(c); 264.194(b) |  |  |  |
| D-2a(3) Diagram of Piping, Instrumentation, and Process Flow | 270.16(d) |  |  |  |
| D-2a(4) Ignitable, Reactive, and Incompatible Wastes | 270.16(j); 264.17(b); 264.198,199 | Demonstrate that waste is stored or treated in a way that protects against ignition or reaction. |  |  |
| D-2b Existing Tank Systems |  |  |  |  |
| D-2b(1) Assessment of Existing Tank System’s Integrity | 270.16(a); 264.191; 264a.191 | A written tank assessment must be certified by an independent, qualified, registered professional engineer. |  |  |
| D-2c New Tank System |  |  |  |  |
| D-2c(1) Assessment of New Tank System’s Integrity | 270.16(a),(e); 264.192(a) | A written tank assessment must be certified by an independent, qualified, registered professional engineer. |  |  |
| D-2c(2) Description of Tank System Installation and Testing Plans and Procedures | 270.16(f); 264.192(b) - (e) | A new tank installation must be inspected by an independent, qualified, installation inspector or registered professional engineer. |  |  |
| D-2d Containment and Detection of Releases | 270.16(g); 264.193; 264a.193 | Leak detection system must be capable of detecting leaks within 24 hours. |  |  |
| D-2d(1) Plans and Description of the Design, Construction, and Operation of the Secondary Containment System | 270.16(g); 264.193(b) - (f) |  |  |  |
| D-2d(1)(a) Tank Age Determination | 270.16(g); 264.193(a); 264a.193 | Age of each tank must be accurately determined to ascertain when secondary containment requirements apply. |  |  |
| D-2d(1)(b) Requirements for Secondary Containment and Leak Detection | 270.16(g); 264.193(b),(c);264.1101(b)(3)(iii) | A detailed description of the construction, installation, and operation of the secondary containment system is required. |  |  |
| D-2d(1)(c) Requirements for External Liner, Vault, Double-walled Tank or Equivalent Device | 270.16(g); 264.193(d),(e) | Secondary containment must consist of liner, vault, double-walled tank, or equivalent device approved by regional administrator. |  |  |
| D-2d(1)(d) Secondary Containment and Leak Detection Requirements for Ancillary Equipment | 270.16(g); 264.193(f) | Secondary containment is required for ancillary equipment except as provided in 264.193(f). |  |  |
| D-2d(1)(e) Containment Buildings Used as Secondary Containment for Tank Systems | 270.16(g); 264.1101(b)(3)(iii) | A containment building can serve as secondary containment for a tank system provided it meets requirements of 264.193(b),(c)(1&2),(d)(1). |  |  |
| D-2d(2) Requirements for Tank Systems until Secondary Containment is Implemented | 270.16(h); 264.193(i) | Annual leak tests are required until secondary containment is provided. |  |  |
| D-2d(3) Variance from Secondary Containment Requirements | 270.16(h); 264.193(g) |  |  |  |
| D-2d(3)(a) Variance Based on a Demonstration of Equivalent Protection of Groundwater and Surface Water | 270.16(h)(1); 264.193(g)(1),(h) | Detailed plans and engineering and hydrogeologic reports are required to demonstrate equivalent protection of groundwater and surface water. |  |  |
| D-2d(3)(b) Variance Based on a Demonstration of No Substantial Present or Potential Hazard | 270.16(h)(2); 264.193(g)(2),(h) | Provide detailed assessment of substantial present or potential hazards posed to human health or the environment, should a release enter the environment. |  |  |
| D-2d(3)(c) Exemption Based on No Free Liquids and Location Inside a Building | 270.16(h); 264.190(a) | Demonstrate that tanks used to treat or store hazardous waste contain no free liquid as defined by Paint Filter Test (SW-846 Method 9095). |  |  |
| D-2e Controls and Practices to Prevent Spills and Overflows | 270.16(i); 264.194(a),(b);264.195;264a.195 | Provide detailed description of controls and practices used to prevent spills and overflows. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - WASTE PILES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-3 Waste Piles | 270.18; 264.250 -259;264a.251 |  |  |  |
| D-3a List of Wastes | 270.18(a) | List all hazardous waste to be placed in waste piles. |  |  |
| D-3b Liner Exemption | 270.18(b) |  |  |  |
| D-3b(1) Enclosed Dry Piles | 270.18(b); 264.250(c) | Demonstrate that neither runoff, nor leachate is generated from the pile. |  |  |
| D-3b(1)(a) Protection from Precipitation | 270.18(b); 264.250(c) | Demonstrate that pile is inside or under structure that provides complete protection from precipitation. |  |  |
| D-3b(1)(b) Free Liquids | 270.18(b); 264.250(c)(1) | Demonstrate that neither liquids, nor materials containing free liquids are placed in the pile. |  |  |
| D-3b(1)(c) Runon Protection | 270.18(b); 264.250(c)(2) | Demonstrate that pile is protected from surface water runon. |  |  |
| D-3b(1)(d) Wind Dispersal Control | 270.18(b); 264.250(c)(3) | Demonstrate that pile design and operation controls wind dispersal of waste. |  |  |
| D-3b(1)(e) Leachate Generation | 270.18(b); 264.250(c)(4) | Demonstrate that pile will not generate leachate through decomposition or other reactions. |  |  |
| D-3b(2) Exemption for Monofills | 270.18(b); 264.251(e) | This exemption applies only to waste generated from foundry furnace emission controls or metal casting molding sand that are not hazardous waste for reasons other than toxicity characteristics. |  |  |
| D-3b(3) Alternate Design/No Migration | 270.18(c)(1); 264.251(b) | This exemption from liner requirements is based on documenting that design, operating practices, and local aspects will prevent migration of hazardous constituents into groundwater or surface water in the future. |  |  |
| D-3b(4) Exemption Based on Alternative Design and Location | 270.18(c)(1); 264.251(d) | Document that alternative design and operating practices, together with location characteristics, will prevent migration of any hazardous constituent into groundwater or surface water at least as effectively as a double liner with leachate detection system, and will allow detection of hazardous constituents through the top liner as least as effectively. |  |  |
| D-3b(5) Exemption for Replacement Waste Piles | 270.18(c); 264.251(f) | Demonstrate (1) that existing unit was constructed in compliance with design standards of Sections 3004(o)(1)(A)(i) and 3004(o)(5) of Resource Conservation and Recovery Act, and (2) there is no reason to believe that liner is not functioning as designed. |  |  |
| D-3c Liner System | 270.18(c)(1); 264.251(a)(1)(i),(c); 264a.251 | Describe liner system and demonstrate that flow of liquids through liner will be prevented. |  |  |
| D-3c(1) Liner Description | 270.18(c)(1); 264.251(a)(1)(i),(c); 264a.251 | Describe and draw liner system to demonstrate that any flow of liquids through the liner will be prevented. |  |  |
| D-3c(1)(a) Synthetic Liners | 270.18(c)(1); 264.251(a)(1),(c)(1); 264a.251 | Describe type, thickness, material, and brand name and manufacturer of liner. |  |  |
| D-3c(1)(b) Soil Liner | 270.18(c)(1); 264.251(a),(c)(1)(i) (B); 264a.251(i) | Describe bottom composite liner including its classification, thickness, and hydraulic conductivity. |  |  |
| D-3c(2) Liner Location Relative to High Water Table | 270.18(c)(1); 264.251(a)(1)(i) | Provide data showing seasonal fluctuations in depth to water table and the location of seasonal high water table in relation to liner system. |  |  |
| D-3c(3) Calculation of Required Soil Liner Thickness | 270.18(c)(1); 264.251(a)(1)(i) | Calculations using either numerical simulation techniques (unsaturated flow conditions) or Darcy Law-derived transit time equations (saturated flow conditions) must be provided. |  |  |
| D-3c(4) Liner Strength Requirements | 270.18(c)(1); 264.251(a)(1)(i) | Provide calculations showing minimum strength requirements for liners considering pressure gradients, installation and operating stresses, and climatic change stresses. |  |  |
| D-3c(5) Liner Strength Demonstration | 270.18(c)(1); 264.251(a)(1)(i) | Demonstrate that liner exceeds minimum strength requirements. |  |  |
| D-3c(6) Liner/Waste Compatibility Testing Results | 270.18(c)(1); 264.251(a)(1)(i) | Demonstrate that liner material is compatible with both waste and leachate. |  |  |
| D-3c(7) Liner Installation | 270.18(c)(1); 264.251(a)(1)(i) | Describe procedures for installing liner. |  |  |
| D-3c(7)(a) Synthetic Liner Seaming | 270.18(c)(1); 264.251(a)(1)(i) | Describe techniques to be used to bond membrane liner seams and the strength and chemical compatibility of seams with waste and leachate. |  |  |
| D-3c(7)(b) Soil Liner Compaction | 270.18(c)(1); 264.251(a)(1)(i) | Describe procedures for installing soil liner and compacting liner to achieve desired permeability. Include maximum height of lifts to be placed. |  |  |
| D-3c(7)(c) Installation Inspection/testing Programs | 270.18(c)(1); 264.254(a) | Describe quality assurance/quality control procedures to be used during liner installation. |  |  |
| D-3c(8) Liner Coverage | 270.18(c)(1); 264.251(a)(1)(iii) | Demonstrate that liner will be installed to cover all surrounding earth likely to be in contact with waste or leachate. |  |  |
| D-3c(9) Liner Exposure Prevention | 270.18(c)(1); 264.251(a)(1)(i) | Demonstrate that either the liner is protected from, or is resistant to, exposure to climatic conditions. |  |  |
| D-3c(10) Synthetic Liner Bedding | 270.18(c)(1); 264.251(a)(1)(i) | Demonstrate that sufficient bedding will be provided above and below liner to prevent rupture during installation and operation. |  |  |
| D-3d Liner Foundation Report |  |  |  |  |
| D-3d(1) Liner Foundation Design Description | 270.18(c)(1); 264.251(a)(1)(ii) | Describe liner foundation design and materials of construction and ability to withstand expected static and dynamic loadings. |  |  |
| D-3d(2) Subsurface Exploration Data | 270.18(c)(1); 264.251(a)(1)(ii) | Verify engineering characteristics of foundation materials through subsurface exploration. |  |  |
| D-3d(3) Laboratory Testing Data | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(4) Engineering Analyses | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(4)(a) Settlement Potential | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(4)(b) Bearing Capacity and Stability | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(4)(c) Potential for Bottom Heave or Blow-Out | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(4)(d) Construction and Operational Loading | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(5) Foundation Installation Procedures | 270.18(c)(1); 264.251(a)(1)(ii) |  |  |  |
| D-3d(6) Foundation Installation Inspection Program | 270.18(c)(1); 264.251(a)(1)(ii) | Describe quality assurance/quality control procedures to be used during foundation installation. |  |  |
| D-3e Leachate Collection and Removal System | 270.18(c); 264.251(a)(2),(c)(2) | Describe design and operation of system to collect and remove leachate from new portions of existing waste piles and from new waste piles. |  |  |
| D-3e(1) Upper Leachate Collection and Removal System | 270.18(c)(1); 264.251(a)(2),(c)(2) | Describe design and operating conditions to ensure that leachate depth over the liner does not exceed 1 foot. |  |  |
| D-3e(2) Leachate Detection System | 270.18(c)(1); 264.251(a)(2),(c)(3) | Describe design and operating features of leachate detection system. |  |  |
| D-3e(2)(a) Grading and Drainage | 270.18(c)(1); 264.251(a)(2); 264.221(c)(2)(ii) | Demonstrate that leak detection system design meets or exceeds specifications described in referenced regulations. |  |  |
| D-3e(3) Chemical Resistance | 270.18(c); 264.251(a)(2)(i)(A); 264.251(c)(3) | Demonstrate that all leachate collection and removal system components are chemically resistant to waste managed in the pile and the leachate expected to be generated. |  |  |
| D-3e(4) Strength of Materials | 270.18(c); 264.251(a)(2)(i)(B); 264.251(c)(3) | Demonstrate that system components are of sufficient strength and thickness to prevent collapse under expected static and dynamic loadings. |  |  |
| D-3e(5) Prevention of Clogging | 270.18(c); 264.251(a)(2)(ii); 264.251(c)(3) | Demonstrate that leachate collection and removal system’s design and operation will prevent clogging throughout active life and post-closure period of waste pile. |  |  |
| D-3e(6) Installation | 270.18(c); 264.251(a)(2) | Describe installation methods and construction quality assurance/quality control procedures. |  |  |
| D-3e(7) Maintenance | 270.18(c); 264.251(a)(2) | Describe anticipated maintenance activities that will be used to assure proper leachate management system operation throughout pile’s expected active life. |  |  |
| D-3e(8) Liquid Removal | 270.18(c); 264.251(c)(3) | Describe leachate removal system, including sumps and other equipment, and fate of the collected leachate. |  |  |
| D-3e(9) Location Relative to Water Table | 270.18(c); 264.251(c)(4) | Demonstrate that operation of leak detection system will not be adversely affected by presence of groundwater. |  |  |
| D-3f Action Leakage Rate | 270.18(c)(1)(v); 264.252 | Action leakage rate must be approved by regional administrator based on system design. |  |  |
| D-3f(1) Determination of Action Leakage Rate | 270.18(c)(1)(v); 264.252(a) | Determine action leakage rate for waste pile units subject to 264.251(c),(d). Include adequate safety margin to allow for uncertainties in design, construction, operation, and location of leak detection system, waste and leachate characteristics, sources of other liquids in system, and proposed response actions. |  |  |
| D-3f(2) Monitoring of Leakage | 270.18(c)(1)(v); 264.252(b) | Weekly leachate flow rate data must be converted to average daily flow rate. |  |  |
| D-3g Leakage Response Action Plan | 270.18(c)(1)(v); 264.253 |  |  |  |
| D-3g(1) Response Action | 270.18(c)(1)(v); 264.253(a) | Provide response action plan to describe actions to be taken if flow rate into leak detection system exceeds action leakage rate. |  |  |
| D-3g(2) Leak and/or Remedial Determinations | 270.18(c)(1)(v); 264.253(b),(c) | Response action plan must describe actions to be taken to comply with 264.223(b),(c) if the action leakage rate is exceeded. |  |  |
| D-3g(3) Notifications | 270.18(c)(1)(v); 264.253(b) | Response action plan must indicate that regional administrator will be (1) notified in writing within 7 days of determining that action leakage rate has been exceeded, (2) provided with preliminary assessment and action plan within 14 days of initial determination that action leakage rate has been exceeded, and (3) provided with status report within 30 days after original notification that action leakage rate has been exceeded. Regional administrator must receive monthly status reports for as long as flow rate exceeds action leakage rate. |  |  |
| D-3h Runon Control System | 270.18(c)(2); 264.251(g) | Describe system that will be used to prevent runon into active portions of piles. |  |  |
| D-3h(1) Calculation of Peak Flow | 270.18(c)(2); 264.251(g) | Identify peak surface water flow expected to result from 25-year design storm. Describe data sources and methods used to make peak flow calculation. |  |  |
| D-3h(2) Design and Performance | 270.18(c)(2); 264.251(g) | Demonstrate that runon control system design will prevent runon from reaching active portions of unit. |  |  |
| D-3h(3) Construction | 270.18(c)(2); 264.251(g) | Describe runon control system construction methods and any construction quality assurance/quality control procedures. |  |  |
| D-3h(4) Maintenance | 270.18(c)(2); 264.251(g) | Describe any maintenance activities required to assure continued proper runon system operation throughout unit’s active life. |  |  |
| D-3i Runoff Control System | 270.18(c)(3); 264.251(h) | Describe the runoff control system to be used to collect and control runoff from active portions. |  |  |
| D-3i(1) Calculation of Peak Flow | 270.18(c)(3); 264.251(h) | Identify the total runoff volume expected to result from a 24-hour, 25-year storm, and include data sources and methods used to make peak flow calculation. |  |  |
| D-3i(2) Design and Performance | 270.18(c)(3); 264.251(h) | Demonstrate that system has sufficient capacity to collect and hold total runoff volume calculated in D-3i(1). |  |  |
| D-3i(3) Construction | 270.18(c)(3); 264.251(h) | Describe runoff system construction methods and any construction quality assurance/quality control procedures. |  |  |
| D-3i(4) Maintenance | 270.18(c)(3); 264.251(h) | Describe any maintenance activities required to assure continued proper runoff system operation throughout unit’s active life. |  |  |
| D-3j Management of Collection and Holding Units | 270.18(c)(4); 264.251(i) | Describe how collection and holding facilities will be managed to maintain system design capacity. |  |  |
| D-3k Control of Wind Dispersal | 270.18(c)(5); 264.251(j) | Describe how pile is covered or otherwise managed to control wind dispersal. |  |  |
| D-3l Groundwater Monitoring Exemption | 270.18(b); 264.90(b)(2) | To receive exemption from groundwater monitoring requirements of Subpart F, conditions specified in D-3l(1) through D-3l(7) must be met. |  |  |
| D-3l(1) Engineered Structure | 270.18(b); 264.90(b)(2)(i) | Provide design data showing that unit is engineered structure. |  |  |
| D-3l(2) No Liquid Wastes | 270.18(b); 264.90(b)(2)(ii) | Describe procedures for ensuring that no liquid waste or waste containing free liquids will be received by, or contained in, unit. |  |  |
| D-3l(3) Exclusion of Liquids | 270.18(b); 264.90(b)(2)(iii) | Demonstrate how liquids, precipitation, and other runon and runoff will be excluded from unit. |  |  |
| D-3l(4) Containment System | 270.18(b); 264.90(b)(2)(iv) | Describe containment system (both inner and outer layers) that will enclose waste. |  |  |
| D-3l(5) Leak Detection System | 270.18(b); 264.90(b)(2)(v) | Describe design and operating data demonstrating leak detection system built into each containment layer. |  |  |
| D-3l(6) Operation of Leak Detection System | 270.18(b); 264.90(b)(2)(vi) | Demonstrate means for ensuring continuing operation and maintenance of leak detection systems during active life of unit and closure and post-closure care periods. |  |  |
| D-3(7) No Migration | 270.18(b); 264.90(b)(2)(vii) | Demonstrate to reasonable degree of certainty that unit will not allow hazardous constituents to migrate beyond outer layer of containment system prior to end of post-closure care period. |  |  |
| D-3m Treatment Within the Pile | 270.18(e) | If any treatment is conducted in pile, provide descriptions specified in D-3m(1) through D-3m(3). |  |  |
| D-3m(1) Treatment Process Description | 270.18(e) | Describe the process by which wastes are treated and the effect of the treatment on the wastes. |  |  |
| D-3m(2) Equipment Used | 270.18(e) | Describe any equipment or other materials required to initiate or promote treatment. |  |  |
| D-3m(3) Residuals Description | 270.18(e) | Describe nature and quantity of waste remaining in pile after treatment is complete. |  |  |
| D-3n Special Waste Management Plan for Piles Containing Wastes F020, F021, F022, F023, F026, and F027 | 270.18(i); 264.259 | If waste pile is not enclosed, provide plan describing how pile will be designed, constructed, operated, and maintained in order to protect human health and environment. |  |  |
| D-3n(1) Waste Description | 270.18(i)(1); 264.259(a)(1) | Identify volume, physical, and chemical characteristics of waste, including potential to migrate through soil or volatilize or escape into atmosphere. |  |  |
| D-3n(2) Soil Description | 270.18(i)(2); 264.259(a)(2) | Describe attenuative properties of underlying and surrounding soils or other materials. |  |  |
| D-3n(3) Mobilizing Properties | 270.18(i)(3); 264.259(a)(3) | Describe mobilizing properties of other materials codisposed of with this waste. |  |  |
| D-3n(4) Additional Management Techniques | 270.18(i)(4); 264.259(a)(4) | Document effectiveness of additional treatment, design, operating, or monitoring techniques. |  |  |
| D-3o Construction Quality Assurance Program | 270.18(c)(iv); 264.19 | Provide written construction quality assurance program to comply with regulations found in 264.19. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - SURFACE IMPOUNDMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-4 Surface Impoundments |  |  |  |  |
| D-4a List of Wastes | 270.17(a) | Provide list of all hazardous waste placed, or to be placed, in surface impoundments. |  |  |
| D-4b Liner System Exemption Requests | 270.17(b) |  |  |  |
| D-4b(1) Exemption Based on Existing Portion | 270.17(b)(1); 264.221(c); 264a.221 | Existing portions of surface impoundments with waste in place on November 8, 1994, and having only vertical expansion are exempted from liner system requirements. New units, lateral expansion of existing units, and replacement units at existing facilities are not exempt. Provide plan indicating limits of existing portions. |  |  |
| D-4b(2) Exemption Based on Alternative Design and Location | 270.17(b)(1); 264.221(d) |  |  |  |
| D-4b(3) Exemption for Replacement Surface Impoundments | 270.17(b); 264.221(f) |  |  |  |
| D-4c Liner System, General Items | 270.17(b)(1) | Provides discussion of the following items that apply to liner system as a whole. |  |  |
| D-4c(1) Liner System Description | 270.17(b)(1) | Provide detailed description of liner system, demonstrating that any flow of liquids into and through liners will be prevented. The liner system includes liner foundation, bottom composite liner, leachate detection system, top synthetic liner, and any protective layer placed to protect top synthetic liner. |  |  |
| D-4c(2) Liner System Location Relative to High Water Table | 270.17(b)(1), (3); 264.221(a); 264a.221 | Provide geological cross sections showing groundwater levels with seasonal fluctuations and liner foundation elevations. |  |  |
| D-4c(3) Load on Liner System | 270.17(b)(1); 264.221(a)(1),(b) | Provide results of calculations defining maximum loads or stresses that will be placed on liner system. |  |  |
| D-4c(4) Liner System Coverage | 270.17(b)(1); 264.221(a)(1), (b) | Demonstrate that liner system will be installed to cover all surrounding earth likely to be in contact with waste or leachate. |  |  |
| D-4c(5) Liner System Exposure Prevention | 270.17(b)(1); 264.221(a)(1), (b) | Demonstrate that liner system will not be exposed to elements, or that if exposed, exposure will not result in unacceptable degradation of system. |  |  |
| D-4d Liner System Foundation |  |  |  |  |
| D-4d(1) Foundation Description | 270.17(b)(1); 264.221(a)(2) | Describe foundation for liner system, including materials, and indicate bearing elevations and any load-bearing embankments placed to support liner system. |  |  |
| D-4d(2) Subsurface Exploration Data | 270.17(b)(1); 264.221(a)(2) | The engineering characteristics of liner system foundation materials should be verified through subsurface explorations. Provide information to fully describe these efforts. |  |  |
| D-4d(3) Laboratory Testing Data | 270.17(b)(1); 264.221(a)(2) | Provide index testing results to classify site materials and lab test data to evaluate engineering properties of foundation materials. Provide references to standard test procedures. |  |  |
| D-4d(4) Engineering Analyses | 270.17(b)(1); 264.221(a)(2) | Provide engineering analyses based on subsurface exploration and laboratory testing data. Include discussion of methods used, assumptions, copies of calculations, and appropriate references. |  |  |
| D-4d(4)(a) Settlement Potential | 270.17(b)(1); 264.221(a)(2) | Provide estimates of total and differential settlement of liner system foundation. |  |  |
| D-4d(4)(b) Bearing Capacity | 270.17(b)(1); 264.221(a)(2) | Provide analysis of allowable bearing capacity of liner system foundation. |  |  |
| D-4d(4)(c) Potential for Excess Hydrostatic or Gas Pressure | 270.17(b)(1); 264.221(a)(2) | Provide estimates of potential or bottom heave or blow-out of liner system or line foundation due to unequal hydrostatic or gas pressures. |  |  |
| D-4e Liner System, Liners |  |  |  |  |
| D-4e(1) Synthetic Liners | 270.17(b)(1); 264.221(a),(c) 264a.221 | For each synthetic liner in system or under consideration, provide the following general information: thickness; type; material; brand name; and manufacturer. |  |  |
| D-4e(1)(a) Synthetic Liner Compatibility Data | 270.17(b)(1); 264.221(a)(1) | Provide summary and discussion of test results and conclusions as to suitability of synthetic liner based on liner/waste compatibility testing. |  |  |
| D-4e(1)(b) Synthetic Liner Strength | 270.17(b)(1); 264.221(a)(1) | Provide data showing that synthetic liners, including seams, have sufficient strength after exposure to waste and waste leachate. |  |  |
| D-4e(1)(c) Synthetic Liner Bedding | 270.17(b)(1); 264.221(a)(2) | Demonstrate that sufficient bedding will be provided above and below the synthetic liners to prevent rupture during installation and operation. Synthetic membrane of bottom composite liner should be placed directly on soil portion. |  |  |
| D-4e(2) Soil Liners | 270.17(b)(1); 264.221(a); (c)(1) | Describe soil portion of bottom composite liner, including classification, thickness, hydraulic conductivity, and material specifications. |  |  |
| D-4e(2)(a) Material Testing Data | 270.17(b)(1); 264.221(c) 264a.221 | Provide complete results for index tests, laboratory and/or in situ permeability tests, strength tests, consolidation tests, and shrink-swell properties of soil liner material. Discuss potential for dispersion and piping of soil due to flow of liquid through soil liner layer. |  |  |
| D-4e(2)(b) Soil Liner Compatibility Data | 270.17(b)(1); 264.221(a)(1) | Provide complete results of permeability testing of soil liner material using representative of leachate from surface impoundment. |  |  |
| D-4e(2)(c) Soil Liner Strength | 270.17(b)(1); 264.221(a)(1) | Demonstrate that soil liner has sufficient strength to support loads/stresses computed in item D-4c(3). |  |  |
| D-4f Liner System, Leachate Detection System | 270.17(b)(1); 264.221(c)(2) |  |  |  |
| D-4f(1) Systems Operation and Design | 270.17(b)(1); 264.221(c)(2),(4); 264a.221 | Describe design features of leachate detection system and how system will function to detect any leakage through either liner in timely manner. |  |  |
| D-4f(2) Drainage Material | 270.17(b)(1); 264.221(c)(2)(ii) | Describe leachate detection system drainage material. |  |  |
| D-4f(3) Grading and Drainage | 270.17(b)(1); 264.221(c)(2) | Indicate slopes of leachate detection system and provide contour plan for system along with plan showing layout and spacing of piping system and any sumps, pumps, etc. Demonstrate that leak detection system is appropriately graded to assure that leakage at any point in liner system is detected in timely manner. |  |  |
| D-4f(4) System Compatibility | 270.17(b)(1); 264.221(c)(2)(iii) |  |  |  |
| D-4f(5) System Strength |  |  |  |  |
| D-4f(5)(a) Stability of Drainage Layers | 270.17(b)(1); 264.221(c)(2)(iii) | Demonstrate that drainage layer of leachate detection system has sufficient soil-bearing capacity to support loads. Provide calculations showing that drainage layer placed on sloped surfaces of surface impoundment or foundations will be stable during construction. |  |  |
| D-4f(5)(b) Strength of Piping | 270.17(b)(1); 264.221(c)(2)(iii) | Demonstrate that pipes used in piping systems have sufficient strength to support loads as computed in item D-4c(3). |  |  |
| D-4f(6) Prevention of Clogging | 270.17(b)(1); 264.221(c)(2)(iv) |  |  |  |
| D-4f(7) Liquid Removal | 270.17(b)(1); 264.221(c)(2)(v), (c)(3) | Indicate fate of collected leachate, which is considered hazardous waste. |  |  |
| D-4f(8) Location Relative to Water Table | 270.17(b)(3); 264.221(c)(4); 264a.221 |  |  |  |
| D-4g Liner System, Construction and Maintenance |  |  |  |  |
| D-4g(1) Material Specifications | 270.17(b)(1);264.221(a); 264a.221 |  |  |  |
| D-4g(1)(a) Synthetic Liners | 270.17(b)(1); 264.221(a); 264a.221 | Provide detailed material specifications for specific synthetic liner(s) to be used. |  |  |
| D-4g(1)(b) Soil Liners | 270.17(b)(1); 264.221(a); 264a.221 | For soil liners constructed of borrowed material, provide specifications; for soil liners using in-place soil, provide specifications to be used to assure that all existing materials meet requirements of liner design. |  |  |
| D-4g(1)(c) Leachate Detection System | 270.17(b)(1); 264.221(a); 264a.221 | Provide material specifications for drainage layer material, filter fabric or filter layer, piping, and sumps. |  |  |
| D-4g(2) Construction Specifications |  |  |  |  |
| D-4g(2)(a) Liner System Foundation | 270.17(b)(1); 264.221(a); 264a.221 | For installed foundations, provide construction specifications of foundation installation procedures. For units that use the in-place material for liner system foundation, provide construction specifications for preparation. |  |  |
| D-4g(2)(b) Soil Liner | 270.17(b)(1); 264.221(a),(a)(2); 264a.221 | Describe procedures for installing soil liner. |  |  |
| D-4g(2)(c) Synthetic Liners | 270.17(b)(1); 264.221(a); 264a.221; 264.226(a)(1) | Provide construction specifications for placement of synthetic liners. |  |  |
| D-4g(2)(d) Leachate Detection System | 270.17(b)(1); 264.221(a); 264a.221 | Provide construction specifications for placement of leachate detection system components, including drainage layers, piping, filter layers, sumps, pumps, etc. |  |  |
| D-4g(3) Construction Quality Assurance (CQA) Program | 270.17(b)(1),(4); 270.30(k)(2); 264.19; 264.226(a) | Provide complete details of CQA program to be used during construction of liner system to assure that it is built as designed. |  |  |
| D-4g(4) Maintenance Procedures for Leachate Detection System | 270.17(b)(1); 264.221(a); 264a.221 | Describe anticipated maintenance activities that will be used to assure proper operation of leachate detection systems throughout surface impoundment’s expected life. |  |  |
| D-4g(5) Liner Repairs During Operations | 270.17(b)(1); 264.221(a); 264a.221 | Describe methods that will be used to repair any damage to liner that occurs while surface impoundment is in operation (such as a drag line ripping the liner during cleaning operations). |  |  |
| D-4h Action Leakage Rate | 270.17(b)(5); 264.222 |  |  |  |
| D-4h(1) Determination of Action Leakage Rate | 270.17(b)(5); 264.222(a) | Identify action leakage rate for surface impoundment units subject to liner system provisions of 264.221(c) and 264.221(d). |  |  |
| D-4h(2) Monitoring of Leakage | 270.17(b)(5); 264.222(b) |  |  |  |
| D-4i Leakage Response Action Plan | 270.17(b)(5); 264.223 |  |  |  |
| D-4i(1) Response Action | 270.17(b)(5); 264.223(a) |  |  |  |
| D-4i(2) Leak and/or Remedial Determinations | 270.17(b)(5); 264.223(b),(c) |  |  |  |
| D-4i(3) Notifications | 270.17(b)(5); 264.223(b) |  |  |  |
| D-4j Prevention of Overtopping | 270.17(b)(6); 264.221(g) | Describe design and/or operating procedures that will protect against impoundment overtopping/overflow. |  |  |
| D-4j(1) Design Features | 270.17(b)(6); 264.221(g) | Describe design features used to prevent overtopping, such as spillways or weirs for flow-through systems, automatic or manual controls, and sensors and alarms. |  |  |
| D-4j(2) Operating Procedure | 270.17(b)(6); 264.221(g) | If operating procedures are instrumental to preventing overtopping, describe those procedures. |  |  |
| D-4j(3) Overtopping Prevention | 270.17(b)(6); 264.221(g) | Unless foolproof controls are used to prevent overtopping, provide results of calculations showing that adequate freeboard will be available following 100-year, 24-hour storm event. |  |  |
| D-4j(4) Freeboard Requirements | 270.17(b); 264.221(g) | Freeboard requirements associated with normal and extreme wind activity should be determined unless automatic controls are used and freeboard equals or exceeds 2 feet. |  |  |
| D-4j(5) Outflow Destination | 270.17(b); 264.221(g) | Describe fate of liquids released through flow control devices. Identify location to which waste would be moved in event of emergency. |  |  |
| D-4k Dike Stability |  |  |  |  |
| D-4k(1) Engineer’s Certification | 270.17(d); 264.226(c) |  |  |  |
| D-4k(2) Dike Design Description | 270.17(b)(7); 264.221(h) | Provide data and/or drawings specifying design layout of the dikes and their components, including materials of construction. Determine capability of dikes to withstand failure from expected static and dynamic loadings and effects of erosion. |  |  |
| D-4k(3) Erosion and Piping Protection | 270.17(b); 264.221(h) | Demonstrate that dikes are designed and constructed to minimize erosion and piping, and to prevent failure due to excessive erosion. Describe procedures for correcting erosion problems identified during unit’s operating life. |  |  |
| D-4k(4) Subsurface Soil Conditions | 270.17(b)(7); 264.221(h) | Engineering characteristics of dike foundation materials should be verified through testing and subsurface explorations, as necessary. These explorations may include: test borings; test pits or trenches; in situ tests; and geophysical exploration methods. |  |  |
| D-4k(5) Stability Analysis | 270.17(b); 264.221(h) | Describe stability analyses and results for the following conditions, as appropriate: foundation soil bearing failure of settlement; failure in dike slopes; failure of impoundment cut slopes; build-up of hydrostatic pressure due to failure of drainage system, dike cover, and liner; and rapid drawdown. |  |  |
| D-4k(6) Strength and Compressibility Test Results | 270.17(b); 264.221(h) | Provide results of strength and consolidation tests on dike materials together with description of sampling procedures and test methods. |  |  |
| D-4k(7) Dike Construction Procedures | 270.17(b); 264.221(h) | Describe methods to be used to construct dikes at new units. |  |  |
| D-4k(8) Dike Construction Inspection Program | 270.17(b); 264.221(h) | Describe inspection, monitoring, sampling and testing methods, and frequencies to be used during dike construction to assure that new dikes meet design requirements. |  |  |
| D-4l Special Waste Management Plan for Surface Impoundments Containing Wastes F020, F021, F022, F023, F026, and F027 | 270.17(i); 264.231(a) |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - INCINERATORS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-5 Incinerators | 270.19; 264.340; 264.351 |  |  |  |
| D-5a Justification for Exemption | 270.19(a) | To justify exemption under 264.340(b) or (c), document the following: (1) waste contains no, or insignificant, concentrations of Part 261, Appendix VIII materials; and (2) waste is considered hazardous solely because it is (a) ignitable and/or corrosive, or (b) reactive. |  |  |
| D-5b Trial Burn | 270.19(b) |  |  |  |
| D-5b(1) Trial Burn Plan | 270.19(b) | Submit trial burn plan or results of trial burn, including all required determinations. |  |  |
| D-5b(1)(a) Detailed Engineering Description of Incinerator | 270.62(b)(2)(ii); 270.19(c)(2) | Provide information per regulatory citation. Also, include process and instrumentation diagram. |  |  |
| D-5b(1)(b) Sampling and Monitoring Procedures | 270.62(b)(2)(iii) | Describe sampling and monitoring procedures during trial burn per regulatory citation. Sampling and analysis methods approved by the U.S. Environmental Protection Agency (EPA) must be used or, alternatively, a demonstration of equivalence with EPA-approved methods must be made. |  |  |
| D-5b(1)(c) Trial Burn Schedule | 270.62(b)(2)(iv) |  |  |  |
| D-5b(1)(d) Test Protocols | 270.62(b)(2)(v) |  |  |  |
| D-5b(1)(e) Pollution Control Equipment Operation | 270.62(b)(2)(vi) |  |  |  |
| D-5b(1)(f) Shutdown Procedures | 270.62(b)(2)(vii) |  |  |  |
| D-5b(1)(g) Incinerator Performance | 270.62(a) |  |  |  |
| D-5b(2) New Incinerator Conditions | 270.62(a) |  |  |  |
| D-5b(2)(a) Startup | 270.62(a) |  |  |  |
| D-5b(2)(b) Shakedown | 264.344(c)(1) |  |  |  |
| D-5b(2)(c) Post-trial Burn Operation | 270.62(c) |  |  |  |
| D-5b(2)(d) Incinerator Performance | 270.62(a) |  |  |  |
| D-5c Data Submitted in Lieu of Trial Burn | 270.19(c) | Provide information per regulatory citation in lieu of trial burn plan. |  |  |
| D-5c(1) Detailed Engineering Description of Incinerator | 270.19(c)(2) | Provide information per regulatory citation. Also, include process and instrumentation diagram. |  |  |
| D-5c(2) Expected Incinerator Operation | 270.19(c)(6) |  |  |  |
| D-5c(3) Design and Operating Conditions | 270.19(c)(4) |  |  |  |
| D-5c(4) Previous Trial Burn Results | 270.19(c)(5) | Describe results from all previously conducted, approved trial burns. |  |  |
| D-5d Determinations | 270.62(b)(7) |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - LANDFILLS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-6 Landfills | 270.21; 264.300 - 264.317; 264a.301 |  |  |  |
| D-6a List of Wastes | 270.21(a) |  |  |  |
| D-6b(1) Exemption Based on Existing Portion | 270.21(b)(1); 264.301(a); 264a.301 | Existing portions of landfills that have waste in place on November 8, 1984, and will have only vertical expansion are exempted from liner system requirements. Provide plan showing limits of existing portion. |  |  |
| D-6b(2) Exemption Based on Alternative Design and Location | 270.21(b)(1); 264.301(d) |  |  |  |
| D-6b(3) Exemption for Replacement Landfill Unit | 270.21(b)(1); 264.301(f) |  |  |  |
| D-6b(4) Exemption for Monofills | 270.21(b)(1); 264.301(e) |  |  |  |
| D-6b(5) Groundwater Monitoring Exemption | 270.21(c); 264.90(b)(2) | If exemption from Subpart F groundwater monitoring requirements is sought, provide data demonstrating that the following conditions are met. |  |  |
| D-6b(5)(a) Engineered Structure | 270.21(c); 264.90(b)(2)(i) | Provide design data showing that unit for which exemption is sought is an engineered structure. |  |  |
| D-6b(5)(b) No Liquid Waste | 270.21(c); 264.90(b)(2)(ii) | Describe procedures for ensuring that no liquid waste or waste containing free liquids will be received by, or contained, in the unit. |  |  |
| D-6b(5)(c) Exclusion of Liquids | 270.21(c); 264.90(b)(2)(iii) | Provide design and operating data demonstrating how liquids, precipitation, and other runon and runoff will be excluded from the unit. |  |  |
| D-6b(5)(d) Containment System | 270.21(c); 264.90(b)(2)(iv) | Describe containment system (both inner and outer layers) that will enclose waste. |  |  |
| D-6b(5)(e) Leak Detection System | 270.21(c); 264.90(b)(2)(v) | Describe design and operating data demonstrating leak detection system built into each containment layer. |  |  |
| D-6b(5)(f) Operation of Leak Detection System | 270.21(c); 264.90(b)(2)(vi) | Demonstrate means for ensuring continuing operation and maintenance of leak detection systems during active life of unit and closure and post-closure care periods. |  |  |
| D-6b(5)(g) No Migration | 270.21(c); 264.90(b)(2)(vii) | Demonstrate that unit will not allow hazardous constituents to migrate beyond outer layer of containment system prior to end of post-closure care period. |  |  |
| D-6c Liner System, General Items | 270.21(b)(1); 264.301(a),(c); 264a.301 | Discuss the items that apply to liner system as a whole. |  |  |
| D-6c(1) Liner System Description | 270.21(b)(1); 264.301(a),(c); 264a.301 | Provide detailed description of liner system, demonstrating that any flow of liquids into and through liners will be prevented. Liner system includes liner foundation, bottom composite liner, leachate detection system, top synthetic liner, and any protective layer placed to protect leachate collection system from damage. |  |  |
| D-6c(2) Liner System Location Relative to High Water Table | 270.21(b)(1); 264.301(a)(1)(i); 264a.301 | Provide geological cross sections showing groundwater levels with seasonal fluctuations and liner foundation elevations. |  |  |
| D-6c(3) Loads on Liner System | 270.21(b)(1); 264.301(a)(1)(i) | - Provide results of calculations defining maximum loads or stresses that will be placed on liner system considering: both static and dynamic loads, stresses due to installation or construction stresses resulting from operating equipment stresses due to maximum quantity of waste, cover, and proposed post-closure land use stresses resulting from settlement, subsidence, or uplift internal and external pressure gradients. |  |  |
| D-6c(4) Liner System Coverage | 270.21(b)(1); 264.301(a)(1)(iii) |  |  |  |
| D-6c(5) Liner System Exposure Prevention | 270.21(b)(1); 264.301(a)(1)(i) | Demonstrate that the liner system will not be exposed to wind or sunlight or, if exposure to any part of the system is to be permitted, that such exposure will not result in unacceptable degradation of that portion of the system. |  |  |
| D-6d Liner System, Foundation |  |  |  |  |
| D-6d(1) Foundation Description | 270.21(b)(1); 264.301(a)(1)(ii); 264a.301 | Describe foundation for liner system, including foundation materials and indicate bearing elevations on geological and construction drawings. Indicate any load-bearing embankments placed to support liner system. |  |  |
| D-6d(2) Subsurface Exploration Data | 270.21(b)(1); 264.301(a)(1)(ii) | Verify engineering characteristics of liner system foundation materials through subsurface explorations. Provide information to fully describe these efforts. |  |  |
| D-6d(3) Laboratory Testing Data | 270.21(b)(1); 264.301(a)(1)(ii) | Provide index testing results to classify site materials and lab test data to evaluate engineering properties of foundation materials. Provide references to standard test procedures. |  |  |
| D-6d(4) Engineering Analyses | 270.21(b)(1); 264.301(a)(1)(ii) | Provide engineering analyses based on subsurface exploration and laboratory testing data. Include discussion of methods used, assumptions, copies of calculations, and appropriate references. |  |  |
| D-6d(4)(a) Settlement Potential | 270.21(b)(1); 264.301(a)(1)(ii) | Provide estimates of total and differential settlement of liner system foundation. Consider stresses imposed by liner system and applicable stresses computed in item D-6c(3). |  |  |
| D-6d(4)(b) Bearing Capacity | 270.21(b)(1); 264.301(a)(1)(ii) | Provide analysis of allowable bearing capacity of liner system foundation. |  |  |
| D-6d(4)(c) Stability of Landfill Slopes | 270.21(b)(1); 264.301(a)(1)(ii) | - Provide, as appropriate, analyses of stability of: excavated slopes for units constructed below grade embankment slopes for units constructed with earthen dikes or berms, landfill slopes consisting of liner system or cover system placed on waste. |  |  |
| D-6d(4)(d) Potential for Excess Hydrostatic or Gas Pressure | 270.21(b)(1); 264.301(a)(1)(ii) | Provide estimates of potential for bottom heave or blow-out of liner system due to unequal hydrostatic or gas pressures. |  |  |
| D-6e Liner System, Liners |  |  |  |  |
| D-6e(1) Synthetic Liners | 270.21(b)(1); 264.301(a)(1)(ii),(c); 264a.301 | For each synthetic liner in system or under consideration, provide following general information: thickness; type; material; brand name; and manufacturer. |  |  |
| D-6e(1)(a) Synthetic Liner Compatibility Data | 270.21(b)(1); 264.301(a)(1)(i) | Provide summary and discuss test results and conclusions as to suitability of synthetic liner based on liner/waste compatibility testing. |  |  |
| D-6e(1)(b) Synthetic Liner Strength | 270.21(b)(1); 264.301(a)(1)(i) | Provide data showing that synthetic liners, including seams, have sufficient strength after exposure to waste and waste leachate. |  |  |
| D-6e(1)(c) Synthetic Liner Bedding | 270.21(b)(1); 264.301(a)(1)(ii); 264a.301 | Demonstrate that sufficient bedding will be provided above and below synthetic liners to prevent rupture during installation and operation. Synthetic membrane of bottom composite liner should be placed directly on soil portion. |  |  |
| D-6e(2) Soil Liners | 270.21(b)(1); 264.301(a),(c); 264a.301 | Provide description of soil portion of bottom composite liner, including its classification, thickness, hydraulic conductivity, and material specifications. |  |  |
| D-6e(2)(a) Material Testing Data | 270.21(b)(1); 264.301(c); 264a.301 | Provide complete results for index tests, laboratory and/or in situ permeability tests, strength tests, consolidation tests, and shrink-swell properties of soil liner material. Discuss potential for dispersion and piping of soil due to flow of liquid through soil liner layer. |  |  |
| D-6e(2)(b) Soil Liner Compatibility Data | 270.21(b)(1); 264.301(a)(1)(i); 264.301(c)(3)(iii) | Provide complete test results of permeability testing of soil liner material using representative of leachate from surface impoundment. |  |  |
| D-6e(2)(c) Soil Liner Strength | 270.21(b)(1); 264.301(a)(1)(i); 264.301(c)(3)(iii) | Demonstrate that soil liner has sufficient strength to support loads/stresses computed in item D-4c(3). |  |  |
| D-6f Liner System, Leachate Collection/Detection Systems | 270.21(b)(1); 264.301(a)(2); 264.301(c)(2),(3) |  |  |  |
| D-6f(1) System Operation and Design | 270.21(b)(1); 264.301(a)(2); 264.301(c)(2),(3) | Describe design features of leachate detection system and how system will function to detect any leakage through either liner in timely manner. |  |  |
| D-6f(2) Drainage Material | 270.21(b)(1); 264.301(a)(2),(c)(3) (ii) | Describe leachate detection system drainage material. |  |  |
| D-6f(3) Grading and Drainage | 270.21(b)(1); 264.301(a)(2),(c)(2),(3) | Indicate slopes of leachate detection system and provide contour plan for system along with plan showing layout and spacing of piping system and any sumps, pumps, etc. Demonstrate that leak detection system is appropriately graded to assure that leakage at any point in liner system is detected in timely manner. |  |  |
| D-6f(4) Maximum Leachate Head | 270.21(b)(1); 264.301(a)(2),(c)(2) |  |  |  |
| D-6f(5) Systems Compatibility | 270.21(b)(1); 264.301(a)(2)(i)(A), (c)(3)(iii) |  |  |  |
| D-6f(6) Systems Strength | 270.21(b)(1); 264.301(a)(2)(i)(B), (c)(3)(iii) |  |  |  |
| D-6f(6)(a) Stability of Drainage Layers | 270.21(b)(1); 264.301(a)(2)(i)(B), (c)(3)(iii) |  |  |  |
| D-6f(6)(b) Strength of Piping | 270.21(b)(1); 264.301(a)(2)(i)(B), (c)(3)(iii) | Demonstrate that pipe used in piping systems have sufficient strength to support loads as computed in item D-6c(3). |  |  |
| D-6f(7) Prevention of Clogging | 270.21(b)(1); 264.301(a)(2)(ii), (c)(3)(iv) |  |  |  |
| D-6f(8) Liquid Removal | 270.21(b)(1); 264.301(c)(3)(v),(4) |  |  |  |
| D-6f(9) Location Relative to Water Table | 270.21(b)(1)(iii); 264.301(c)(5); 264a.301 |  |  |  |
| D-6g Liner System, Construction and Maintenance |  |  |  |  |
| D-6g(1) Material Specifications |  |  |  |  |
| D-6g(1)(a) Synthetic Liners | 270.21(b)(1); 264.301(a)(1); 264a.301 | Provide detailed material specifications for specific synthetic liner or liners to be used. |  |  |
| D-6g(1)(b) Soil Liners | 270.21(b)(1); 264.301(a)(1); 264a.301 | For soil liners constructed of borrowed material, provide specifications. For soil liners using in-place soil, provide specifications to be used to assure that all existing materials meet requirements of liner design. |  |  |
| D-6g(1)(c) Leachate Collection/Detection Systems | 270.21(b)(1); 264.301(a),(c) | Provide material specifications for drainage layer material, filter fabric or filter layer, piping, and sumps. |  |  |
| D-6g(2) Construction Specifications |  |  |  |  |
| D-6g(2)(a) Liner System Foundation | 270.21(b)(1); 264.301(a)(1); 264.303(a); 264a.301 | Provide construction specifications of foundation installation procedures. For units that use in-place material for liner system foundation, provide construction specifications for preparation of foundation. |  |  |
| D-6g(2)(b) Soil Liner | 264a.301; 270.21(b)(1); 264.301(a)(1); 264.303(a)(2) | Describe procedures for installing soil liner. |  |  |
| D-6g(2)(c) Synthetic Liners | 270.21(b)(1); 264a.301; 264.301(a)(1); 264.303(a)(1) | Provide construction specifications for placement of synthetic liners. |  |  |
| D-6g(2)(d) Leachate Collection/Detection Systems | 270.31(b)(1); 264.301(a),(c) | Provide construction specifications for placement of all components of leachate collection/detection systems. |  |  |
| D-6g(3) Certified Quality Auditor (CQA) Program | 270.21(b)(1); 270.30(k)(2); 264.19; 264.303(a) | Provide complete details of CQA program to be used during construction of liner system to assure that it is built as designed. |  |  |
| D-6g(4) Maintenance Procedures for Leachate Collection/Detection Systems | 270.21(b)(1); 264.301(a),(c) | Describe anticipated maintenance activities that will be used to assure proper operation of leachate collection/detection systems throughout landfill’s expected life. |  |  |
| D-6g(5) Liner Repairs During Operations | 270.21(b)(1); 264.301(a) | Describe methods that will be used to repair any damage to liner that occurs while landfill is in operation during placement of waste (such as a dozer ripping the liner). |  |  |
| D-6h Action Leakage Rate | 270.21(b)(1)(v); 264.302 |  |  |  |
| D-6h(1) Determination of the Action Leakage Rate | 270.21(b)(1)(v); 264.302(a) |  |  |  |
| D-6h(2) Monitoring the Leakage | 270.21(b)(1)(v); 264.302(b) | To determine if action leakage rate has been exceeded, owner/operator must convert required leachate flow rate monitoring data to average daily flow rate for each sump. This average daily flow rate must be calculated weekly during active life of facility and closure period, and monthly during post-closure care period. |  |  |
| D-6i Leakage Response Action Plan | 270.21(b)(1)(v); 264.304 |  |  |  |
| D-6i(1) Response Actions | 270.21(b)(1)(v); 264.304(a) |  |  |  |
| D-6i(2) Leak and/or Remedial Determinations | 270.21(b)(1)(v); 264.304(b),(c) |  |  |  |
| D-6i(3) Notifications | 270.21(b)(1)(v); 264.304(b) |  |  |  |
| D-6j Runon and Runoff Control Systems |  |  |  |  |
| D-6j(1) Runon Control System | 270.21(b)(2); 264.301(g) | Describe system that will be used to prevent runon onto active portions of landfills. |  |  |
| D-6j(1)(a) Design and Performance | 270.21(b)(2); 264.301(g) | Describe runon control system design and how that design prevents runon from reaching active portions of site. Provide plan view. |  |  |
| D-6j(1)(b) Calculation of Peak Flow | 270.21(b)(1); 264.301(g) | Identify peak surface water flow expected to result from 2-year design storm. Provide copies of calculations and data. |  |  |
| D-6j(2) Runoff Control System | 270.21(b)(3); 264.301(h) | Describe runoff control system to be used to collect and control runoff from active portions. |  |  |
| D-6j(2)(a) Design and Performance | 270.21(b)(3); 264.301(h) | Describe runoff collection and control system design. Indicate fate of collected runoff that is considered hazardous waste until tested and/or treated. |  |  |
| D-6j(2)(b) Calculation of Peak Flow | 270.21(b)(3); 264.301(h) | Identify total runoff volume expected to result from at least a 24-hour, 25-year storm event. Provide copies of calculations and data. |  |  |
| D-6j(3) Management of Collection and Holding Units | 270.21(b)(4); 264.301(i) | Describe how collection and holding facilities associated with runon and runoff control systems will be emptied or otherwise managed expeditiously after storms to maintain system design capacity. Describe fate of liquids discharged from these systems. |  |  |
| D-6j(4) Construction | 270.21(b)(2),(3); 264.301(g),(h) | Provide detailed construction and material specifications for runon and runoff control systems. |  |  |
| D-6j(5) Maintenance | 270.21(b)(2),(3); 264.301(g),(h) | Describe any maintenance activities required to assure continued proper operations of runon and runoff control systems throughout active life of unit. |  |  |
| D-6k Control of Wind Dispersal | 270.21(b)(5); 264.301(j) |  |  |  |
| D-6L Liquids in Landfills |  |  |  |  |
| D-6L(1) Bulk or Noncontainerized Free Liquids | 270.21(h); 264.314 | Describe procedures that will be used to ensure that no bulk or noncontainerized liquid hazardous waste or waste with free liquids will be placed in landfill. Demonstrate, by paint filter test, Method 9095, that no free liquids will be placed in landfill. |  |  |
| D-6L(2) Containers Holding Free Liquids | 270.21(h); 264.314(d) | For facilities that intend to dispose of containers holding free liquids, describe how free liquids will be removed from containers or stabilized within container before container is placed in landfill. If liquid is removed, container must be backfilled or crushed. |  |  |
| D-6L(3) Restriction to Small Containers | 270.21(h); 264.314(d)(2) | If small containers are to be disposed of in landfill, demonstrate by indicating container volume, that containers will be very small (such as ampules). |  |  |
| D-6L(4) Nonstorage Containers | 270.21(h); 264.314(d)(3) | If nonstorage containers are to be disposed of in landfill, demonstrate by describing the containers designed to hold free liquids for use other than storage (e.g., batteries, capacitors). |  |  |
| D-6L(5) Lab Packs | 270.21(h); 264.314(d)(4) | Describe how it will be assured that lab packs to be landfilled containing free liquids meet requirements for lab packs. |  |  |
| D-6L(5)(a) Inside Containers | 270.21(h); 264.314(d)(4); 264.316(a) |  |  |  |
| D-6L(5)(b) Overpack | 270.21(h); 264.314(d)(4); 264.316(b) | Demonstrate that overpacking consists of metal, Department of Transportation (DOT) containers, metal DOT containers, with open heads no larger than 110 gallons; and sufficient sorbent material determined to be non-biodegradable to completely sorb all liquid contents of inside container. |  |  |
| D-6L(5)(c) Sorbent Material | 270.21(h); 264.314(d)(4),(e) 264.316 | Demonstrate that sorbent materials used are no capable of reacting dangerously with, being decomposed by, or being ignited by contents of inside containers. |  |  |
| D-6L(5)(d) Incompatible Wastes | 270.21(h); 264.314(d)(4); 264.316(d) | Demonstrate that incompatible waste will not be placed in same outside containers. |  |  |
| D-6L(5)(e) Reactive Wastes | 270.21(h); 264.314(d)(4); 264.316(d) | Demonstrate that incompatible waste will not be placed in same outside containers. |  |  |
| D-6m Containerized Wastes | 270.21(i); 264.315 |  |  |  |
| D-6n Special Waste Management Plan for Landfills Containing Wastes F020, F021, F022, F023, F026, and F027 | 270.21(j); 264.317 | Provide plan for waste management in this special facility. Plan must address the following factors. |  |  |
| D-6n(1) Waste Descriptions | 270.21(j)(1); 264.317(a)(1) | Identify volume, physical, and chemical characteristics of waste, including potential to migrate through soil or volatilize or escape into atmosphere. |  |  |
| D-6n(2) Soil Description | 270.21(j)(2); 264.317(a)(2) | Describe attenuative properties of underlying and surrounding soils or other materials. |  |  |
| D-6n(3) Mobilizing Properties | 270.21(j)(2); 264.317(a)(2) | Describe mobilizing properties of other materials codisposed of with this waste. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - LAND TREATMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-7 Land Treatment | 270.20; 264.270 - 264.283;264a.273; 264a.276 |  |  |  |
| D-7a Treatment Demonstration | 270.20(a); 264.272 |  |  |  |
| D-7a(1) Demonstration Wastes | 270.20(a)(1); 264.272(a),(c)(1)(i) | Describe waste used in demonstration and waste to be treated during normal operation. Identify concentrations of all hazardous constituents reasonably expected to be present in both wastes. |  |  |
| D-7a(2) Demonstration Data Sources | 270.20(a)(2); 264.272(b) | Describe source of data used for treatment demonstration and provide available determinations. |  |  |
| D-7a(2)(a) Existing Literature | 270.20(a)(2); 264.272(b) | If existing literature is used to demonstrate treatment, submit brief written review of scientific literature and previous studies that contain pertinent information. Information sources should be properly referenced. In general, existing literature will not be acceptable as demonstration unless it can be shown that site and waste characteristics are identical to those in literature. |  |  |
| D-7a(2)(b) Operating Data | 270.20(a)(2); 264.272(b) | Provide any operating data gathered from units to be permitted, including application rate data and operating records. |  |  |
| D-7a(3) Laboratory/Field Testing Programs | 270.20(a)(3); 264.272(b),(c) | Field and laboratory tests to be used for demonstration must be thoroughly described. Include interpretive discussions as appropriate. |  |  |
| D-7a(3)(a) Toxicity Testing | 270.20(a)(2); 264.272(b) | Describe acute toxicity test procedures used to estimate impact of waste application or waste constituents on soil biota responsible for waste treatment. |  |  |
| D-7a(3)(b) Field Plot Testing | 270.20(a)(2),(3); 264.272(b),(c) | Describe field plot studies used to demonstrate treatability of waste(s) or waste constituents. |  |  |
| D-7a(3)(c) Laboratory Testing | 270.20(b)(2),(3); 264.272(b),(c) | Describe laboratory test methods used to demonstrate treatability of waste(s) or waste constituents. |  |  |
| D-7b Land Treatment Program | 270.20(b); 264.271 | Describe characteristics and operating conditions of land treatment unit(s) to be permitted. |  |  |
| D-7b(1) List of Wastes | 270.20(b)(1); 264.271(b) |  |  |  |
| D-7b(2) Operating Procedures | 270.20(b)(2); 264.273(a); 264a.273 | Describe operating procedures used to assure uniform and complete degradation, transformation, and immobilization. |  |  |
| D-7b(2)(a) Waste Application Rates | 270.20(b)(2)(i); 264.273(a)(1) | Identify rate and frequency of waste application and concentration of limiting constituents in waste. |  |  |
| D-7b(2)(b) Waste Application Methods | 270.20(b)(2)(i); 264.273(a)(1); 264a.273 | Describe method(s) used to apply and incorporate waste into treatment zone. |  |  |
| D-7b(2)(c) Control of Soil pH | 270.20(b)(2)(ii); 264.273(a)(2) | Identify acceptable limits of soil pH and describe rationale for those limits. Describe how soil pH will be measured and adjusted, including a schedule for the same. |  |  |
| D-7b(2)(d) Enhancement of Microbial or Chemical Reactions | 270.20(b)(2)(iii); 264.273(a)(3) | Describe measures used to enhance treatment, including method and frequency of such measures (e.g., fertilization, microbial inoculations, soil aeration). |  |  |
| D-7b(2)(e) Control of Soil Moisture | 270.20(b)(2)(iv); 264.273(a)(4); 264a.273(2),(3) | Identify limits on soil moisture content. Describe how soil moisture will be monitored and adjusted, if necessary. |  |  |
| D-7c Unsaturated Zone Monitoring Plan | 270.20(b)(3); 264.278 | Submit unsaturated zone monitoring plan describing measures used to determine if hazardous wastes have migrated from treatment zone. |  |  |
| D-7c(1) Soil-Pore Liquid Monitoring | 270.20(b)(3); 264.278 | Describe program for sampling and analysis of soil-pore liquid to detect migration of dissolved constituents below treatment zone. |  |  |
| D-7c(1)(a) Sampling Location | 270.20(b)(3)(ii); 264.278(b), (d) | Identify sampling locations and indicate that samples will be collected immediately below treatment zone. |  |  |
| D-7c(1)(b) Sampling Frequency | 270.20(b)(3)(i); 264.278(e) | Provide schedule for sampling soil-pore liquid. |  |  |
| D-7c(1)(c) Sampling Equipment | 270.20(b)(3)(i); 264.278(e) | Identify equipment used to obtain soil-pore liquid samples. |  |  |
| D-7c(1)(d) Sampling Equipment Installation | 270.20(b)(3)(i); 264.278(e) | Describe procedures used to install soil-pore liquid monitoring devices. |  |  |
| D-7c(1)(e) Sampling Procedures | 270.20(b)(3)(i); 264.278(e)(1),(2) |  |  |  |
| D-7c(1)(f) Analytical Procedures | 270.20(b)(3)(iii); 264.278(e)(3) | Identify analytical procedures used to determine concentration of hazardous constituents in soil-pore liquid samples. |  |  |
| D-7c(1)(g) Chain of Custody | 270.20(b)(3)(iv); 264.278(e)(4) |  |  |  |
| D-7c(1)(h) Background Values | 270.20(b)(3)(v); 264.278(c) | Describe sampling and analytical program used to establish background soil-pore liquid concentrations of hazardous constituents. Provide background data, if available. |  |  |
| D-7c(1)(i) Statistical Methods | 270.20(b)(3)(vi); 264.278(f) | Describe statistical methods that will be used to determine differences between background and treatment zone concentrations of hazardous constituents. |  |  |
| D-7c(1)(j) Justification of Principle Hazardous Constituents | 270.20(b)(3)(vii); 264.278(a)(2) | Provide suggested list of 261 Appendix VIII hazardous constituents to be monitored for in soil-pore liquids. |  |  |
| D-7c(2) Soil Core Monitoring | 270.20(b)(3); 264.278 | Describe program for monitoring soil cores to detect migration of hazardous constituents below treatment zone. |  |  |
| D-7c(2)(a) Sampling Location | 270.20(b)(3)(ii); 264.278(b),(d) | Identify sampling locations and indicate that soil cores will be collected immediately below treatment zone. |  |  |
| D-7c(2)(b) Sampling Frequency | 270.20(b)(3)(i); 264.278(e) | Provide schedule for sampling soil. |  |  |
| D-7c(2)(c) Sampling Equipment | 270.20(b)(3)(i); 264.278(e) | Identify equipment used to sample soil cores. |  |  |
| D-7c(2)(d) Sampling Procedures | 270.20(b)(3)(i); 264.278(e)(1),(2) |  |  |  |
| D-7c(2)(e) Analytical Procedures | 270.20(b)(3)(iii); 264.278(e)(3) | Identify analytical methods used to determine concentration of hazardous constituents in soil core samples. |  |  |
| D-7c(2)(f) Chain of Custody | 270.20(b)(3)(iv); 264.278(e)(4) |  |  |  |
| D-7c(2)(g) Background Values | 270.20(b)(3)(v); 264.278(c) | Describe sampling and analytical program used to establish background soil core concentrations of hazardous constituents. Provide background data, if available. |  |  |
| D-7c(2)(h) Statistical Methods | 270.20(b)(3)(vi); 264.278(f) | Describe statistical methods that will be used to determine differences between background and treatment zone concentrations of hazardous constituents. |  |  |
| D-7c(2)(i) Justification of Principal Hazardous Constituents | 270.20(b)(3)(vii); 264.278(a)(2) | Provide suggested list of 261 Appendix VIII hazardous constituents to be monitored for in soil core samples. |  |  |
| D-7d Treatment Zone Description | 270.20(b)(5); 264.271(c) | Identify dimensions of treatment zone. |  |  |
| D-7d(1) Horizontal and Vertical Dimensions | 270.20(b)(5); 264.271(c) |  |  |  |
| D-7d(2) Soil Survey | 270.20(b)(2); 264.272(c)(1)(iv) | Provide map or plat plan delineating horizontal boundaries of treatment zone and all soil series occurring within treatment zone. |  |  |
| D-7d(3) Soil Series Descriptions | 270.20(b)(2); 264.272(c)(1)(iv) | Submit description of each soil series identified within treatment zone. |  |  |
| D-7d(4) Soil Sampling Data | 270.20(b)(2); 264.272(1)(iv) |  |  |  |
| D-7d(5) Seasonal High Water Table | 270.20(b); 264.271(c)(2) | Identify depth to seasonal high water table and source of that data. |  |  |
| D-7e Unit Design, Construction, Operation, and Maintenance | 270.20(c); 264.273; 264a.273 | Describe design, construction, operation, and maintenance of runon, runoff, and wind dispersal controls. |  |  |
| D-7e(1) Runon Control | 270.20(c)(1); 264.273(c) | Submit scale drawing of unit showing any runon controls used. |  |  |
| D-7e(2) Runoff Control | 270.20(c)(1); 264.273(c) | Describe runoff collection and control system. |  |  |
| D-7e(3) Minimizing Hazardous Constituent Runoff | 270.20(c)(3); 264.273(b) |  |  |  |
| D-7e(4) Management of Accumulated Runon and Runoff | 270.20(c)(4); 264.273(e); 264a.273(2) | Describe fate of collected surface water, including sampling and analysis protocols for determining contaminant levels. |  |  |
| D-7e(5) Control of Wind Dispersal | 270.20(c)(6); 264.273(f) |  |  |  |
| D-7f Food-Chain Crops | 270.20(d); 264.276; 264a.276 | Demonstrate that there is no substantial risk to human health or environment caused by growth of food-chain crops on unit. |  |  |
| D-7f(1) Food-Chain Crop Demonstration | 270.20(d); 264.276(a)(1); 264a.276 |  |  |  |
| D-7f(1)(a) Demonstration Basis | 270.20(d)(1),(2); 264.276(a)(3)(i); 264a.276 | Show that demonstration results will be representative of unit to be permitted. |  |  |
| D-7f(1)(b) Test Procedures | 270.20(d)(3); 264.276(a)(3)(ii) | Describe procedures used in any tests referenced or conducted. |  |  |
| D-7f(2) Cadmium-Bearing Wastes | 270.20(e); 264.276(b); 264a.276 |  |  |  |
| D-7f(2)(a) Crops for Human Consumption | 270.20(e); 264.276(b)(1); 264a.276 | If crops are to be grown for human consumption, provide: soil pH; soil pH controls; cadmium-loading rate; and soil cation exchange capacity. |  |  |
| D-7f(2)(b) Animal Feed | 270.20(e); 264.276(b)(2) | If only animal feed is to be grown, provide soil pH and soil pH controls. Provide copy of operating plan demonstrating how animal feed will be distributed to preclude ingestion by humans, including control of alternative land use. |  |  |
| D-7g Special Waste Management Plan for Land Treatment Units Containing Wastes F020, F021, F022, F023, F026, and F027 | 270.20(i); 264.283 | Provide plan describing how land treatment units containing referenced waste are, or will be, designed, constructed, operated, and maintained to protect human health and environment. |  |  |
| D-7g(1) Waste Description | 270.20(i)(1); 264.283(a)(1) |  |  |  |
| D-7g(2) Soil Description | 270.20(i)(2); 264.283(a)(2) |  |  |  |
| D-7g(3) Mobilizing Properties | 270.20(i)(3); 264.283(a)(3) |  |  |  |
| D-7g(4) Additional Management Techniques | 270.20(i)(4); 264.283(a)(4) |  |  |  |
| D-7h Incompatible Wastes | 270.20(h); 264.282 | Indicate that incompatible waste will not be placed in, or on, the same treatment zone. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - MISCELLANEOUS TREATMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-8 Miscellaneous Units | 270.23; 264.601 | - Identify all miscellaneous units that treat, store, or dispose of hazardous waste at facility, but do not fit current definition of container, tank, surface impoundment, etc. These units may include: geologic repositories, deactivated missile silos, thermal treatment units other than incinerators, boilers, or industrial furnaces units, open burning and open detonating explosive waste certain chemical/physical/biological treatment units. |  |  |
| D-8a Description of Miscellaneous Units | 270.23(a) |  |  |  |
| D-8b Waste Characterization | 270.23; 264.601(a)(1), (b)(1),(c)(1) | Provide information on volume and concentration of waste in order to determine release potential. |  |  |
| D-8c Treatment Effectiveness | 270.23(d) |  |  |  |
| D-8d Environmental Performance Standards for Miscellaneous Units |  | Environmental performance standards must be established and maintained to protect human health and environment. |  |  |
| D-8d(1) Protection of Groundwater and Subsurface Environment | 270.23(b),(c); 264.601(a) |  |  |  |
| D-8d(1)(a) Environmental Assessment | 270.23(b),(c); 264.601(a) | Applicant must conduct assessment of potential for releases to groundwater or the subsurface environment. Both saturated and unsaturated zones must be considered in evaluating potential for subsurface migration. |  |  |
| D-8d(1)(b) Performance Standards | 270.23(b); 264.601 | Based on assessments, performance standards must be developed and maintained. |  |  |
| D-8d(2) Protection of Surface Water, Wetlands, and Soil Surfaces | 270.23(b),(c); 264.601(b) |  |  |  |
| D-8d(2)(a) Environmental Assessment | 270.23(b),(c); 264.601(b) | Applicant must conduct assessment of potential for releases to surface water, wetlands, or soil surface. |  |  |
| D-8d(2)(b) Performance Standards | 270.23; 264.601 | Based on assessments, performance standards must be developed and maintained. |  |  |
| D-8d(3) Protection of the Atmosphere | 270.23(b),(c); 264.601 |  |  |  |
| D-8d(3)(a) Environmental Assessment | 270.23(b),(c); 264.601(c) | Applicant must conduct assessment of potential for release to air. |  |  |
| D-8d(3)(b) Performance Standards | 270.23; 264.601 | Based on assessments, performance standards must be developed and maintained. |  |  |
| D-8e Monitoring, Analysis, Inspection, Response, Reporting, and Corrective Action | 270.23(a); 264.602 |  |  |  |
| D-8e(1) Elements of a Monitoring Program | 270.23(a); 264.602 | Monitoring program must include procedures for sampling, analysis, and evaluation of data, suitable response procedures, and a regular inspection schedule. |  |  |
| D-8e(2) Air Monitoring Alternatives | 270.23(a); 264.602 | For situations in which ambient air monitoring would be unsafe or impractical, possible alternatives may include analysis of waste, emissions measurements, and periodic monitoring with portable detectors. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - BOILERS/INDUSTRIAL FURNACES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-9 Boilers and Industrial Furnaces (BIF) |  |  |  |  |
| D-9a Waivers/Exemptions | 270.22(a)(2)(i); 266.104(a)(4); 266.110 | If applying for waiver or exemption, provide information demonstrating compliance with requirements outlined in this section. |  |  |
| D-9a(1) Waiver of Destruction and Removal Efficiency (DRE) Trial Burn for Boilers | 270.22(a)(2)(i); 266.104(a)(4); 266.110 |  |  |  |
| D-9a(2) Low Risk Waste Exemption | 270.22(a)(2)(ii); 266.104(a)(5); 266.109(a) | The DRE standard for a BIF may be waived provided certain criteria listed in regulatory citation are met and documented. |  |  |
| D-9a(3) Waiver of Particulate Matter Standard | 270.22(a)(4); 266.109(b) | The particulate matter standard of 266.105 and trial burn for particulate matter may be waived if: the BIF complies with Tier I or Adjusted Tier I metals feed rate screening limits under 266.106(b) or (e) and submits documentation showing conformance with trial burn waiver under checklist Section D-9a(4) below; and BIF meets requirements of low risk waste exemption under checklist Section D-9a(2) above. |  |  |
| D-9a(4) Waiver of Trial Burn for Metals | 270.22(a)(3); 266.106(b),(e) |  |  |  |
| D-9a(5) Waiver of Trial Burn for Hydrogen Chloride (HCl)/Cl2 | 270.22(a)(5); 266.107(b),(e) |  |  |  |
| D-9b Pretrial Burn Requirements for New BIFs | 270.66(b)(1); 266.102(d)(4)(i); 266.102(e) | Time required to bring new BIF to point of operational readiness for trial burn must be minimum necessary and cannot exceed 720 hours, or up to 1,440 hours if applicant shows good cause for requiring an extension. |  |  |
| D-9b(1) Pretrial Burn Requirements for New BIFs - Organic Emission Standards | 270.66(b)(1)(i); 266.102(e)(2); 266.104(d),(e) |  |  |  |
| D-9b(2) Pretrial Burn Requirements for New BIFs - Particle Matter Emissions Standards | 270.66(b)(1)(i); 266.105 |  |  |  |
| D-9b(3) Pretrial Burn Requirements for New BIFs - Metal Emissions Standards | 270.66(b)(1)(i); 266.102(e)(4)(i), (ii); 266.106 |  |  |  |
| D-9b(4) Pretrial Burn Requirements for New BIFs - Alternative Metals Approach | 270.66(b); 266.102(e)(4)(iii); 266.106(f) | For conformance with alternative metals approach, description of operating conditions must: describe approach that will be used to comply; specify how approach ensures compliance with metals emissions standards of 266.106(c) and (d); specify how approach can be effectively implemented and monitored; and provide such other information as necessary to ensure that the standards of 266.106(c) or (d) are met. |  |  |
| D-9b(5) Pretrial Burn Requirements for New BIFs - Hydrogen Chloride/Chlorine Emission Standards | 270.66(b)(1)(i); 266.102(e)(5)(i); 266.107 |  |  |  |
| D-9b(6) Pretrial Burn Requirements for New BIFs - Fugitive Emissions | 270.66(b)(1)(i); 266.102(e)(7)(i) | Description of operating conditions must thoroughly describe method by which fugitive emissions will be controlled. |  |  |
| D-9b(7) Pretrial Burn Requirements for New BIFs - Automatic Waste Feed Cutoff | 270.66(b)(1)(i); 266.102(e)(7)(ii), (iii) |  |  |  |
| D-9b(8) Pretrial Burn Requirements for New BIFs - Monitoring Requirements | 270.66(b)(1)(i); 266.102(e)(8),(10) |  |  |  |
| D-9c Trial Burn Plan Requirements for All BIFs | 270.66(b)(2),(c), (e); 266.102(d)(4)(ii) |  |  |  |
| D-9d Trial Burn Results | 270.22(a)(6); 270.66(d),(f) 270a.66 | Results of trial burn, as specified in regulatory citation, must be submitted within 90 days of completing trial burn. The submittal must be certified on behalf of applicant by signature of a person authorized to sign a permit application or a report under 270.11. |  |  |
| D-9e Post-Trial Burn Requirements for New BIFs | 270.66(b)(3)(ii); 266.102(d)(4)(iii),(e) | - Post-trial burn requirements for new BIFs are the same as pretrial burn requirements for new BIFs with the following exceptions: No documentation of total burn hours is required; no limit to length of time for burning. Must submit statement identifying conditions necessary to operate in compliance. Must submit statement specifying that BIF will stop burning when changes in combustion properties or feed rates or BIF design or operating conditions deviate from approved post-trial burn period. |  |  |
| D-9f Data in Lieu of Trial Burn | 270.22(a)(6); 270.66(c)(3) | A BIF may seek exemption from trial burn requirements by submitting information provided by previous compliance testing of same device, or from compliance testing or trial or operational burns of similar BIFs burning similar hazardous waste under similar conditions. |  |  |
| D-9g Alternative Hydrocarbons (HC) Limit for Industrial Furnaces with Organic Matter in Raw Materials | 270.22(b); 266.104(f) |  |  |  |
| D-9h Alternative Metals Implementation Approach | 270.22(c); 266.106(f) | - For conformance with an alternative metals implementation approach, the information must: Describe approach that will be used to comply. Specify how approach ensures compliance with the metals emissions standards of 266.106(c) and (d). Specify how approach can be effectively implemented and monitored. Provide such other information as necessary to ensure that standards are met. |  |  |
| D-9i Monitoring Requirements | 270.22; 266.102(e)(6),(8) | Various parameters must be continuously monitored per 266.102(e)(6) while burning hazardous waste. Data must be maintained in operating record until closure of facility. |  |  |
| D-9j Automatic Waste Feed Cutoff System | 270.22(d); 266.102(e)(7)(ii) | All facilities must submit description of automatic waste feed cutoff system, including any pre-alarm systems that may be used. |  |  |
| D-9k Direct Transfer Standards | 270.22(e); 266.111; Part 264 Subparts I and J; Part 264a Subparts I and J | BIFs that directly feed hazardous waste from a transport vehicle to a BIF without use of a storage unit must submit a description of the direct transfer procedures that will be used, along with other information as specified in regulatory citation. |  |  |
| D-9k(1) Direct Transfer Standards - Containment System | 270.22(e); 264.175 | In areas where direct transfer vehicles are located, a complete description of containment system must be provided. |  |  |
| D-9k(2) Direct Transfer Standards - Condition of Containers | 270.22(e); 264.171 |  |  |  |
| D-9k(3) Direct Transfer Standards - Compatibility of Waste with Container | 270.22(e); 264.172 |  |  |  |
| D-9k(4) Direct Transfer Standards - Management of Containers | 270.22(e); 264.173; 264a.173 |  |  |  |
| D-9k(5) Direct Transfer Standards - Special Requirements of Ignitable or Reactive Waste | 270.22(e); 264.176 | Provide documentation of location of all containers holding ignitable/reactive waste. |  |  |
| D-9k(6) Direct Transfer Standards - Special Requirements of Incompatible Wastes | 270.22(e); 264.177 | Provide statement and description of procedures to ensure compliance with management standards for incompatible waste. |  |  |
| D-9k(7) Direct Transfer Standards - Closure | 270.22(e); 264.178 | Describe how all hazardous waste and hazardous waste residues will be removed from containment system at closure. |  |  |
| D-9k(8) Direct Transfer Standards - Secondary Containment Requirements | 270.22(e); 266.111(e) | Owners/operators must submit documentation demonstrating conformance with secondary containment requirements of 265.193(b),(c), and (f) - (h). |  |  |
| D-9L Bevill Residues | 270.22(f); 266.112; Part 266 Appendices VII and IX |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - CONTAINMENT BUILDINGS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-10 Containment Buildings | 270.14(a),(b); 270a.14; 264.1100 - 264.1102; 264a.1100; 264a.1101 |  |  |  |
| D-10a Containment Building Description | 270.14(a),(b); 264a.1100; 264.1100(a); 264.1101(a) |  |  |  |
| D-10a(1) Construction | 270.14(a),(b); 264a.1100; 264.1100(a); 264.1101(a) | Provide description of unit; include dimensions and materials of construction. |  |  |
| D-10a(2) Strength Requirements | 270.14(a),(b); 264a.1100; 264.1100(a); 264.1101(a) | Provide results of calculations defining maximum loads or stresses that will be placed on containment building system. |  |  |
| D-10a(3) Design Requirements for Units Not Managing Liquids | 270.14(a),(b) 264.1100(b); 264.1101(d) |  |  |  |
| D-10a(3)(a) Primary Barrier | 270.14(a),(b) 264.1100(a),(b); 264.1101(a)(4); 264a.1100 | Provide detailed description of primary barrier, and demonstrate that it is sufficiently durable to withstand movement of personnel, waste, and handling equipment within unit. |  |  |
| D-10a(4) Design Requirements for Units Managing Liquids | 270.14(a),(b) 264.1100(c); 264.1101(a)(4),(b) |  |  |  |
| D-10a(4)(a) Primary Barrier | 270.14(a),(b) 264.1100(c)(1); 264.1101(b)(1) | Describe how primary barrier is designed and constructed to prevent migration of hazardous constituents into barrier. |  |  |
| D-10a(4)(b) Liquid Collection System | 270.14(a),(b) 264.1100(c)(2); 264.1101(b)(3) | Describe in detail liquid collection system that must be designed and constructed of materials to minimize accumulation of liquid on primary barrier. |  |  |
| D-10a(4)(c) Secondary Containment System | 270.14(a),(b) 264.1100(c)(3) |  |  |  |
| D-10a(4)(c)(i) Leak Detection System | 270.14(a),(b) 264.1100(c)(3); 264.1101(a),(b)(3) | Describe design and operating features of leak detection system. |  |  |
| D-10a(4)(C)(ii) Secondary Barrier | 270.14(a),(b) 264.1100(b)(3); 264.1101(b)(3) | Describe how secondary barrier is designed and constructed to prevent migration of hazardous constituents into barrier. |  |  |
| D-10a(4)(d) Temporary Variance from Secondary Containment Requirements | 270.14(a),(b) 264.1101(b)(4); 264a.1101(i) |  |  |  |
| D-10a(4)(e) Waiver of Secondary Containment Requirements | 270.14(a),(b) 264.1101(e) |  |  |  |
| D-10a(5) Design of Units Managing Both Liquids and Nonliquids in the Same Unit | 270.14(a),(b) 264.1101(d) | Identify areas of containment building that are constructed both with and without secondary containment, if applicable. |  |  |
| D-10a(6) Compatibility of Structure with Wastes | 270.14(a),(b) 264.1101(a)(2), (b)(3)(iii) | Demonstrate that all surfaces in contact with hazardous waste, collected liquids, or leachate must be chemically compatible with those wastes. |  |  |
| D-10a(7) Fugitive Dust Emissions | 270.14(a),(b) 264.1100(d); 264.1101(c)(1)(iv); Part 60 Appendix A |  |  |  |
| D-10a(8) Structural Integrity Requirements | 270.14(a),(b) 264.1101(a)(2) |  |  |  |
| D-10a(9) Certification of Design | 270.14(a),(b) 264.1101(c)(2); 264a.1101(c)(2) |  |  |  |
| D-10b Containment Building Operations | 270.14(a),(b) 264.1101(c) |  |  |  |
| D-10b(1) Primary Barrier Integrity | 270.14(a),(b) 264.1101(b)(2)(ii), (c)(1)(i) |  |  |  |
| D-10b(2) Volume of Waste | 270.14(a),(b) 264.1101(c)(1)(ii) | Describe how owner/operator will maintain level of stored and/or treated hazardous waste within containment walls of unit so that height of any containment wall is not exceeded. |  |  |
| D-10b(3) Tracking of Waste Out of Unit | 270.14(a),(b) 264.1100(e); 264.1101(c)(1)(iii) |  |  |  |
| D-10b(4) Liquids Removal | 270.14(a),(b) 264.1101(b)(2)(ii), (b)(3) | Describe sumps and liquid removal methods for liquids collection and leak detection systems. Indicate fate of collected liquids and leachates, which are considered hazardous waste. |  |  |
| D-10b(5) Management of Incompatible Wastes | 270.14(a),(b) 264.1101(a)(3) | Indicate whether incompatible waste or treatment reagents will be placed in the unit or its secondary containment system. |  |  |
| D-10b(6) Management of Liquids and Nonliquids in the Same Unit | 270.14(a),(b) 264.1101(d)(2),(3) | For containment buildings that contain areas both with and without secondary containment, describe measures to prevent release of liquids or wet materials into areas without secondary containment. |  |  |
| D-10b(7) Fugitive Dust Emissions | 270.14(a),(b) 264.1100(d); 264.1101(c)(1)(iv); Part 60 Appendix A |  |  |  |
| D-10b(8) Treatment of Wastes | 270.14(a),(b); 264.1101(b)(3)(ii) | If treatment of waste is conducted in containment building, describe how treatment will be conducted to prevent release of liquids, wet materials, or liquid aerosols to other portions of building. |  |  |
| D-10b(9) Equipment Decontamination | 270.14(a),(b) 264.1101(c)(1)(iii) | Identify area used to decontaminate equipment and collect and manage any rinsate from decontamination. Identify fate of decontamination residues. |  |  |
| D-10c Containment Buildings as Tank Secondary Containment | 270.14(a),(b) 264.1101(b)(3)(iii) | Indicate whether containment building is intended to serve as a secondary containment system for a tank placed in the building. The unit must meet the requirements of 264.193(b), 264.193(c)(1), 264.193(c)(2), and 264.193(d)(1). |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION D. PROCESS INFORMATION - DRIP PADS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| D-11 Drip Pads | 270.26; 264.570 - .575; 264a.570 |  |  |  |
| D-11a Drip Pad Description | 270.26(c); 264.573(a) |  |  |  |
| D-11a(1) Construction | 270.26(c); 264.573(a)(1) - (4); 264.573(b)(1) - (3) | Provide a description of the unit including dimensions and materials of construction. Drip pads must: be constructed of nonearthen materials; be sloped to free-drain treated wood drippage, rain and other waters or wastes to the associated collection system; and, have a curb or berm around the perimeter. |  |  |
| D-11a(1)(a) Existing Drip Pads | 270.26(c); 264.572(a); 264.573(a)(4) | Existing drip pads must have a hydraulic conductivity of less than or equal to 1x10-7 centimeters per second. Provide a copy of the most recent written assessment of the drip pad. This assessment must be reviewed and certified by an independent, qualified registered professional engineer (PE). The assessment must be reviewed, updated and recertified annually. |  |  |
| D-11a(1)(b) New Drip Pads | 270.26(c); 264.572(b); 264.573(b) | New drip pads must have a synthetic liner installed below the pad. The liner must be constructed of materials that will prevent waste from being absorbed into the liner. A leakage detection system and a leakage collection system are also required. |  |  |
| D-11b(1) Preventive Maintenance | 270.26(c); 264.573(c) | Drip pads must be maintained to remain free of cracks, gaps, corrosion, etc., which could cause a release of hazardous waste. |  |  |
| D-11b(2) Prevent Runon and Runoff | 270.26(c); 264.573(d), (e), (L) | The drip pad and associated collection system must be operated to prevent runoff. Unless protected by a structure, the runon and runoff control systems must have the capacity to prevent flow onto the drip pad from a 24-hour, 25-year storm. All collection systems must be emptied as soon as possible after storms to maintain design capacity. |  |  |
| D-11b(3) Certification | 270.26(c); 264.573(g) | Provide certification from a qualified, registered PE stating the drip pad meets the requirements of section 264.573. |  |  |
| D-11b(4) Maintaining Collection System | 270.26(c); 264.573(h) | Provide plan for removal of drippage and accumulated precipitation from collection system as necessary to prevent overflow. |  |  |
| D-11b(5) Cleaning Drip Pad Surface | 270.26(c); 264.573(i),(j) | Drip pad surface must be cleaned appropriately to allow weekly inspection of the entire surface and to minimize tracking of hazardous waste or hazardous waste constituents off the drip pad. |  |  |
| D-11b(6) Recordkeeping | 270.26(c); 264.573(k) | Maintain records sufficient to document that all treated wood is held on the pad following treatment in accordance with the requirements of this section. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION E. GROUNDWATER MONITORING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section and Requirement | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| E-1 Exemption from Groundwater Protection Requirements | 270.14(c) |  |  |  |
| E-1a Waste Piles | 270.18(b); 264.90(b)(2), (5) |  |  |  |
| E-1b Landfill | 270.14(c); 264.90(b)(2) |  |  |  |
| E-1c No Migration | 270.14(c); 264.90(b)(4) |  |  |  |
| E-1d Drip Pad | 270.26(b); 264.90(b)(2) |  |  |  |
| E-2 Interim Status Groundwater Monitoring Data | 270.14(c)(1) |  |  |  |
| E-2a Description of Wells | 270.14(c)(1) | A copy of topographic map provided for 270.14(b) on which location and identification of each interim status monitoring well is indicated. Details of design and construction of each interim status monitoring well. |  |  |
| E-2b Description of Sampling and Analysis Procedures | 270.14(c)(1); 265.92 | A copy of facility’s groundwater sampling and analysis plan. See Form 2510-FM-BWM0091 relating to the Hazardous Waste Monitoring Semi-Annual Report. See Form 2510-FM-BWM0062 relating to Commercial Hazardous Waste Facility Private Water Supply. |  |  |
| E-2c Monitoring Data | 270.14(c)(1); 265.92 | Provide all interim status monitoring results. |  |  |
| E-2d Statistical Procedures | 270.14(c)(1); 265.93 | Provide information relating to statistical procedures. |  |  |
| E-2e Groundwater Assessment Plan | 270.14(c)(1); 265.93(d)(2) | If required, based on statistical comparison results, provide specific plan for groundwater quality assessment program along with results obtained from implementation of plan. |  |  |
| E-3 General Hydrogeologic Information | 270.14(c)(2) | Include description of regional and site-specific geologic and hydrogeological setting. |  |  |
| E-4 Topographic Map Requirements | 270.14(c)(2), (3),(4)(i) |  |  |  |
| E-5 Contaminant Plume Description | 270.14(c)(2), (4),(7) | In some cases, contaminant plumes may be defined under groundwater quality assessment programs carried out during interim status period which may not address complete list of Appendix VIII constituents as required under 270.14(c)(4). Additional monitoring may be required to identify concentration of each Appendix VIII constituent in plume. |  |  |
| E-6 General Monitoring Program Requirements | 270.14(c)(5); 264.90(b)(4); 264.97; 264a.97 | See Form 2510-FM-BWM0092 relating to Hazardous Waste Monitoring Quarterly Report. |  |  |
| E-6a Description of Wells | 270.14(c)(5); 264.97(a),(b),(c) |  |  |  |
| E-6b Description of Sampling and Analysis Procedures | 270.14(c)(5); 264.97(d),(e),(f) |  |  |  |
| E-6c Procedures for Establishing Background Quality | 270.14(c)(5); 264.97(a)(1),(g); 264a.97 | See Form 2510-FM-BWM0033 relating to Hazardous Waste Monitoring Initial Background Report. |  |  |
| E-6d Statistical Procedures | 270.14(c)(5); 264.97(h), (i)(1),(5),(6) |  |  |  |
| E-6d(1) Parametric Analysis of Variance (ANOVA) | 270.14(c)(5); 264.97(h)(1), (i)(2) |  |  |  |
| E-6d(2) Nonparametric ANOVA (based on ranks) | 270.14(c)(5); 264.97(h)(2), (i)(2) |  |  |  |
| E-6d(3) Tolerance or Prediction Interval Procedure | 270.14(c)(5); 264.97(h)(3), (i)(4) |  |  |  |
| E-6d(4) Control Chart Approach | 270.14(c)(5); 264.97(h)(4), (i)(3) |  |  |  |
| E-6d(5) Alternative Approach | 270.14(c)(5); 264.97(h)(5),(i) |  |  |  |
| E-7 Detection Monitoring Program | 270.14(c)(6); 264.91(a)(4); 264.98 |  |  |  |
| E-7a Indicator Parameters, Waste Constituents, Reaction Products to be Monitored | 270.14(c)(6)(i); 264.98(a) |  |  |  |
| E-7b Groundwater Monitoring System | 270.14(c)(6)(ii); 264.97(a)(2),(b), (c); 264.98(b); 264a.97 | Identify number, location, and depth of each well, and describe well construction materials. |  |  |
| E-7c Background Groundwater Concentration Values for Proposed Parameters | 270.14(c)(6)(iii); 264.97(g); 264.98(c), (d); 264a.97 |  |  |  |
| E-7d Proposed Sampling and Analysis Procedures | 270.14(c)(6)(iv); 264.97(d),(e),(f); 264.98(d),(e), (f); 264a.97 | See Form 2510-FM-BWM0091 relating to Hazardous Waste Monitoring Semi-Annual Report. |  |  |
| E-7e Statistically Significant Increase in any Constituent or Parameter Identified at any Compliance Point Monitoring Well | 270.14(c)(6); 264.98(g); Part 264 Appendix IX |  |  |  |
| E-8 Compliance Monitoring Program | 270.14(c)(7); 264.99 |  |  |  |
| E-8a Waste Description | 270.14(c)(7)(i) | Description must include historical records of volumes, types, and chemical composition of waste placed in units in waste management areas. |  |  |
| E-8b Characterization of Contaminated Groundwater | 270.14(c)(7)(ii) | For each well at point of compliance and for each background well, provide concentrations of each constituent in 261 Appendix VIII, major cations and anions, and constituents listed in Table 1 of 264.94, if not already mentioned above. |  |  |
| E-8c Hazardous Constituents to be Monitored in Compliance Program | 270.14(c)(7)(iii); 264.98(g)(3); 264.99(a)(1) |  |  |  |
| E-8d Concentration Limits | 270.14(c)(7)(iv); 264.94, 264.97(g),(h); 264.99(a)(2); 264a.97 |  |  |  |
| E-8e Alternate Concentration Limits | 270.14(c)(7)(iv); 264.94(b); 264.99(a)(2) | Provide justification for establishing alternate concentration limits. Justification must address the following two factors. |  |  |
| E-8e(1) Adverse Effects on Groundwater Quality | 270.14(c)(7)(iv); 264.94(b)(1) |  |  |  |
| E-8e(2) Potential Adverse Effects | 270.14(c)(7)(iv); 264.94(b)(2) |  |  |  |
| E-8f Engineering Report Describing Groundwater Monitoring Systems | 270.14(c)(7)(v); 264.95; 264.97(a)(2), (b),(c); 264.99(b) | Provide details supporting representative nature of groundwater quality at background monitoring points and compliance monitoring point. |  |  |
| E-8g Proposed Sampling and Statistical Analysis Procedures for Groundwater Data | 270.14(c)(7)(vi); 264.97(d),(e),(f); 264a.97; 264.99(c) - (g) |  |  |  |
| E-8h Groundwater Protection Standard Exceeded at Compliance Point Monitoring Well | 270.14(c)(8); 264.99(h),(i) |  |  |  |
| E-9 Corrective Action Program | 270.14(c)(8); 264.99(j); 264.100 |  |  |  |
| E-9a Characterization of Contaminated Groundwater | 270.14(c)(8)(i) | For each well at point of compliance and for each background well, provide concentrations of each constituent in 261 Appendix VIII, major cations and anions, and constituents listed in Table 1 of 264.94, if not already determined by the above. |  |  |
| E-9b Concentration Limits | 270.14(c)(8)(ii); 264.94; 264.100(a)(2) |  |  |  |
| E-9c Alternate Concentration Limits | 270.14(c)(8)(ii); 264.94(b); 264.100(a)(2) | Provide justification for establishing alternate concentration limits. Justification must address the following two factors. |  |  |
| E-9c(1) Adverse Effects on Groundwater Quality | 270.14(c)(8); 264.94(b)(1) |  |  |  |
| E-9c(2) Potential Adverse Effects | 270.14(c)(8); 264.94(b)(2) |  |  |  |
| E-9d Corrective Action Plan | 270.14(c)(8)(iii); 264.100(b) | Provide detailed plans and engineering report on corrective actions proposed for facility, including maps of engineered structures, construction details, plans for removing waste, description of treatment technologies, effectiveness of correction program, description of reinjection system, additional hydrogeologic data, operation and maintenance plans, and closure and post-closure plans. |  |  |
| E-9e Groundwater Monitoring Program | 270.14(c)(8)(iv); 264.100(d) |  |  |  |
| E-9e(1) Description of Monitoring System | 270.14(c)(7)(v),(8) |  |  |  |
| E-9e(2) Description of Sampling and Analysis Procedures | 270.14(c)(7)(v),(8) |  |  |  |
| E-9e(3) Monitoring Data and Statistical Analysis Procedures | 270.14(c)(7)(v),(8) |  |  |  |
| E-9e(4) Reporting Requirements | 270.14(c)(7); 264.100(g) |  |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.

| CHECKLIST FOR REVIEW OF RCRA PART B PERMIT APPLICATIONS SECTION F. PROCEDURES TO PREVENT HAZARDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Section and Requirement |  | Pennsylvania (25 Pa. Code) or Incorporated Federal (40 CFR) Regulation | Review Considerationa | Location in Applicationb | See Attached Comment Numberc |
| F-1 | Security | 270.14(b)(4); 264.14 |  |  |  |
| F-1a | Security Procedures and Equipment | 270.14(b)(4); 264.14 | Unless waiver is granted, facility must have surveillance system or a barrier to entry. |  |  |
| F-1a(1) | 24-Hour Surveillance System | 270.14(b)(4); 264.14 | Monitor/camera, guards, or personnel must continuously monitor or control access to active parts of facility. |  |  |
| F-1a(2)(a) | Barrier | 270.14(b)(4); 264.14 | This item required if 24-hour surveillance system is not feasible. Describe artificial or natural barrier. |  |  |
| F-1a(2)(b) | Means to Control Entry | 270.14(b)(4); 264.14 | This item required if 24-hour surveillance system is not feasible. |  |  |
| F-1a(3) | Warning Signs | 270.14(b)(4); 264.14 | Signs in English must be posted at each entrance, and be legible from 25 feet. |  |  |
| F-1b | Waiver | 270.14(b)(4); 264.14 | Owner/operator must prevent unknowing entry, and minimize unauthorized entry of persons or livestock unless can demonstrate: |  |  |
| F-1b(1) | Injury to Intruder | 270.14(b)(4); 264.14 | Assure physical contact with waste, structure, or equipment will not injure unknowing intruder. |  |  |
| F-1b(2) | Violation Caused by Intruder | 270.14(b)(4); 264.14 | Assure disturbance of waste or equipment by unauthorized intruder will not cause a violation. |  |  |
| F-2 | Inspection Schedule | 270.14(b)(5); 264.15; 264a.15 | Inspection is required for monitoring equipment, safety emergency equipment, communication and alarm systems, decontamination equipment, security devices, and operating and structural equipment. |  |  |
| F-2a | General Inspection Requirements | 270.14(b)(5); 264.15(a),(b); 264.33; 264a.15 |  |  |  |
| F-2a(1) | Types of Problems | 270.14(b)(5); 264.15(b)(3) | Inspection checklist must identify types of problem. |  |  |
| F-2a(2) | Frequency of Inspections | 270.14(b)(5); 264.15(b)(4) | Based on rate of deterioration of equipment and probability of environmental or human health incident. |  |  |
| F-2a(3) | Schedule of Remedial Action | 270.14(b)(5); 264.15(c) | Owner/operator must immediately remedy any deterioration or malfunction of equipment or structures to ensure problem does not lead to environmental or human health hazard. |  |  |
| F-2a(4) | Inspection Log | 270.14(b)(5); 264.15(d) | Provide example log or summary. |  |  |
| F-2b | Specific Process Inspection Requirements | 270.14(b)(5) |  |  |  |
| F-2b(1) | Container Inspection | 270.14(b)(5); 264.174 | Inspect at least weekly. |  |  |
| F-2b(2) | Tank System Inspection | 270.14(b)(5); 264.195; 264a.195 | Owner/operator must develop schedule and inspect at least once daily. |  |  |
| F-2b(2)(a) | Tank System External Corrosion and Releases | 270.14(b)(5); 264.195(b)(1) | Owner/operator must inspect that aboveground portion and check for corrosion. |  |  |
| F-2b(2)(b) | Tank System Construction Materials and Surrounding Area | 270.14(b)(5); 264.195(b)(3) | Observe construction materials and area around external portion for signs of release of hazardous waste. |  |  |
| F-2b(2)(c) | Tank System Overfilling Control Equipment | 270.14(b)(5); 264.195(a) | Develop and follow schedule for inspection of overfill controls. |  |  |
| F-2b(2)(d) | Tank System Monitoring and Leak Detection Equipment | 270.14(b)(5); 264.195(b)(2) | Analyze data gathered from monitoring equipment to ensure tank is operating according to design. |  |  |
| F-2b(2)(e) | Tank System Cathodic Protection | 270.14(b)(5); 264.195(c) | Inspect according to schedule. |  |  |
| F-2b(3) | Waste Pile Inspection | 270.14(b)(5); 270.18(d); 264.254(b) | Describe how waste pile will be inspected daily and after storms. |  |  |
| F-2b(3)(a) | Runon and Runoff Control System | 270.14(b)(5); 264.254(b)(1) | Inspections should identify deterioration, malfunction, or improper operation of control system. |  |  |
| F-2b(3)(b) | Wind Dispersal System | 270.14(b)(5); 264.254(b)(2) | Facility should inspect proper function of wind dispersal system. |  |  |
| F-2b(3)(c) | Leachate Collection and Removal System | 270.14(b)(5); 270.18 (d); 264.254(b)(3),(c) | Determine whether there is leachate present in functioning double liner system. |  |  |
| F-2b(4) | Surface Impoundment Inspection | 270.14(b)(5); 270.17(c); 264.226(b),(c) | Describe how each surface impoundment will be inspected to meet requirements of monitoring and inspection and waiver requirement. |  |  |
| F-2b(4)(a) | Condition Assessment | 270.14(b)(5); 264.226(b) | Describe how surface impoundment will be inspected weekly and after storms. |  |  |
| F-2b(4)(a)(1) | Overtopping Control System | 270.14(b)(5); 264.226(b)(1) | Inspect for deteriorating, malfunction, or improper operation of control system. |  |  |
| F-2b(4)(a)(2) | Impoundment Contents | 270.14(b)(5); 264.226(b)(2) | Inspect for sudden drop in level of impoundment contents. |  |  |
| F-2b(4)(a)(3) | Dikes and Containment Devices | 270.14(b)(5); 264.226(b)(3) | Inspect for severe erosion in containment devices. |  |  |
| F-2b(4)(b) | Structural Integrity | 270.14(b)(5); 264.226(c) | Specify procedure for assessing integrity of surface impoundments. |  |  |
| F-2b(4)(c) | Leak Detection System | 270.14(b)(5); 270.17(c); 264.226(d) | Describe how double liner system and leak detection system will be inspected. |  |  |
| F-2b(5)(a) | Incinerator and Associated Equipment | 270.14(b)(5); 264.347(b) | Describe procedures for daily visual inspection of incinerator and associated equipment. |  |  |
| F-2b(5)(b) | Incinerator Waste Feed Cutoff System and Alarms | 270.14(b)(5); 264.347(c) | Describe procedure and frequency of testing emergency waste feed cutoff system. |  |  |
| F-2b(6) | Landfill Inspection | 270.14(b)(5); 264.303(b) | For operating landfill, describe how it will be inspected weekly and after storms. |  |  |
| F-2b(6)(a) | Runon and Runoff Control System | 270.14(b)(5); 264.303(b)(1) | Deterioration, malfunction, or improper operation of runon and runoff control system. |  |  |
| F-2b(6)(b) | Wind Dispersal Control System | 270.14(b)(5); 264.303(b)(2) | Proper functioning of wind dispersal control systems, where present. |  |  |
| F-2b(6)(c) | Leachate Collection and Removal System | 270.14(b)(5); 264.303(b)(3),(c) | In properly functioning double liner system, is there a presence of leachate? Leak detection required under 264.301(c) or 264.301(d) must record amount of leakage from each system weekly. |  |  |
| F-2b(7) | Land Treatment Facility Inspection | 270.14(b)(5); 264.273(g) | Describe how land treatment facility will be inspected weekly and after storms. |  |  |
| F-2b(7)(a) | Runon and Runoff Control System | 270.14(b)(5); 264.273(g)(1); 264a.273(2) |  |  |  |
| F-2b(7)(b) | Wind Dispersal Control System | 270.14(b)(5); 264.273(g)(2) |  |  |  |
| F-2b(8) | Miscellaneous Unit Inspections | 270.14(b)(5); 264.602 | Provide inspection program that ensures compliance with standards in 264.601 and 270.23. |  |  |
| F-2b(9) | Boilers and Industrial Furnaces (BIF) Inspection | 270.14(b)(5); 264.15; 264a.15; 266.102(a)(2)(ii),(e)(8); 266.111(e)(3) | Demonstrate that BIF will be visually inspected daily, automatic waste feed cutoff inspected at least weekly, and direct transfer area at least once an hour when waste is being transferred. |  |  |
| F-2b(10) | Containment Building Inspection | 270.14(b)(5); 264.1101(c)(3), (4) | Demonstrate owner/operator will inspect and document at least weekly, monitoring equipment, leak detection equipment, containment building, and surrounding areas for waste releases. |  |  |
| F-2b(11) | Drip Pad Inspection | 270.14(b)(5); 264.574 | Demonstrate that the drip pad owner/operator will inspect and document at least weekly and after storms, the leak detection and collection equipment, the drip pad surface, and the runon and runoff control systems for evidence of deterioration, malfunction, improper operation, or leakage of hazardous waste. |  |  |
| F-3 | Waiver or Documentation of Preparedness and Prevention Requirements | 270.14(b)(6); 264.32(a) - (d) | Facility must submit justification for any waiver to requirements of this section. |  |  |
| F-3(a) | Equipment Requirements | 270.14(b); 264.32 |  |  |  |
| F-3(a)(1) | Internal Communication | 270.14(b); 264.32(a) | Describe internal communication or alarm system used to provide immediate emergency instruction to personnel. |  |  |
| F-3(a)(2) | External Communication | 270.14(b); 264.32(b) | Describe device for summoning emergency assistance from local police, fire, or state/local emergency response. |  |  |
| F-3(a)(3) | Emergency Equipment | 270.14(b); 264.32(c) | Demonstrate that portable fire extinguishers, fire control equipment, spill control equipment, and decontamination equipment are available. |  |  |
| F-3(a)(4) | Water and Fire Control | 270.14(b); 264.32(d) | Demonstrate facility has adequate fire control systems, water volume and pressure, foaming equipment, automatic sprinklers, etc. |  |  |
| F-3(a)(5) | Testing and Maintenance of Equipment | 270.14(b); 264.33 | Demonstrate communication, alarm, fire control equipment, spill control equipment, and decontamination equipment are tested and maintained. |  |  |
| F-3(a)(6) | Access to Communication or Alarm System | 270.14(b); 264.34 | When waste is being hauled, all personnel must have access to internal alarm or communication device. |  |  |
| F-3(b) | Aisle Space Requirement | 270.14(b); 264.35 | Aisle space is required for unobstructed movement of personnel, fire protection equipment, spill control equipment, and decontamination equipment in case of emergency. |  |  |
| F-3(c) | Documentation of Arrangements with: | 270.14(b); 264.37 | Owner/operator must make arrangements, as appropriate, with type of waste and hazard potential, for the potential need for services. |  |  |
| F-3(c)(1) | Police/Fire Department | 270.14(b); 264.37(a)(1) | Arrange to familiarize local fire department and police with facility. |  |  |
| F-3(c)(2) | Emergency Response Teams | 270.14(b); 264.37(a)(2), (a)(3) |  |  |  |
| F-3(c)(3) | Local Hospitals | 270.14(b); 264.37(a)(4) | Arrange to familiarize local hospital with properties of hazardous waste and possible types of injury or illness to expect. |  |  |
| F-3(c)(4) | Document Agreement Refusal | 270.14(b); 264.37(b) | Document refusal to enter into a coordination agreement. |  |  |
| F-4 | Prevention Procedures, Structures, and Equipment | 270.14 |  |  |  |
| F-4(a) | Unloading Procedures | 270.14(b)(8)(i) | Describe procedure used to prevent hazards in unloading operations. Identify possible loading and unloading hazards, and document steps taken to minimize or eliminate possibility of these hazards. |  |  |
| F-4(b) | Runoff | 270.14(b)(8)(ii) | Describe procedure used to prevent runoff from hazardous waste handling areas. |  |  |
| F-4(c) | Water Supplies | 270.14(b)(8)(iii) | Describe procedure, structures, equipment used to prevent contamination of water supply. |  |  |
| F-4(d) | Equipment and Power Failure | 270.14(b)(8)(iv) | Describe procedure used to mitigate the effects of equipment failure and power outages. |  |  |
| F-4(e) | Personnel Protection Procedures | 270.14(b)(8)(v) | Describe procedure, structures, equipment used to prevent contamination of personnel to hazardous waste. |  |  |
| F-4(f) | Procedures to Minimize Releases to the Atmosphere | 270.14(b)(8)(vi) | Describe procedure, structures, equipment used to prevent hazardous waste releases to the atmosphere. |  |  |
| F-5 | Prevention of Reaction of Ignitable, Reactive, and Incompatible Waste | 270.14(b)(9) |  |  |  |
| F-5a | Precautions to Prevent Ignition or Reaction of Ignitable or Reactive Wastes | 270.14(b)(9); 264.17(a),(b) | Waste must be protected from sources of ignition or reaction. Describe precautions taken by facility to prevent actual ignition, including sources of spontaneous ignition and radiant heat. Owner/operator must designate safe areas for smoking and open flames. Post signs where hazard exists. |  |  |
| F-5b | General Precautions for Handling Ignitable or Reactive Waste and Mixing of Incompatible Waste | 270.14(b)(9); 264.17(a) | Describe precautions taken by facility to prevent reactions that generate heat, produce flammable byproducts, cause risk of fire or explosion, threaten structural integrity, or pose threat to human life or the environment. |  |  |
| F-5b(1) | Documentation of Adequacy of Procedures | 270.14(b); 264.17(c) | Published literature, trial test, waste analyses, or similar processes may be used. |  |  |
| F-5c | Management of Ignitable or Reactive Wastes in Containers | 270.15(c); 264.176 | Demonstrate that ignitable containers are at least 15 meters from facility property line. |  |  |
| F-5d | Management of Incompatible Wastes in Containers | 270.15(d); 264.177 | Describe procedures that ensure incompatible wastes and materials are not placed in same container. |  |  |
| F-5e | Management of Ignitable or Reactive Wastes in Tank Systems | 270.16(j); 264.198 | Describe operation procedures and how facility treats waste so it is no longer ignitable or how facility stores ignitable or reactive waste. |  |  |
| F-5f | Management of Incompatible Wastes in Tank Systems | 270.16(j); 264.199 | Demonstrate that incompatible waste and materials are not stored in same tank. |  |  |
| F-5g | Management of Ignitable or Reactive Wastes Placed in Waste Piles | 270.18(g); 264.256 | If waste is reactive or ignitable, describe how handling process will render waste pile nonreactive and/or nonignitable. |  |  |
| F-5h | Management of Incompatible Wastes Placed in Waste Piles | 270.18(h); 264.257 | Document how hazardous waste piles of incompatible materials are separated to render them nonreactive. |  |  |
| F-5i | Management of Ignitable or Reactive Wastes in Surface Impoundments | 270.17(h); 264.229 | If waste is reactive or ignitable, describe how handling process will render surface impoundments nonreactive and/or nonignitable. |  |  |
| F-5j | Management of Incompatible Wastes in Surface Impoundments | 270.17(h); 264.230 | Document how hazardous surface impoundments of incompatible materials are separated to render them nonreactive. |  |  |
| F-5k | Management of Ignitable or Reactive Wastes Placed in Landfills | 270.21(f); 264.312 | If waste is reactive or ignitable, describe how handling process will prevent reaction or ignition to landfills. |  |  |
| F-5l | Management of Incompatible Wastes Placed in Landfills | 270.21(g); 264.313 | Document how hazardous landfills of incompatible materials are separated to render them nonreactive. |  |  |
| F-5m | Management of Ignitable or Reactive Wastes Placed in Land Treatment Units | 270.20(g); 264.281 | If waste is reactive or ignitable, describe how handling process will render land treatment units nonreactive and/or nonignitable. |  |  |
| F-5n | Management of Incompatible Wastes Placed in Land Treatment Units | 270.20(h); 264.282 | Document how land treatment unit piles of incompatible materials are separated to render them nonreactive. |  |  |
| F-5o | Management of Incompatible Wastes Placed in Containment Buildings | 270.14(a); 264.1101(a)(3) | Subsections include design, primary and secondary containment, barriers to prevent migration, leak detection, and facility logs. |  |  |

Notes:

a	Considerations in addition to the requirements presented in the regulations.

b	For each requirement, this column must indicate one of the following: NA for not applicable, IM for information missing, or the exact location of the information in the application.

c	If application is deficient in an area, prepare a comment describing the deficiency, attach it to the checklist, and reference the comment in this column.

d.	Listed Federal regulatory citations refer to provisions of Title 40 of the Code of Federal Regulations (40CFR) that have been incorporated by reference into Title 25 of the Pennsylvania Code (25 Pa. Code).  Citations containing a small letter “a” refer to provisions of 25 Pa. Code that contain additions to, deletions from, or modifications of the Federal Regulations that have been incorporated.