Table IV-9

Short List of Petroleum Products

| Product Stored | Parameters to be tested in Soil | Analytical Method (reported on a dry weight basis) | Parameters to be tested in Water | ANALYTICAL METHOD |
| --- | --- | --- | --- | --- |
| Leaded Gasoline, | Benzene | EPA Method 5035/8021B or | Benzene | EPA Method |
| Aviation | Toluene | 5035/8260B | Toluene | 5030B/8021B, |
| Gasoline, | Ethyl Benzene |  | Ethyl Benzene | 5030B/8260B or 524.2 |
| and Jet Fuel | Xylenes (total) |  | Xylenes (total) |  |
|  | Cumene |  | Cumene |  |
|  | Naphthalene |  | Naphthalene |  |
|  | Trimethyl benzene, 1,3,4- |  | Trimethyl benzene, 1,3,4- |  |
|  | Trimethyl benzene, 1,3,5- |  | Trimethyl benzene, 1,3,5- |  |
|  | Dichloroethane, 1,2- |  | Dichloroethane, 1,2- |  |
|  | Dibromoethane, 1,2-(Ethylene Dibromide) |  | Dibromoethane, 1,2-(Ethylene Dibromide) | EPA Method 8011 or 504.1 |
|  | Lead (total) | EPA Method 6010B or 7420 | Lead (dissolved) | EPA Method 6020 or 7421 |
| Unleaded | Benzene | EPA Method 5035/8260B | Benzene | EPA Method |
| Gasoline | Toluene |  | Toluene | 5030B/8260B or 524.2 |
|  | Ethyl Benzene |  | Ethyl Benzene |  |
|  | Xylenes (total) |  | Xylenes (total) |  |
|  | Cumene |  | Cumene |  |
|  | Methyl tert-Butyl Ether (MTBE) |  | Methyl tert-Butyl Ether (MTBE) |  |
|  | Trimethyl benzene, 1,3,4- |  | Trimethyl benzene, 1,3,4- |  |
|  | Trimethyl benzene, 1,3,5- |  | Trimethyl benzene, 1,3,5- |  |
|  | Naphthalene |  | Naphthalene |  |
| Kerosene, | Benzene | EPA Method 5035/8021B or | Benzene | EPA Method |
| Fuel Oil No. 1 | Toluene | 5035/8260B | Toluene | 5030B/8021B, |
|  | Ethyl Benzene |  | Ethyl Benzene | 5030B/8260B or 524.2 |
|  | Cumene |  | Cumene |  |
|  | Naphthalene |  | Naphthalene |  |
|  | Fluorene | EPA Method 8270C or 8310 | Fluorene | EPA Method 8270C, |
|  |  |  |  | 8310 or 525.2 |
| Diesel Fuel, | Benzene | EPA Method 5035/8021B or | Benzene | EPA Method |
| Fuel Oil No. 2 | Toluene | 5035/8260B | Toluene | 5030B/8021B, |
|  | Ethyl Benzene |  | Ethyl Benzene | 5030B/8260B or 524.2 |
|  | Cumene |  | Cumene |  |
|  | Methyl tert-Butyl Ether |  | Methyl tert-Butyl Ether |  |
|  | Naphthalene |  | Naphthalene |  |
|  | Fluorene | EPA Method 8270C or 8310 | Fluorene | EPA Method 8270C, |
|  | Phenanthrene |  | Phenanthrene | 8310 or 525.2 |
| Fuel Oil Nos. | Benzene | EPA Method 5035/8021B or | Benzene | EPA Method |
| 4, 5 and 6, and | Naphthalene | 5035/8260B | Naphthalene | 5030B/8021B, |
| Lubricating Oils |  |  |  | 5030B/8260B or 524.2 |
| and Fluids | Fluorene | EPA Method 8270C or 8310 | Phenanthrene | EPA Method 8270C, |
|  | Anthracene |  | Pyrene | 8310 or 525.2 |
|  | Phenanthrene |  | Chrysene |  |
|  | Pyrene |  |  |  |
|  | Benzo(a)anthracene |  |  |  |
|  | Chrysene |  |  |  |
|  | Benzo(b)fluoranthene |  |  |  |
|  | Benzo(a)pyrene |  |  |  |
|  | Benzo(g,h,i)perylene |  |  |  |

Table IV-9

Short List of Petroleum Products

(cont’d)

| Used Motor Oil | Benzene | EPA Method 5035/8021B or | Benzene | EPA Method |
| --- | --- | --- | --- | --- |
|  | Toluene | 5035/8260B | Toluene | 5030B/8021B, |
|  | Ethyl Benzene |  | Ethyl Benzene | 5030B/8260B or 524.2 |
|  | Cumene |  | Cumene |  |
|  | Naphthalene |  | Naphthalene |  |
|  | Pyrene | EPA Method 8270C or 8310 | Pyrene | EPA Method 525.2 |
|  | Benzo(a)anthracene |  | Benzo(a)anthracene |  |
|  | Chrysene |  | Chrysene |  |
|  | Benzo(b)fluoranthene |  | Benzo(b)fluoranthene |  |
|  | Benzo(a)pyrene |  | Benzo(a)pyrene |  |
|  | Indeno(1,2,3-cd)pyrene |  | Indeno(1,2,3-cd)pyrene |  |
|  | Benzo(g,h,i)perylene |  | Benzo(g,h,i)perylene |  |
|  | Lead (total) | EPA Method 6010B or 7420 | Lead (dissolved) | EPA Method 6020 or 7421 |
| Mineral Insulating Oil | PCB-1016 (Aroclor) | EPA Method 8082 | PCB-1016 (Aroclor) | EPA Method 8082 |
|  | PCB-1221 (Aroclor) |  | PCB-1221 (Aroclor) |  |
|  | PCB-1232 (Aroclor) |  | PCB-1232 (Aroclor) |  |
|  | PCB-1242 (Aroclor) |  | PCB-1242 (Aroclor) |  |
|  | PCB-1248 (Aroclor) |  | PCB-1248 (Aroclor) |  |
|  | PCB-1254 (Aroclor) |  | PCB-1254 (Aroclor) |  |
|  | PCB-1260 (Aroclor) |  | PCB-1260 (Aroclor) |  |
|  | Trimethyl benzene, 1,3,4- |  | Trimethyl benzene, 1,3,4- |  |
|  | Trimethyl benzene, 1,3,5- |  | Trimethyl benzene, 1,3,5- |  |

* Other substances were proposed to be added but were not for the following reasons:

- t-butyl alcohol – no MSC could be developed due to lack of toxicity data

- hexane – not added because of low toxicity, common lab contaminant

- cyclohexane – proposed for leaded gasoline, jet fuel, #2 and #6 fuel oil. No SW-846 method available