| REGULATED SUBSTANCE | CASRN | Used Aquifers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Nonuse Aquifers |  |  |  |  |  |  | Soil Buffer Distance (feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TDS ≤ 2500 mg/L |  |  |  |  |  |  |  | TDS > 2500 mg/L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Residential |  |  |  | Nonresidential |  |  |  | Residential |  |  |  |  | Nonresidential |  |  | Residential |  |  |  | Nonresidential |  |  |  |  |
|  |  | 100 X GW MSC | Generic Value |  | 100 X GW MSC |  | Generic Value |  | 100 X GW MSC |  | Generic Value |  |  | 100 X GW MSC |  | Generic Value |  | 100 X GW MSC | Generic Value |  | 100 X GW MSC |  | Generic Value |  |  |
| ACENAPHTHENE | 83-32-9 | 210 | 2,600 | E | 380 |  | 4,700 | E | 380 |  | 4,700 | E | 380 |  |  | 4,700 | E | 380 | 4,700 | E | 380 |  | 4,700 | E | 15 |
| ACENAPHTHYLENE | 208-96-8 | 210 | 2,400 | E | 580 |  | 6,600 | E | 1,600 |  | 18,000 | E | 1,600 |  |  | 18,000 | E | 1,600 | 18,000 | E | 1,600 |  | 18,000 | E | 15 |
| ACEPHATE | 30560-19-1 | 4.2 | 0.5 | E | 12 |  | 1.4 | E | 420 |  | 50 | E | 1,200 |  |  | 140 | E | 4.2 | 0.5 | E | 12 |  | 1.4 | E | NA |
| ACETALDEHYDE | 75-07-0 | 1.9 | 0.23 | E | 7.9 |  | 0.96 | E | 190 |  | 23 | E | 790 |  |  | 96 | E | 1.9 | 0.23 | E | 7.9 |  | 0.96 | E | NA |
| ACETONE | 67-64-1 | 3,100 | 350 | E | 8,800 |  | 980 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 3,500 | E | 10,000 |  | 9,800 | E | NA |
| ACETONITRILE | 75-05-8 | 13 | 1.5 | E | 53 |  | 6 | E | 1,300 |  | 150 | E | 5,300 |  |  | 600 | E | 130 | 15 | E | 530 |  | 60 | E | NA |
| ACETOPHENONE | 98-86-2 | 350 | 190 | E | 970 |  | 520 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 350 | 190 | E | 970 |  | 520 | E | NA |
| ACETYLAMINOFLUORENE, 2- (2AAF) | 53-96-3 | 0.017 | 0.07 | E | 0.072 |  | 0.3 | E | 1.7 |  | 7 | E | 7.2 |  |  | 30 | E | 17 | 70 | E | 72 |  | 300 | E | 20 |
| ACROLEIN | 107-02-8 | 0.0042 | 0.00047 | E | 0.018 |  | 0.002 | E | 0.42 |  | 0.047 | E | 1.8 |  |  | 0.2 | E | 0.042 | 0.0047 | E | 0.18 |  | 0.02 | E | NA |
| ACRYLAMIDE | 79-06-1 | 0.019 | 0.0033 | E | 0.25 |  | 0.043 | E | 1.9 |  | 0.33 | E | 25 |  |  | 4.3 | E | 0.019 | 0.0033 | E | 0.25 |  | 0.043 | E | NA |
| ACRYLIC ACID | 79-10-7 | [0.21] 0.042 | [0.039] 0.0077 | E | [0.88] 0.18 |  | [0.16] 0.033 | E | [21] 4.2 |  | [3.9] 0.77 | E | [88] 18 |  |  | [16] 3.3 | E | [21] 4.2 | [3.9] 0.77 | E | [88] 18 |  | [16] 3.3 | E | NA |
| ACRYLONITRILE | 107-13-1 | 0.072 | 0.01 | E | 0.37 |  | 0.051 | E | 7.2 |  | 1 | E | 37 |  |  | 5.1 | E | 7.2 | 1 | E | 37 |  | 5.1 | E | NA |
| ALACHLOR | 15972-60-8 | 0.2 | 0.077 | E | 0.2 |  | 0.077 | E | 20 |  | 7.7 | E | 20 |  |  | 7.7 | E | 0.2 | 0.077 | E | 0.2 |  | 0.077 | E | NA |
| ALDICARB | 116-06-3 | 0.3 | 0.05 | E | 0.3 |  | 0.05 | E | 30 |  | 5 | E | 30 |  |  | 5 | E | 300 | 50 | E | 300 |  | 50 | E | NA |
| ALDICARB SULFONE | 1646-88-4 | 0.2 | 0.027 | E | 0.2 |  | 0.027 | E | 20 |  | 2.7 | E | 20 |  |  | 2.7 | E | 0.2 | 0.027 | E | 0.2 |  | 0.027 | E | NA |
| ALDICARB SULFOXIDE | 1646-87-3 | 0.4 | 0.045 | E | 0.4 |  | 0.045 | E | 40 |  | 4.5 | E | 40 |  |  | 4.5 | E | 0.4 | 0.045 | E | 0.4 |  | 0.045 | E | NA |
| ALDRIN | 309-00-2 | 0.0038 | 0.46 | E | 0.016 |  | 1.9 | E | 0.38 |  | 46 | E | 1.6 |  |  | 190 | E | 2 | 240 | E | 2 |  | 240 | E | 10 |
| ALLYL ALCOHOL | 107-18-6 | 0.021 | 0.0025 | E | 0.088 |  | 0.01 | E | 2.1 |  | 0.25 | E | 8.8 |  |  | 1 | E | 2.1 | 0.25 | E | 8.8 |  | 1 | E | NA |
| AMETRYN | 834-12-8 | 6 | 6.5 | E | 6 |  | 6.5 | E | 600 |  | 650 | E | 600 |  |  | 650 | E | 6 | 6.5 | E | 6 |  | 6.5 | E | NA |
| AMINOBIPHENYL, 4- | 92-67-1 | 0.0031 | 0.0012 | E | 0.013 |  | 0.005 | E | 0.31 |  | 0.12 | E | 1.3 |  |  | 0.5 | E | 3.1 | 1.2 | E | 13 |  | 5 | E | NA |
| AMITROLE | 61-82-5 | 0.069 | 0.028 | E | 0.29 |  | 0.12 | E | 6.9 |  | 2.8 | E | 29 |  |  | 12 | E | 69 | 28 | E | 290 |  | 120 | E | NA |
| AMMONIA | 7664-41-7 | 3,000 | 360 | E | 3,000 |  | 360 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 3,000 | 360 | E | 3,000 |  | 360 | E | NA |
| AMMONIUM SULFAMATE | 7773-06-0 | 200 | 24 | E | 200 |  | 24 | E | 20,000 |  | 2,400 | E | 20,000 |  |  | 2,400 | E | 200 | 24 | E | 200 |  | 24 | E | NA |
| ANILINE | 62-53-3 | 0.21 | 0.12 | E | 0.88 |  | 0.52 | E | 21 |  | 12 | E | 88 |  |  | 52 | E | 0.21 | 0.12 | E | 0.88 |  | 0.52 | E | NA |
| ANTHRACENE | 120-12-7 | 6.6 | 350 | E | 6.6 |  | 350 | E | 6.6 |  | 350 | E | 6.6 |  |  | 350 | E | 6.6 | 350 | E | 6.6 |  | 350 | E | 10 |
| ATRAZINE | 1912-24-9 | 0.3 | 0.13 | E | 0.3 |  | 0.13 | E | 30 |  | 13 | E | 30 |  |  | 13 | E | 0.3 | 0.13 | E | 0.3 |  | 0.13 | E | NA |
| AZINPHOS-METHYL (GUTHION) | 86-50-0 | 5.2 | 5.9 | E | 15 |  | 17 | E | 520 |  | 590 | E | 1,500 |  |  | 1,700 | E | 5.2 | 5.9 | E | 15 |  | 17 | E | NA |
| BAYGON (PROPOXUR) | 114-26-1 | 0.3 | 0.057 | E | 0.3 |  | 0.057 | E | 30 |  | 5.7 | E | 30 |  |  | 5.7 | E | 300 | 57 | E | 300 |  | 57 | E | NA |
| BENOMYL | 17804-35-2 | 27 | 130 | E | 110 |  | 530 | E | 200 |  | 970 | E | 200 |  |  | 970 | E | 27 | 130 | E | 110 |  | 530 | E | 20 |
| BENTAZON | 25057-89-0 | 20 | 2.9 | E | 20 |  | 2.9 | E | 2,000 |  | 290 | E | 2,000 |  |  | 290 | E | 20 | 2.9 | E | 20 |  | 2.9 | E | NA |
| BENZENE | 71-43-2 | 0.5 | 0.13 | E | 0.5 |  | 0.13 | E | 50 |  | 13 | E | 50 |  |  | 13 | E | 50 | 13 | E | 50 |  | 13 | E | NA |
| BENZIDINE | 92-87-5 | 0.000092 | 0.12 | E | 0.0012 |  | 1.6 | E | 0.0092 |  | 12 | E | 0.12 |  |  | 160 | E | 0.092 | 120 | E | 1.2 |  | 1,600 | E | 5 |
| BENZO[A]ANTHRACENE | 56-55-3 | [0.03] 0.21 | [26] 180 | E | [0.39] 1.1 |  | [340] 960 | E | 1.1 |  | 960 | E | 1.1 |  |  | 960 | E | 1.1 | 960 | E | 1.1 |  | 960 | E | 5 |
| BENZO[A]PYRENE | 50-32-8 | 0.02 | 46 | E | 0.02 |  | 46 | E | 0.38 |  | 860 | E | 0.38 |  |  | 860 | E | 0.38 | 860 | E | 0.38 |  | 860 | E | 5 |
| BENZO[B]FLUORANTHENE | 205-99-2 | [0.018] 0.12 | [25] 170 | E | 0.12 |  | 170 | E | 0.12 |  | 170 | E | 0.12 |  |  | 170 | E | 0.12 | 170 | E | 0.12 |  | 170 | E | 5 |
| BENZO[GHI]PERYLENE | 191-24-2 | 0.026 | 180 | E | 0.026 |  | 180 | E | 0.026 |  | 180 | E | 0.026 |  |  | 180 | E | 0.026 | 180 | E | 0.026 |  | 180 | E | 5 |
| BENZO[K]FLUORANTHENE | 207-08-9 | [0.018] 0.055 | [200] 610 | E | 0.055 |  | 610 | E | 0.055 |  | 610 | E | 0.055 |  |  | 610 | E | 0.055 | 610 | E | 0.055 |  | 610 | E | 5 |
| BENZOIC ACID | 65-85-0 | 14,000 | 2,700 | E | 39,000 |  | 7,500 | E | 190,000 |  | 52,000 | E | 190,000 |  |  | 52,000 | E | 14,000 | 2,700 | E | 39,000 |  | 7,500 | E | NA |
| BENZOTRICHLORIDE | 98-07-7 | 0.005 | 0.012 | E | 0.021 |  | 0.051 | E | 0.5 |  | 1.2 | E | 2.1 |  |  | 5.1 | E | 0.5 | 1.2 | E | 2.1 |  | 5.1 | E | 30 |
| BENZYL ALCOHOL | 100-51-6 | 350 | 130 | E | 970 |  | 350 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 350 | 130 | E | 970 |  | 350 | E | NA |
| BENZYL CHLORIDE | 100-44-7 | 0.1 | 0.059 | E | 0.51 |  | 0.3 | E | 10 |  | 5.9 | E | 51 |  |  | 30 | E | 10 | 5.9 | E | 51 |  | 30 | E | NA |
| BETA PROPIOLACTONE | 57-57-8 | 0.0012 | 0.00015 | E | 0.0063 |  | 0.00076 | E | 0.12 |  | 0.015 | E | 0.63 |  |  | 0.076 | E | 0.012 | 0.0015 | E | 0.063 |  | 0.0076 | E | NA |
| BHC, ALPHA | 319-84-6 | 0.01 | 0.046 | E | 0.043 |  | 0.2 | E | 1 |  | 4.6 | E | 4.3 |  |  | 20 | E | 10 | 46 | E | 43 |  | 200 | E | 20 |
| BHC, BETA- | 319-85-7 | 0.036 | 0.21 | E | 0.15 |  | 0.88 | E | 3.6 |  | 21 | E | 10 |  |  | 59 | E | 10 | 59 | E | 10 |  | 59 | E | 15 |
| BHC, GAMMA (LINDANE) | 58-89-9 | 0.02 | 0.072 | E | 0.02 |  | 0.072 | E | 2 |  | 7.2 | E | 2 |  |  | 7.2 | E | 20 | 72 | E | 20 |  | 72 | E | 20 |
| BIPHENYL, 1,1- | 92-52-4 | 0.084 | 0.37 | E | 0.35 |  | 1.5 | E | 8.4 |  | 37 | E | 35 |  |  | 150 | E | 8.4 | 37 | E | 35 |  | 150 | E | 20 |
| BIS(2-CHLOROETHOXY) METHANE | 111-91-1 | 10 | 2.6 | E | 29 |  | 7.6 | E | 1,000 |  | 260 | E | 2,900 |  |  | 760 | E | 10 | 2.6 | E | 29 |  | 7.6 | E | NA |
| BIS(2-CHLOROETHYL)ETHER | 111-44-4 | 0.015 | 0.0045 | E | 0.076 |  | 0.023 | E | 1.5 |  | 0.45 | E | 7.6 |  |  | 2.3 | E | 1.5 | 0.45 | E | 7.6 |  | 2.3 | E | NA |
| BIS(2-CHLORO-ISOPROPYL)ETHER | 108-60-1 | 30 | 8 | E | 30 |  | 8 | E | 3,000 |  | 800 | E | 3,000 |  |  | 800 | E | 3,000 | 800 | E | 3,000 |  | 800 | E | NA |
| BIS(CHLOROMETHYL)ETHER | 542-88-1 | 0.000079 | 0.000012 | E | 0.0004 |  | 0.00006 | E | 0.0079 |  | 0.0012 | E | 0.04 |  |  | 0.006 | E | 0.0079 | 0.0012 | E | 0.04 |  | 0.006 | E | NA |
| BIS[2-ETHYLHEXYL] PHTHALATE | 117-81-7 | 0.6 | 130 | E | 0.6 |  | 130 | E | 29 |  | 6,300 | E | 29 |  |  | 6,300 | E | 29 | 6,300 | E | 29 |  | 6,300 | E | 10 |
| BISPHENOL A | 80-05-7 | 170 | 660 | E | 490 |  | 1,900 | E | 12,000 |  | 46,000 | E | 12,000 |  |  | 46,000 | E | 12,000 | 46,000 | E | 12,000 |  | 46,000 | E | 20 |
| BROMACIL | 314-40-9 | 7 | 1.8 | E | 7 |  | 1.8 | E | 700 |  | 180 | E | 700 |  |  | 180 | E | 7 | 1.8 | E | 7 |  | 1.8 | E | NA |
| BROMOBENZENE | 108-86-1 | [0.006] 6 | [0.0047] 4.7 | E | [0.006] 6 |  | [0.0047] 4.7 | E | [0.6] 600 |  | [0.47] 470 | E | [0.6] 600 |  |  | [0.47] 470 | E | [0.006] 6 | [0.0047] 4.7 | E | [0.006] 6 |  | [0.0047] 4.7 | E | NA |
| BROMOCHLOROMETHANE | 74-97-5 | 9 | 1.6 | E | 9 |  | 1.6 | E | 900 |  | 160 | E | 900 |  |  | 160 | E | 9 | 1.6 | E | 9 |  | 1.6 | E | NA |
| BROMODICHLORO METHANE (THM) | 75-27-4 | 8 | 2.7 | E | 8 |  | 2.7 | E | 800 |  | 270 | E | 800 |  |  | 270 | E | 8 | 2.7 | E | 8 |  | 2.7 | E | NA |
| BROMOMETHANE | 74-83-9 | 1 | 0.54 | E | 1 |  | 0.54 | E | 100 |  | 54 | E | 100 |  |  | 54 | E | 100 | 54 | E | 100 |  | 54 | E | NA |
| BROMOXYNIL | 1689-84-5 | 0.63 | 0.54 | E | 2.6 |  | 2.2 | E | 63 |  | 54 | E | 260 |  |  | 220 | E | 0.63 | 0.54 | E | 2.6 |  | 2.2 | E | NA |
| BROMOXYNIL OCTANOATE | 1689-99-2 | 0.63 | 28 | E | 2.6 |  | 120 | E | 8 |  | 360 | E | 8 |  |  | 360 | E | 8 | 360 | E | 8 |  | 360 | E | 15 |
| BUTADIENE, 1,3- | 106-99-0 | 0.11 | 0.045 | E | 0.45 |  | 0.19 | E | 11 |  | 4.5 | E | 45 |  |  | 19 | E | 11 | 4.5 | E | 45 |  | 19 | E | NA |
| BUTYL ALCOHOL, N- | 71-36-3 | 350 | 42 | E | 970 |  | 120 | E | 10,000 |  | 4,200 | E | 10,000 |  |  | 10,000 | C | 3,500 | 420 | E | 9,700 |  | 1,200 | E | NA |
| BUTYLATE | 2008-41-5 | 40 | 58 | E | 40 |  | 58 | E | 4,000 |  | 5,800 | E | 4,000 |  |  | 5,800 | E | 40 | 58 | E | 40 |  | 58 | E | 30 |
| BUTYLBENZENE, N- | 104-51-8 | 170 | 1,100 | E | 490 |  | 3,100 | E | 1,500 |  | 9,500 | E | 1,500 |  |  | 9,500 | E | 170 | 1,100 | E | 490 |  | 3,100 | E | 15 |
| BUTYLBENZENE, SEC- | 135-98-8 | 350 | 820 | E | 970 |  | 2,300 | E | 1,700 |  | 4,000 | E | 1,700 |  |  | 4,000 | E | 350 | 820 | E | 970 |  | 2,300 | E | 30 |
| BUTYLBENZENE, TERT- | 98-06-6 | 350 | 630 | E | 970 |  | 1,800 | E | 3,000 |  | 5,400 | E | 3,000 |  |  | 5,400 | E | 350 | 630 | E | 970 |  | 1,800 | E | 30 |
| BUTYLBENZYL PHTHALATE | 85-68-7 | 34 | 2,900 | E | 140 |  | 10,000 | C | 270 |  | 10,000 | C | 270 |  |  | 10,000 | C | 270 | 10,000 | C | 270 |  | 10,000 | C | 10 |
| CAPTAN | 133-06-2 | 28 | 17 | E | 50 |  | 31 | E | 50 |  | 31 | E | 50 |  |  | 31 | E | 50 | 31 | E | 50 |  | 31 | E | NA |
| CARBARYL | 63-25-2 | 350 | 210 | E | 970 |  | 570 | E | 12,000 |  | 7,000 | E | 12,000 |  |  | 7,000 | E | 12,000 | 7,000 | E | 12,000 |  | 7,000 | E | NA |
| [CARBAZOLE] | [86-74-8] | [3.3] | [21] | [E] | [14] |  | [89] | [E] | [120] |  | [760] | [E] | [120] |  |  | [760] | [E] | [3.3] | [21] | [E] | [14] |  | [89] | [E] | [15] |
| CARBOFURAN | 1563-66-2 | 4 | 0.87 | E | 4 |  | 0.87 | E | 400 |  | 87 | E | 400 |  |  | 87 | E | 4 | 0.87 | E | 4 |  | 0.87 | E | NA |
| CARBON DISULFIDE | 75-15-0 | 150 | 130 | E | 620 |  | 530 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 150 | 130 | E | 620 |  | 530 | E | NA |
| CARBON TETRACHLORIDE | 56-23-5 | 0.5 | 0.26 | E | 0.5 |  | 0.26 | E | 50 |  | 26 | E | 50 |  |  | 26 | E | 5 | 2.6 | E | 5 |  | 2.6 | E | NA |
| CARBOXIN | 5234-68-4 | 70 | 53 | E | 70 |  | 53 | E | 7,000 |  | 5,300 | E | 7,000 |  |  | 5,300 | E | 70 | 53 | E | 70 |  | 53 | E | NA |
| CHLORAMBEN | 133-90-4 | 10 | 1.6 | E | 10 |  | 1.6 | E | 1,000 |  | 160 | E | 1,000 |  |  | 160 | E | 10 | 1.6 | E | 10 |  | 1.6 | E | NA |
| CHLORDANE | 57-74-9 | 0.2 | 49 | E | 0.2 |  | 49 | E | 5.6 |  | 1,400 | E | 5.6 |  |  | 1,400 | E | 5.6 | 1,400 | E | 5.6 |  | 1,400 | E | 10 |
| CHLORO-1,1-DIFLUOROETHANE, 1- | 75-68-3 | 10,000 | 1,800 | E | 10,000 |  | 7,300 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 1,800 | E | 10,000 |  | 7,300 | E | NA |
| CHLORO-1-PROPENE, 3- (ALLYL CHLORIDE) | 107-05-1 | 0.21 | 0.049 | E | 0.88 |  | 0.2 | E | 21 |  | 4.9 | E | 88 |  |  | 20 | E | 21 | 4.9 | E | 88 |  | 20 | E | NA |
| CHLOROACETALDEHYDE | 107-20-0 | 0.24 | 0.029 | E | 1 |  | 0.12 | E | 24 |  | 2.9 | E | 100 |  |  | 12 | E | 0.24 | 0.029 | E | 1 |  | 0.12 | E | NA |
| CHLOROANILINE, P- | 106-47-8 | 0.33 | 0.42 | E | 1.4 |  | 1.8 | E | 33 |  | 42 | E | 140 |  |  | 180 | E | 0.33 | 0.42 | E | 1.4 |  | 1.8 | E | NA |
| CHLOROBENZENE | 108-90-7 | 10 | 6.1 | E | 10 |  | 6.1 | E | 1,000 |  | 610 | E | 1,000 |  |  | 610 | E | 1,000 | 610 | E | 1,000 |  | 610 | E | NA |
| CHLOROBENZILATE | 510-15-6 | 0.59 | 3.9 | E | 2.5 |  | 17 | E | 59 |  | 390 | E | 250 |  |  | 1,700 | E | 590 | 3,900 | E | 1,300 |  | 8,600 | E | 15 |
| CHLOROBUTANE, 1- | 109-69-3 | 140 | 220 | E | 390 |  | 610 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 140 | 220 | E | 390 |  | 610 | E | 30 |
| CHLORODIBROMO METHANE (THM) | 124-48-1 | 8 | 2.5 | E | 8 |  | 2.5 | E | 800 |  | 250 | E | 800 |  |  | 250 | E | 800 | 250 | E | 800 |  | 250 | E | NA |
| CHLORODIFLUORO METHANE (THM) | 75-45-6 | 10,000 | 2,800 | E | 10,000 |  | 10,000 | C | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 2,800 | E | 10,000 |  | 10,000 | C | NA |
| CHLOROETHANE | 75-00-3 | [2,100] 840 | [450] 180 | E | [8,800] 3,500 |  | [1,900] 760 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| CHLOROFORM (THM) | 67-66-3 | 8 | 2 | E | 8 |  | 2 | E | 800 |  | 200 | E | 800 |  |  | 200 | E | 80 | 20 | E | 80 |  | 20 | E | NA |
| CHLORONAPHTHALENE, 2- | 91-58-7 | 280 | 6,000 | E | 780 |  | 17,000 | E | 1,200 |  | 26,000 | E | 1,200 |  |  | 26,000 | E | 280 | 6,000 | E | 780 |  | 17,000 | E | 15 |
| CHLORONITROBENZENE, P- | 100-00-5 | 0.42 | 0.55 | E | 1.8 |  | 2.4 | E | 42 |  | 55 | E | 180 |  |  | 240 | E | 0.42 | 0.55 | E | 1.8 |  | 2.4 | E | NA |
| CHLOROPHENOL, 2- | 95-57-8 | 4 | 4.4 | E | 4 |  | 4.4 | E | 400 |  | 440 | E | 400 |  |  | 440 | E | 4 | 4.4 | E | 4 |  | 4.4 | E | NA |
| CHLOROPRENE | 126-99-8 | 0.016 | 0.0038 | E | 0.083 |  | 0.02 | E | 1.6 |  | 0.38 | E | 8.3 |  |  | 2 | E | 1.6 | 0.38 | E | 8.3 |  | 2 | E | NA |
| [CHLOROPROPANE, 2-] | [75-29-6] | [21] | [16] | [E] | [88] |  | [67] | [E] | [2,100] |  | [1,600] | [E] | [8,800] |  |  | [6,700] | [E] | [21] | [16] | [E] | [88] |  | [67] | [E] | [NA] |
| CHLOROTHALONIL | 1897-45-6 | 3.8 | 9.7 | E | 16 |  | 41 | E | 60 |  | 150 | E | 60 |  |  | 150 | E | 3.8 | 9.7 | E | 16 |  | 41 | E | 30 |
| CHLOROTOLUENE, O- | 95-49-8 | 10 | 20 | E | 10 |  | 20 | E | 1,000 |  | 2,000 | E | 1,000 |  |  | 2,000 | E | 10 | 20 | E | 10 |  | 20 | E | 30 |
| CHLOROTOLUENE, P- | 106-43-4 | 10 | 10 | E | 10 |  | 10 | E | 1,000 |  | 1,000 | E | 1,000 |  |  | 1,000 | E | 10 | 10 | E | 10 |  | 10 | E | NA |
| CHLORPYRIFOS | 2921-88-2 | 0.2 | 2.3 | E | 0.2 |  | 2.3 | E | 20 |  | 230 | E | 20 |  |  | 230 | E | 0.2 | 2.3 | E | 0.2 |  | 2.3 | E | 15 |
| CHLORSULFURON | 64902-72-3 | [69] 170 | [9.6] 24 | E | [190] 490 |  | [26] 68 | E | [6,900] 17,000 |  | [960] 2,400 | E | 19,000 |  |  | 2,600 | E | [69] 170 | [9.6] 24 | E | [190] 490 |  | [26] 68 | E | NA |
| CHLORTHAL-DIMETHYL (DACTHAL) (DCPA) | 1861-32-1 | 7 | 110 | E | 7 |  | 110 | E | 50 |  | 820 | E | 50 |  |  | 820 | E | 50 | 820 | E | 50 |  | 820 | E | 15 |
| CHRYSENE | 218-01-9 | [0.18] 0.19 | [220] 230 | E | 0.19 |  | 230 | E | 0.19 |  | 230 | E | 0.19 |  |  | 230 | E | 0.19 | 230 | E | 0.19 |  | 230 | E | 5 |
| CRESOL(S) | 1319-77-3 | 130 | 23 | E | 530 |  | 92 | E | 10,000 |  | 2,300 | E | 10,000 |  |  | 9,200 | E | 10,000 | 2,300 | E | 10,000 |  | 9,200 | E | NA |
| CRESOL, 4,6-DINITRO-O- | 534-52-1 | 0.28 | 0.21 | E | 0.78 |  | 0.59 | E | 28 |  | 21 | E | 78 |  |  | 59 | E | 28 | 21 | E | 78 |  | 59 | E | NA |
| CRESOL, O- (2-METHYLPHENOL) | 95-48-7 | 170 | 28 | E | 490 |  | 81 | E | 17,000 |  | 2,800 | E | 49,000 |  |  | 8,100 | E | 17,000 | 2,800 | E | 49,000 |  | 8,100 | E | NA |
| CRESOL, M- (3-METHYLPHENOL) | 108-39-4 | 170 | 34 | E | 490 |  | 97 | E | 10,000 |  | 3,400 | E | 10,000 |  |  | 9,700 | E | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| CRESOL, P- (4-METHYLPHENOL) | 106-44-5 | [17] 69 | [4] 16 | E | [49] 190 |  | [11] 44 | E | [1,700] 6,900 |  | [400] 1,600 | E | [4,900] 19,000 |  |  | [1,100] 4,400 | E | [17,000] 69,000 | [4,000] 16,000 | E | [49,000] 190,000 |  | [11,000] 44,000 | E | NA |
| CRESOL, P-CHLORO-M- | 59-50-7 | 350 | 720 | E | 970 |  | 2,000 | E | 35,000 |  | 72,000 | E | 97,000 |  |  | 190,000 | C | 350 | 720 | E | 970 |  | 2,000 | E | 30 |
| CROTONALDEHYDE | 4170-30-3 | [0.034] 3.5 | [0.0043] 0.44 | E | [0.14] 9.7 |  | [0.018] 1.2 | E | [3.4] 350 |  | [0.43] 44 | E | [14] 970 |  |  | [1.8] 120 | E | [3.4] 350 | [0.43] 44 | E | [14] 970 |  | [1.8] 120 | E | NA |
| CROTONALDEHYDE, TRANS- | 123-73-9 | [0.034] 3.5 | [0.0043] 0.44 | E | [0.14] 9.7 |  | [0.018] 1.2 | E | [3.4] 350 |  | [0.43] 44 | E | [14] 970 |  |  | [1.8] 120 | E | [3.4] 350 | [0.43] 44 | E | [14] 970 |  | [1.8] 120 | E | NA |
| CUMENE (ISOPROPYL BENZENE) | 98-82-8 | 84 | 600 | E | 350 |  | 2,500 | E | 5,000 |  | 10,000 | C | 5,000 |  |  | 10,000 | C | 5,000 | 10,000 | C | 5,000 |  | 10,000 | C | 15 |
| CYANAZINE | 21725-46-2 | 0.1 | 0.061 | E | 0.1 |  | 0.061 | E | 10 |  | 6.1 | E | 10 |  |  | 6.1 | E | 0.1 | 0.061 | E | 0.1 |  | 0.061 | E | NA |
| CYCLOHEXANE | 110-82-7 | 1,300 | 1,700 | E | 5,300 |  | 6,900 | E | 5,500 |  | 7,200 | E | 5,500 |  |  | 7,200 | E | 1,300 | 1,700 | E | 5,300 |  | 6,900 | E | NA |
| CYCLOHEXANONE | 108-94-1 | 150 | 41 | E | 620 |  | 170 | E | 10,000 |  | 4,100 | E | 10,000 |  |  | 10,000 | C | 150 | 41 | E | 620 |  | 170 | E | NA |
| CYFLUTHRIN | 68359-37-5 | 0.1 | 33 | E | 0.1 |  | 33 | E | 0.1 |  | 33 | E | 0.1 |  |  | 33 | E | 0.1 | 33 | E | 0.1 |  | 33 | E | 10 |
| CYROMAZINE | 66215-27-8 | 1,700 | 5,300 | E | 4,900 |  | 15,000 | E | 170,000 |  | 190,000 | C | 190,000 |  |  | 190,000 | C | 1,700 | 5,300 | E | 4,900 |  | 15,000 | E | 20 |
| DDD, 4,4'- | 72-54-8 | 0.27 | 30 | E | 1.1 |  | 120 | E | 16 |  | 1,800 | E | 16 |  |  | 1,800 | E | 16 | 1,800 | E | 16 |  | 1,800 | E | 10 |
| DDE, 4,4'- | 72-55-9 | 0.19 | 41 | E | 0.8 |  | 170 | E | 4 |  | 870 | E | 4 |  |  | 870 | E | 4 | 870 | E | 4 |  | 870 | E | 10 |
| DDT, 4,4'- | 50-29-3 | 0.19 | 110 | E | 0.55 |  | 330 | E | 0.55 |  | 330 | E | 0.55 |  |  | 330 | E | 0.55 | 330 | E | 0.55 |  | 330 | E | 5 |
| DI(2-ETHYLHEXYL)ADIPATE | 103-23-1 | 40 | 10,000 | C | 40 |  | 10,000 | C | 4,000 |  | 10,000 | C | 4,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | 5 |
| DIALLATE | 2303-16-4 | 1.1 | 0.64 | E | 4.5 |  | 2.6 | E | 110 |  | 64 | E | 450 |  |  | 260 | E | 1,100 | 640 | E | 4,000 |  | 2,300 | E | NA |
| DIAMINOTOLUENE, 2,4- | 95-80-7 | 0.016 | 0.0032 | E | 0.068 |  | 0.014 | E | 1.6 |  | 0.32 | E | 6.8 |  |  | 1.4 | E | 16 | 3.2 | E | 68 |  | 14 | E | NA |
| DIAZINON | 333-41-5 | 0.1 | 0.14 | E | 0.1 |  | 0.14 | E | 10 |  | 14 | E | 10 |  |  | 14 | E | 0.1 | 0.14 | E | 0.1 |  | 0.14 | E | 30 |
| DIBENZO[A,H] ANTHRACENE | 53-70-3 | [0.0052] 0.021 | [23] 95 | E | 0.06 |  | 270 | E | 0.06 |  | 270 | E | 0.06 |  |  | 270 | E | 0.06 | 270 | E | 0.06 |  | 270 | E | 5 |
| DIBENZOFURAN | 132-64-9 | 3.5 | 90 | E | 9.7 |  | 250 | E | 350 |  | 9,000 | E | 450 |  |  | 12,000 | E | 350 | 9,000 | E | 450 |  | 12,000 | E | 15 |
| DIBROMO-3-CHLOROPROPANE, 1,2- | 96-12-8 | 0.02 | 0.0092 | E | 0.02 |  | 0.0092 | E | 2 |  | 0.92 | E | 2 |  |  | 0.92 | E | 2 | 0.92 | E | 2 |  | 0.92 | E | NA |
| DIBROMOBENZENE, 1,4- | 106-37-6 | 35 | 140 | E | 97 |  | 400 | E | 2,000 |  | 8,200 | E | 2,000 |  |  | 8,200 | E | 35 | 140 | E | 97 |  | 400 | E | 20 |
| DIBROMOETHANE, 1,2- (ETHYLENE DIBROMIDE) | 106-93-4 | 0.005 | 0.0012 | E | 0.005 |  | 0.0012 | E | 0.5 |  | 0.12 | E | 0.5 |  |  | 0.12 | E | 0.5 | 0.12 | E | 0.5 |  | 0.12 | E | NA |
| DIBROMOMETHANE | 74-95-3 | 0.84 | 0.32 | E | 3.5 |  | 1.4 | E | 84 |  | 32 | E | 350 |  |  | 140 | E | 84 | 32 | E | 350 |  | 140 | E | NA |
| DIBUTYL PHTHALATE, N- | 84-74-2 | 350 | 1,400 | E | 970 |  | 4,000 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | 20 |
| DICAMBA | 1918-00-9 | 400 | 45 | E | 400 |  | 45 | E | 40,000 |  | 4,500 | E | 40,000 |  |  | 4,500 | E | 400 | 45 | E | 400 |  | 45 | E | NA |
| DICHLOROACETIC ACID (HAA) | 76-43-6 | 6 | 0.79 | E | 6 |  | 0.79 | E | 600 |  | 79 | E | 600 |  |  | 79 | E | 6 | 0.79 | E | 6 |  | 0.79 | E | NA |
| DICHLORO-2-BUTENE, 1,4- | 764-41-0 | 0.0012 | 0.00067 | E | 0.006 |  | 0.0034 | E | 0.12 |  | 0.067 | E | 0.6 |  |  | 0.34 | E | 0.0012 | 0.00067 | E | 0.006 |  | 0.0034 | E | NA |
| DICHLORO-2-BUTENE, TRANS-1,4- | 110-57-6 | 0.0012 | 0.00078 | E | 0.006 |  | 0.0039 | E | 0.12 |  | 0.078 | E | 0.6 |  |  | 0.39 | E | 0.0012 | 0.00078 | E | 0.006 |  | 0.0039 | E | NA |
| DICHLOROBENZENE, 1,2- | 95-50-1 | 60 | 59 | E | 60 |  | 59 | E | 6,000 |  | 5,900 | E | 6,000 |  |  | 5,900 | E | 6,000 | 5,900 | E | 6,000 |  | 5,900 | E | NA |
| DICHLOROBENZENE, 1,3- | 541-73-1 | 60 | 61 | E | 60 |  | 61 | E | 6,000 |  | 6,100 | E | 6,000 |  |  | 6,100 | E | 6,000 | 6,100 | E | 6,000 |  | 6,100 | E | NA |
| DICHLOROBENZENE, P- | 106-46-7 | 7.5 | 10 | E | 7.5 |  | 10 | E | 750 |  | 1,000 | E | 750 |  |  | 1,000 | E | 750 | 1,000 | E | 750 |  | 1,000 | E | 30 |
| DICHLOROBENZIDINE, 3,3'- | 91-94-1 | 0.14 | 7.7 | E | 0.6 |  | 33 | E | 14 |  | 770 | E | 60 |  |  | 3,300 | E | 140 | 7,700 | E | 310 |  | 17,000 | E | 10 |
| DICHLORODIFLUORO-METHANE (FREON 12) | 75-71-8 | 100 | 100 | E | 100 |  | 100 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| DICHLOROETHANE, 1,1- | 75-34-3 | 3.1 | 0.75 | E | 16 |  | 3.9 | E | 310 |  | 75 | E | 1,600 |  |  | 390 | E | 31 | 7.5 | E | 160 |  | 39 | E | NA |
| DICHLOROETHANE, 1,2- | 107-06-2 | 0.5 | 0.1 | E | 0.5 |  | 0.1 | E | 50 |  | 10 | E | 50 |  |  | 10 | E | 5 | 1 | E | 5 |  | 1 | E | NA |
| DICHLOROETHYLENE, 1,1- | 75-35-4 | 0.7 | 0.19 | E | 0.7 |  | 0.19 | E | 70 |  | 19 | E | 70 |  |  | 19 | E | 7 | 1.9 | E | 7 |  | 1.9 | E | NA |
| DICHLOROETHYLENE, CIS-1,2- | 156-59-2 | 7 | 1.6 | E | 7 |  | 1.6 | E | 700 |  | 160 | E | 700 |  |  | 160 | E | 70 | 16 | E | 70 |  | 16 | E | NA |
| DICHLOROETHYLENE, TRANS-1,2- | 156-60-5 | 10 | 2.3 | E | 10 |  | 2.3 | E | 1,000 |  | 230 | E | 1,000 |  |  | 230 | E | 100 | 23 | E | 100 |  | 23 | E | NA |
| DICHLOROMETHANE (METHYLENE CHLORIDE) | 75-09-2 | 0.5 | 0.076 | E | 0.5 |  | 0.076 | E | 50 |  | 7.6 | E | 50 |  |  | 7.6 | E | 50 | 7.6 | E | 50 |  | 7.6 | E | NA |
| DICHLOROPHENOL, 2,4- | 120-83-2 | 2 | 1 | E | 2 |  | 1 | E | 200 |  | 100 | E | 200 |  |  | 100 | E | 2,000 | 1,000 | E | 2,000 |  | 1,000 | E | NA |
| DICHLOROPHENOXY ACETIC ACID, 2,4- (2,4-D) | 94-75-7 | 7 | 1.8 | E | 7 |  | 1.8 | E | 700 |  | 180 | E | 700 |  |  | 180 | E | 7,000 | 1,800 | E | 7,000 |  | 1,800 | E | NA |
| DICHLOROPROPANE, 1,2- | 78-87-5 | 0.5 | 0.11 | E | 0.5 |  | 0.11 | E | 50 |  | 11 | E | 50 |  |  | 11 | E | 5 | 1.1 | E | 5 |  | 1.1 | E | NA |
| DICHLOROPROPENE, 1,3- | 542-75-6 | 0.65 | 0.12 | E |  | 2.7 | 0.48 | E | 65 |  | 12 | E | 270 |  |  | 48 | E | 65 | 12 | E | 270 |  | 48 | E | NA |
| DICHLOROPROPIONIC ACID, 2,2- (DALAPON) | 75-99-0 | 20 | 5.3 | E | 20 |  | 5.3 | E | 2,000 |  | 530 | E | 2,000 |  |  | 530 | E | 2,000 | 530 | E | 2,000 |  | 530 | E | NA |
| DICHLORVOS | 62-73-7 | 0.22 | 0.052 | E | 0.94 |  | 0.22 | E | 22 |  | 5.2 | E | 94 |  |  | 22 | E | 0.22 | 0.052 | E | 0.94 |  | 0.22 | E | NA |
| DICYCLOPENTADIENE | 77-73-6 | 0.063 | 0.13 | E | 0.26 |  | 0.56 | E | 6.3 |  | 13 | E | 26 |  |  | 56 | E | 0.063 | 0.13 | E | 0.26 |  | 0.56 | E | 30 |
| DIELDRIN | 60-57-1 | 0.0041 | 0.11 | E | 0.017 |  | 0.47 | E | 0.41 |  | 11 | E | 1.7 |  |  | 47 | E | 4.1 | 110 | E | 17 |  | 470 | E | 15 |
| DIETHYL PHTHALATE | 84-66-2 | 2,800 | 880 | E | 7,800 |  | 2,400 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| DIFLUBENZURON | 35367-38-5 | 20 | 52 | E | 20 |  | 52 | E | 20 |  | 52 | E | 20 |  |  | 52 | E | 20 | 52 | E | 20 |  | 52 | E | 20 |
| DIISOPROPYL METHYLPHOSPHONATE | 1445-75-6 | 60 | 8.2 | E | 60 |  | 8.2 | E | 6,000 |  | 820 | E | 6,000 |  |  | 820 | E | 60 | 8.2 | E | 60 |  | 8.2 | E | NA |
| DIMETHOATE | 60-51-5 | 7.6 | 2.9 | E | 21 |  | 8.1 | E | 760 |  | 290 | E | 2,100 |  |  | 810 | E | 7,600 | 2,900 | E | 21,000 |  | 8,100 | E | NA |
| DIMETHOXYBENZIDINE, 3,3- | 119-90-4 | 0.041 | 0.14 | E | 0.17 |  | 0.57 | E | 4.1 |  | 14 | E | 17 |  |  | 57 | E | 41 | 140 | E | 170 |  | 570 | E | 20 |
| DIMETHRIN | 70-38-2 | 3.6 | 240 | E | 3.6 |  | 240 | E | 3.6 |  | 240 | E | 3.6 |  |  | 240 | E | 3.6 | 240 | E | 3.6 |  | 240 | E | 10 |
| DIMETHYLAMINOAZO BENZENE, P- | 60-11-7 | 0.014 | 0.037 | E | 0.059 |  | 0.15 | E | 1.4 |  | 3.7 | E | 5.9 |  |  | 15 | E | 14 | 37 | E | 59 |  | 150 | E | 20 |
| DIMETHYLANILINE, N,N- | 121-69-7 | 2.4 | 1.3 | E | 10 |  | 5.6 | E | 240 |  | 130 | E | 1,000 |  |  | 560 | E | 240 | 130 | E | 1,000 |  | 560 | E | NA |
| DIMETHYLBENZIDINE, 3,3- | 119-93-7 | 0.0059 | 0.33 | E | 0.025 |  | 1.4 | E | 0.59 |  | 33 | E | 2.5 |  |  | 140 | E | 5.9 | 330 | E | 25 |  | 1,400 | E | 10 |
| DIMETHYL METHYLPHOSPHONATE | 756-79-6 | 10 | 1.2 | E | 10 |  | 1.2 | E | 1,000 |  | 120 | E | 1,000 |  |  | 120 | E | 10 | 1.2 | E | 10 |  | 1.2 | E | NA |
| DIMETHYLPHENOL, 2,4- | 105-67-9 | 69 | 30 | E | 190 |  | 83 | E | 6,900 |  | 3,000 | E | 10,000 |  |  | 8,300 | E | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| DINITROBENZENE, 1,3- | 99-65-0 | 0.1 | 0.049 | E | 0.1 |  | 0.049 | E | 10 |  | 4.9 | E | 10 |  |  | 4.9 | E | 100 | 49 | E | 100 |  | 49 | E | NA |
| DINITROPHENOL, 2,4- | 51-28-5 | 6.9 | 0.78 | E | 19 |  | 2.1 | E | 690 |  | 78 | E | 1,900 |  |  | 210 | E | 6,900 | 780 | E | 19,000 |  | 2,100 | E | NA |
| DINITROTOLUENE, 2,4- | 121-14-2 | 0.21 | 0.05 | E | 0.88 |  | 0.21 | E | 21 |  | 5 | E | 88 |  |  | 21 | E | 210 | 50 | E | 880 |  | 210 | E | NA |
| DINITROTOLUENE, 2,6- (2,6-DNT) | 606-20-2 | 0.043 | 0.013 | E | 0.18 |  | 0.053 | E | 4.3 |  | 1.3 | E | 18 |  |  | 5.3 | E | 43 | 13 | E | 180 |  | 53 | E | NA |
| DINOSEB | 88-85-7 | 0.7 | 0.29 | E | 0.7 |  | 0.29 | E | 70 |  | 29 | E | 70 |  |  | 29 | E | 700 | 290 | E | 700 |  | 290 | E | NA |
| DIOXANE, 1,4- | 123-91-1 | 0.65 | 0.085 | E | 2.7 |  | 0.35 | E | 65 |  | 8.5 | E | 270 |  |  | 35 | E | 6.5 | 0.85 | E | 27 |  | 3.5 | E | NA |
| DIPHENAMID | 957-51-7 | 20 | 12 | E | 20 |  | 12 | E | 2,000 |  | 1,200 | E | 2,000 |  |  | 1,200 | E | 20 | 12 | E | 20 |  | 12 | E | NA |
| DIPHENYLAMINE | 122-39-4 | 350 | 210 | E | 970 |  | 570 | E | 30,000 |  | 18,000 | E | 30,000 |  |  | 18,000 | E | 30,000 | 18,000 | E | 30,000 |  | 18,000 | E | NA |
| DIPHENYLHYDRAZINE, 1,2- | 122-66-7 | 0.022 | 0.039 | E | 0.11 |  | 0.19 | E | 2.2 |  | 3.9 | E | 11 |  |  | 19 | E | 2.2 | 3.9 | E | 11 |  | 19 | E | 30 |
| DIQUAT | [85-00-7] 2764-72-9 | 2 | 0.24 | E | 2 |  | 0.24 | E | 200 |  | 24 | E | 200 |  |  | 24 | E | 2 | 0.24 | E | 2 |  | 0.24 | E | NA |
| DISULFOTON | 298-04-4 | 0.07 | 0.18 | E | 0.07 |  | 0.18 | E | 7 |  | 18 | E | 7 |  |  | 18 | E | 70 | 180 | E | 70 |  | 180 | E | 20 |
| DITHIANE, 1,4- | 505-29-3 | 8 | 1.3 | E | 8 |  | 1.3 | E | 800 |  | 130 | E | 800 |  |  | 130 | E | 8 | 1.3 | E | 8 |  | 1.3 | E | NA |
| DIURON | 330-54-1 | 6.9 | 5.9 | E | 19 |  | 16 | E | 690 |  | 590 | E | 1,900 |  |  | 1,600 | E | 6.9 | 5.9 | E | 19 |  | 16 | E | NA |
| ENDOSULFAN | 115-29-7 | 21 | 110 | E | 48 |  | 250 | E | 48 |  | 250 | E | 48 |  |  | 250 | E | 48 | 250 | E | 48 |  | 250 | E | 15 |
| ENDOSULFAN I (ALPHA) | 959-98-8 | 21 | 110 | E | 50 |  | 260 | E | 50 |  | 260 | E | 50 |  |  | 260 | E | 21 | 110 | E | 50 |  | 260 | E | 15 |
| ENDOSULFAN II (BETA) | 33213-65-9 | 21 | 120 | E | 45 |  | 260 | E | 45 |  | 260 | E | 45 |  |  | 260 | E | 21 | 120 | E | 45 |  | 260 | E | 15 |
| ENDOSULFAN SULFATE | 1031-07-8 | 12 | 70 | E | 12 |  | 70 | E | 12 |  | 70 | E | 12 |  |  | 70 | E | 12 | 70 | E | 12 |  | 70 | E | 15 |
| ENDOTHALL | 145-73-3 | 10 | 4.1 | E | 10 |  | 4.1 | E | 1,000 |  | 410 | E | 1,000 |  |  | 410 | E | 10 | 4.1 | E | 10 |  | 4.1 | E | NA |
| ENDRIN | 72-20-8 | 0.2 | 5.5 | E | 0.2 |  | 5.5 | E | 20 |  | 550 | E | 20 |  |  | 550 | E | 0.2 | 5.5 | E | 0.2 |  | 5.5 | E | 15 |
| EPICHLOROHYDRIN | 106-89-8 | 0.21 | 0.042 | E | 0.88 |  | 0.17 | E | 21 |  | 4.2 | E | 88 |  |  | 17 | E | 21 | 4.2 | E | 88 |  | 17 | E | NA |
| ETHEPHON | 16672-87-0 | 17 | 2 | E | 49 |  | 5.7 | E | 1,700 |  | 200 | E | 4,900 |  |  | 570 | E | 17 | 2 | E | 49 |  | 5.7 | E | NA |
| ETHION | 563-12-2 | 1.7 | 37 | E | 4.9 |  | 110 | E | 85 |  | 1,900 | E | 85 |  |  | 1,900 | E | 1.7 | 37 | E | 4.9 |  | 110 | E | 15 |
| ETHOXYETHANOL, 2- (EGEE) | 110-80-5 | [42] 8.4 | [5.9] 1.2 | E | [180] 35 |  | [25] 4.9 | E | [4,200] 840 |  | [590] 120 | E | [10,000] 3,500 |  |  | [2,500] 490 | E | [4,200] 840 | [590] 120 | E | [10,000] 3,500 |  | [2,500] 490 | E | NA |
| ETHYL ACETATE | 141-78-6 | 15 | 3.9 | E | 62 |  | 16 | E | 1,500 |  | 390 | E | 6,200 |  |  | 1,600 | E | 1,500 | 390 | E | 6,200 |  | 1,600 | E | NA |
| ETHYL ACRYLATE | 140-88-5 | [1.4] 1.7 | [ 0.54] 0.66 | E | [5.7] 7 |  | [2.2] 2.7 | E | [140] 170 |  | [54] 66 | E | [570] 700 |  |  | [220] 270 | E | [140] 170 | [54] 66 | E | [570] 700 |  | [220] 270 | E | NA |
| ETHYL BENZENE | 100-41-4 | 70 | 46 | E | 70 |  | 46 | E | 7,000 |  | 4,600 | E | 7,000 |  |  | 4,600 | E | 7,000 | 4,600 | E | 7,000 |  | 4,600 | E | NA |
| ETHYL DIPROPYL THIOCARBAMATE, S- (EPTC) | 759-94-4 | 170 | 120 | E | 490 |  | 350 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 170 | 120 | E | 490 |  | 350 | E | NA |
| ETHYL ETHER | 60-29-7 | 690 | 190 | E | 1,900 |  | 530 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 690 | 190 | E | 1,900 |  | 530 | E | NA |
| ETHYL METHACRYLATE | 97-63-2 | 63 | 10 | E | 260 |  | 43 | E | 6,300 |  | 1,000 | E | 10,000 |  |  | 4,300 | E | 63 | 10 | E | 260 |  | 43 | E | NA |
| ETHYLENE CHLORHYDRIN | 107-07-3 | 69 | 7.9 | E | 190 |  | 22 | E | 6,900 |  | 790 | E | 10,000 |  |  | 2,200 | E | 69 | 7.9 | E | 190 |  | 22 | E | NA |
| ETHYLENE GLYCOL | 107-21-1 | 1,400 | 170 | E | 1,400 |  | 170 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| ETHYLENE THIOUREA (ETU) | 96-45-7 | 0.28 | 0.031 | E | 0.78 |  | 0.087 | E | 28 |  | 3.1 | E | 78 |  |  | 8.7 | E | 280 | 31 | E | 780 |  | 87 | E | NA |
| ETHYLP-NITROPHENYL PHENYLPHOSPHORO THIOATE | 2104-64-5 | 0.035 | 0.11 | E | 0.097 |  | 0.3 | E | 3.5 |  | 11 | E | 9.7 |  |  | 30 | E | 0.035 | 0.11 | E | 0.097 |  | 0.3 | E | 20 |
| FENAMIPHOS | 22224-92-6 | 0.07 | 0.06 | E | 0.07 |  | 0.06 | E | 7 |  | 6 | E | 7 |  |  | 6 | E | 0.07 | 0.06 | E | 0.07 |  | 0.06 | E | NA |
| FENVALERATE (PYDRIN) | 51630-58-1 | 8.5 | 94 | E | 8.5 |  | 94 | E | 8.5 |  | 94 | E | 8.5 |  |  | 94 | E | 8.5 | 94 | E | 8.5 |  | 94 | E | 15 |
| FLUOMETURON | 2164-17-2 | 9 | 2.5 | E | 9 |  | 2.5 | E | 900 |  | 250 | E | 900 |  |  | 250 | E | 9 | 2.5 | E | 9 |  | 2.5 | E | NA |
| FLUORANTHENE | 206-44-0 | 26 | 3,200 | E | 26 |  | 3,200 | E | 26 |  | 3,200 | E | 26 |  |  | 3,200 | E | 26 | 3,200 | E | 26 |  | 3,200 | E | 10 |
| FLUORENE | 86-73-7 | 140 | 2,800 | E | 190 |  | 3,800 | E | 190 |  | 3,800 | E | 190 |  |  | 3,800 | E | 190 | 3,800 | E | 190 |  | 3,800 | E | 15 |
| FLUOROTRICHLORO METHANE (FREON 11) | 75-69-4 | 200 | 87 | E | 200 |  | 87 | E | 10,000 |  | 8,700 | E | 10,000 |  |  | 8,700 | E | 10,000 | 8,700 | E | 10,000 |  | 8,700 | E | NA |
| FONOFOS | 944-22-9 | 1 | 2.9 | E | 1 |  | 2.9 | E | 100 |  | 290 | E | 100 |  |  | 290 | E | 1 | 2.9 | E | 1 |  | 2.9 | E | 20 |
| FORMALDEHYDE | 50-00-0 | 100 | 12 | E | 100 |  | 12 | E | 10,000 |  | 1,200 | E | 10,000 |  |  | 1,200 | E | 10,000 | 1,200 | E | 10,000 |  | 1,200 | E | NA |
| FORMIC ACID | 64-18-6 | 0.063 | 0.0071 | E | 0.26 |  | 0.029 | E | 6.3 |  | 0.71 | E | 26 |  |  | 2.9 | E | 0.63 | 0.071 | E | 2.6 |  | 0.29 | E | NA |
| FOSETYL-AL | 39148-24-8 | 8,700 | 7,700 | E | 24,000 |  | 21,000 | E | 190,000 |  | 190,000 | C | 190,000 |  |  | 190,000 | C | 8,700 | 7,700 | E | 24,000 |  | 21,000 | E | NA |
| FURAN | 110-00-9 | 3.5 | 1.5 | E | 9.7 |  | 4.2 | E | 350 |  | 150 | E | 970 |  |  | 420 | E | 350 | 150 | E | 970 |  | 420 | E | NA |
| FURFURAL | 98-01-1 | 1.9 | 0.24 | E | 7.8 |  | 0.99 | E | 190 |  | 24 | E | 780 |  |  | 99 | E | 1.9 | 0.24 | E | 7.8 |  | 0.99 | E | NA |
| GLYPHOSATE | 1071-83-6 | 70 | 620 | E | 70 |  | 620 | E | 7,000 |  | 62,000 | E | 7,000 |  |  | 62,000 | E | 70 | 620 | E | 70 |  | 620 | E | 15 |
| HEPTACHLOR | 76-44-8 | 0.04 | 0.68 | E | 0.04 |  | 0.68 | E | 4 |  | 68 | E | 4 |  |  | 68 | E | 18 | 310 | E | 18 |  | 310 | E | 15 |
| HEPTACHLOR EPOXIDE | 1024-57-3 | 0.02 | 1.1 | E | 0.02 |  | 1.1 | E | 2 |  | 110 | E | 2 |  |  | 110 | E | 20 | 1,100 | E | 20 |  | 1,100 | E | 10 |
| HEXACHLOROBENZENE | 118-74-1 | 0.1 | 0.96 | E | 0.1 |  | 0.96 | E | 0.6 |  | 5.8 | E | 0.6 |  |  | 5.8 | E | 0.6 | 5.8 | E | 0.6 |  | 5.8 | E | 15 |
| HEXACHLOROBUTADIENE | 87-68-3 | 0.84 | 10 | E | 3.5 |  | 42 | E | 84 |  | 1,000 | E | 290 |  |  | 3,400 | E | 290 | 3,400 | E | 290 |  | 3,400 | E | 15 |
| HEXACHLOROCYCLO PENTADIENE | 77-47-4 | 5 | 91 | E | 5 |  | 91 | E | 180 |  | 3,300 | E | 180 |  |  | 3,300 | E | 180 | 3,300 | E | 180 |  | 3,300 | E | 15 |
| HEXACHLOROETHANE | 67-72-1 | 0.1 | 0.56 | E | 0.1 |  | 0.56 | E | 10 |  | 56 | E | 10 |  |  | 56 | E | 10 | 56 | E | 10 |  | 56 | E | 15 |
| HEXAFLUOROPROPYLENE OXIDE (HFPO) DIMER ACID | 13252-13-6 | 0.001 | N/A |  | 0.001 |  | N/A |  | 0.1 |  | N/A |  | 0.1 |  |  | N/A |  | 0.001 | N/A |  | 0.001 |  | N/A |  | NA |
| HEXAFLUOROPROPYLENE OXIDE (HFPO) DIMER ACID AMMONIUM SALT | 62037-80-3 | 0.001 | N/A |  | 0.001 |  | N/A |  | 0.1 |  | N/A |  | 0.1 |  |  | N/A |  | 0.001 | N/A |  | 0.001 |  | N/A |  | NA |
| HEXANE | 110-54-3 | 150 | 1,400 | E | 580 |  | 5,300 | E | 950 |  | 8,700 | E | 950 |  |  | 8,700 | E | 150 | 1,400 | E | 580 |  | 5,300 | E | 15 |
| HEXAZINONE | 51235-04-2 | 40 | 8.5 | E | 40 |  | 8.5 | E | 4,000 |  | 850 | E | 4,000 |  |  | 850 | E | 40 | 8.5 | E | 40 |  | 8.5 | E | NA |
| HEXYTHIAZOX (SAVEY) | 78587-05-0 | 50 | 820 | E | 50 |  | 820 | E | 50 |  | 820 | E | 50 |  |  | 820 | E | 50 | 820 | E | 50 |  | 820 | E | 15 |
| HMX | 2691-41-0 | 40 | 4.8 | E | 40 |  | 4.8 | E | 500 |  | 60 | E | 500 |  |  | 60 | E | 40 | 4.8 | E | 40 |  | 4.8 | E | NA |
| HYDRAZINE/HYDRAZINE SULFATE | 302-01-2 | 0.001 | 0.00011 | E | 0.0051 |  | 0.00057 | E | 0.1 |  | 0.011 | E | 0.51 |  |  | 0.057 | E | 0.01 | 0.0011 | E | 0.051 |  | 0.0057 | E | NA |
| HYDROQUINONE | 123-31-9 | 1.1 | 0.15 | E | 4.5 |  | 0.61 | E | 110 |  | 15 | E | 450 |  |  | 61 | E | 1,100 | 150 | E | 4,500 |  | 610 | E | NA |
| INDENO[1,2,3-CD]PYRENE | 193-39-5 | [0.018] 0.21 | [1,400] 16,000 | E | [0.23] 2.7 |  | [18,000] 190,000 | [E] C | [1.8] 6.2 |  | [140,000] 190,000 | [E] C | 6.2 |  |  | 190,000 | C | 6.2 | 190,000 | C | 6.2 |  | 190,000 | C | 5 |
| IPRODIONE | 36734-19-7 | 1.5 | 4.3 | E | 6.2 |  | 18 | E | 150 |  | 430 | E | 620 |  |  | 1,800 | E | 1.5 | 4.3 | E | 6.2 |  | 18 | E | 20 |
| ISOBUTYL ALCOHOL | 78-83-1 | 1,000 | 260 | E | 2,900 |  | 760 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| ISOPHORONE | 78-59-1 | 10 | 1.9 | E | 10 |  | 1.9 | E | 1,000 |  | 190 | E | 1,000 |  |  | 190 | E | 10,000 | 1,900 | E | 10,000 |  | 1,900 | E | NA |
| ISOPROPYL METHYLPHOSPHONATE | 1832-54-8 | 70 | 8.1 | E | 70 |  | 8.1 | E | 7,000 |  | 810 | E | 7,000 |  |  | 810 | E | 70 | 8.1 | E | 70 |  | 8.1 | E | NA |
| KEPONE | 143-50-0 | 0.0065 | 0.89 | E | 0.027 |  | 3.7 | E | 0.65 |  | 89 | E | 2.7 |  |  | 370 | E | 6.5 | 890 | E | 27 |  | 3,700 | E | 10 |
| MALATHION | 121-75-5 | 50 | 170 | E | 50 |  | 170 | E | 5,000 |  | 10,000 | C | 5,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | 20 |
| MALEIC HYDRAZIDE | 123-33-1 | 400 | 47 | E | 400 |  | 47 | E | 40,000 |  | 4,700 | E | 40,000 |  |  | 4,700 | E | 400 | 47 | E | 400 |  | 47 | E | NA |
| MANEB | 12427-38-2 | 1.1 | 0.12 | E | 4.5 |  | 0.51 | E | 110 |  | 12 | E | 450 |  |  | 51 | E | 1.1 | 0.12 | E | 4.5 |  | 0.51 | E | NA |
| MERPHOS OXIDE | 78-48-8 | 1.7 | 230 | E | 4.9 |  | 650 | E | 170 |  | 10,000 | C | 230 |  |  | 10,000 | C | 1.7 | 230 | E | 4.9 |  | 650 | E | 10 |
| METHACRYLONITRILE | 126-98-7 | 0.35 | 0.057 | E | 0.97 |  | 0.16 | E | 35 |  | 5.7 | E | 97 |  |  | 16 | E | 0.35 | 0.057 | E | 0.97 |  | 0.16 | E | NA |
| METHAMIDOPHOS | 10265-92-6 | 0.17 | 0.021 | E | 0.49 |  | 0.061 | E | 17 |  | 2.1 | E | 49 |  |  | 6.1 | E | 0.17 | 0.021 | E | 0.49 |  | 0.061 | E | NA |
| METHANOL | 67-56-1 | 4,200 | 500 | E | 10,000 |  | 2,100 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| METHOMYL | 16752-77-5 | 20 | 3.2 | E | 20 |  | 3.2 | E | 2,000 |  | 320 | E | 2,000 |  |  | 320 | E | 20 | 3.2 | E | 20 |  | 3.2 | E | NA |
| METHOXYCHLOR | 72-43-5 | 4 | 630 | E | 4 |  | 630 | E | 4.5 |  | 710 | E | 4.5 |  |  | 710 | E | 4.5 | 710 | E | 4.5 |  | 710 | E | 10 |
| METHOXYETHANOL, 2- | 109-86-4 | [4.2] 1.5 | [0.48] 0.17 | E | [18] 6.2 |  | [2] 0.7 | E | [420] 150 |  | [48] 17 | E | [1,800] 620 |  |  | [200] 70 | E | [42] 15 | [4.8] 1.7 | E | [180] 62 |  | [20] 7 | E | NA |
| METHYL ACETATE | 79-20-9 | 3,500 | 650 | E | 9,700 |  | 1,800 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 3,500 | 650 | E | 9,700 |  | 1,800 | E | NA |
| METHYL ACRYLATE | 96-33-3 | 4.2 | 1 | E | 18 |  | 4.5 | E | 420 |  | 100 | E | 1,800 |  |  | 450 | E | 420 | 100 | E | 1,800 |  | 450 | E | NA |
| METHYL CHLORIDE | 74-87-3 | 3 | 0.38 | E | 3 |  | 0.38 | E | 300 |  | 38 | E | 300 |  |  | 38 | E | 300 | 38 | E | 300 |  | 38 | E | NA |
| METHYL ETHYL KETONE | 78-93-3 | 400 | 76 | E | 400 |  | 76 | E | 10,000 |  | 7,600 | E | 10,000 |  |  | 7,600 | E | 10,000 | 7,600 | E | 10,000 |  | 7,600 | E | NA |
| METHYL HYDRAZINE | 60-34-4 | 0.0042 | 0.00048 | E | 0.018 |  | 0.002 | E | 0.42 |  | 0.048 | E | 1.8 |  |  | 0.2 | E | 0.042 | 0.0048 | E | 0.18 |  | 0.02 | E | NA |
| METHYL ISOBUTYL KETONE | 108-10-1 | [280] 630 | [43] 97 | E | [780] 2,600 |  | [120] 400 | E | 10,000 |  | [4,300] 9,700 | E | 10,000 |  |  | 10,000 | C | 10,000 | [4,300] 9,700 | E | 10,000 |  | 10,000 | C | NA |
| METHYL ISOCYANATE | 624-83-9 | 0.21 | 0.029 | E | 0.88 |  | 0.12 | E | 21 |  | 2.9 | E | 88 |  |  | 12 | E | 0.21 | 0.029 | E | 0.88 |  | 0.12 | E | NA |
| METHYL N-BUTYL KETONE (2-HEXANONE) | 591-78-6 | 6.3 | 1.6 | E | 26 |  | 6.4 | E | 630 |  | 160 | E | 2,600 |  |  | 640 | E | 6.3 | 1.6 | E | 26 |  | 6.4 | E | NA |
| METHYL METHACRYLATE | 80-62-6 | 150 | 20 | E | 620 |  | 84 | E | 10,000 |  | 2,000 | E | 10,000 |  |  | 8,400 | E | 10,000 | 2,000 | E | 10,000 |  | 8,400 | E | NA |
| METHYL METHANESULFONATE | 66-27-3 | 0.66 | 0.082 | E | 2.7 |  | 0.34 | E | 66 |  | 8.2 | E | 270 |  |  | 34 | E | 0.66 | 0.082 | E | 2.7 |  | 0.34 | E | NA |
| METHYL PARATHION | 298-00-0 | 0.1 | 0.21 | E | 0.1 |  | 0.21 | E | 10 |  | 21 | E | 10 |  |  | 21 | E | 100 | 210 | E | 100 |  | 210 | E | 30 |
| METHYL STYRENE (MIXED ISOMERS) | 25013-15-4 | 8.4 | 47 | E | 35 |  | 200 | E | 840 |  | 4,700 | E | 3,500 |  |  | 10,000 | C | 8.4 | 47 | E | 35 |  | 200 | E | 15 |
| METHYL TERT-BUTYL ETHER (MTBE) | 1634-04-4 | 2 | 0.28 | E | 2 |  | 0.28 | E | 200 |  | 28 | E | 200 |  |  | 28 | E | 20 | 2.8 | E | 20 |  | 2.8 | E | NA |
| METHYLCHLOROPHENOXYACETIC ACID (MCPA) | 94-74-6 | 3 | 1.2 | E | 3 |  | 1.2 | E | 300 |  | 120 | E | 300 |  |  | 120 | E | 3,000 | 1,200 | E | 3,000 |  | 1,200 | E | NA |
| METHYLENE BIS(2-CHLOROANILINE), 4,4'- | 101-14-4 | 0.21 | 1.6 | E | 2.7 |  | 21 | E | 21 |  | 160 | E | 270 |  |  | 2,100 | E | 0.21 | 1.6 | E | 2.7 |  | 21 | E | 15 |
| METHYLNAPHTHALENE, 2- | 91-57-6 | 0.63 | 25 | E | 2.6 |  | 100 | E | 63 |  | 2,500 | E | 260 |  |  | 10,000 | E | 0.63 | 25 | E | 2.6 |  | 100 | E | 15 |
| METHYLSTYRENE, ALPHA | 98-83-9 | 240 | 420 | E | 680 |  | 1,200 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 240 | 420 | E | 680 |  | 1,200 | E | 30 |
| METOLACHLOR | 51218-45-2 | 70 | 40 | E | 70 |  | 40 | E | 7,000 |  | 4,000 | E | 7,000 |  |  | 4,000 | E | 70 | 40 | E | 70 |  | 40 | E | NA |
| METRIBUZIN | 21087-64-9 | 7 | 2.4 | E | 7 |  | 2.4 | E | 700 |  | 240 | E | 700 |  |  | 240 | E | 7 | 2.4 | E | 7 |  | 2.4 | E | NA |
| MEVINPHOS | 7786-34-7 | 0.087 | 0.019 | E | 0.24 |  | 0.053 | E | 8.7 |  | 1.9 | E | 24 |  |  | 5.3 | E | 0.087 | 0.019 | E | 0.24 |  | 0.053 | E | NA |
| MONOCHLOROACETIC ACID (HAA) | 79-11-8 | 6 | 0.67 | E | 6 |  | 0.67 | E | 600 |  | 67 | E | 600 |  |  | 67 | E | 6 | 0.67 | E | 6 |  | 0.67 | E | NA |
| NAPHTHALENE | 91-20-3 | 10 | 25 | E | 10 |  | 25 | E | 1,000 |  | 2,500 | E | 1,000 |  |  | 2,500 | E | 1,000 | 2,500 | E | 1,000 |  | 2,500 | E | 30 |
| NAPHTHYLAMINE, 1- | 134-32-7 | 0.036 | 0.29 | E | 0.15 |  | 1.2 | E | 3.6 |  | 29 | E | 15 |  |  | 120 | E | 3.6 | 29 | E | 15 |  | 120 | E | 15 |
| NAPHTHYLAMINE, 2- | 91-59-8 | 0.036 | 0.012 | E | 0.15 |  | 0.049 | E | 3.6 |  | 1.2 | E | 15 |  |  | 4.9 | E | 36 | 12 | E | 150 |  | 49 | E | NA |
| NAPROPAMIDE | 15299-99-7 | 420 | 970 | E | 1,200 |  | 2,800 | E | 7,000 |  | 16,000 | E | 7,000 |  |  | 16,000 | E | 420 | 970 | E | 1,200 |  | 2,800 | E | 30 |
| NITROANILINE, O- | 88-74-4 | 0.011 | 0.002 | E | 0.044 |  | 0.0079 | E | 1.1 |  | 0.2 | E | 4.4 |  |  | 0.79 | E | 0.011 | 0.002 | E | 0.044 |  | 0.0079 | E | NA |
| NITROANILINE, P- | 100-01-6 | 3.3 | 0.49 | E | 14 |  | 2.1 | E | 330 |  | 49 | E | 1,400 |  |  | 210 | E | 3.3 | 0.49 | E | 14 |  | 2.1 | E | NA |
| NITROBENZENE | 98-95-3 | 0.12 | 0.052 | E | 0.63 |  | 0.27 | E | 12 |  | 5.2 | E | 63 |  |  | 27 | E | 12 | 5.2 | E | 63 |  | 27 | [C] E | NA |
| NITROGUANIDINE | 556-88-7 | 70 | 7.8 | E | 70 |  | 7.8 | E | 7,000 |  | 780 | E | 7,000 |  |  | 780 | E | 70 | 7.8 | E | 70 |  | 7.8 | E | NA |
| NITROPHENOL, 2- | 88-75-5 | 28 | 5.7 | E | 78 |  | 16 | E | 2,800 |  | 570 | E | 7,800 |  |  | 1,600 | E | 2,800 | 570 | E | 7,800 |  | 1,600 | E | NA |
| NITROPHENOL, 4- | 100-02-7 | 6 | 4.1 | E | 6 |  | 4.1 | E | 600 |  | 410 | E | 600 |  |  | 410 | E | 600 | 410 | E | 600 |  | 410 | E | NA |
| NITROPROPANE, 2- | 79-46-9 | [0.0018] 0.0084 | [0.00029] 0.0014 | E | [0.0093] 0.043 |  | [0.0015] 0.0069 | E | [0.18] 0.84 |  | [0.029] 0.14 | E | [0.93] 4.3 |  |  | [0.15] 0.69 | E | [0.018] 0.084 | [0.0029] 0.014 | E | [0.093] 0.43 |  | [0.015] 0.069 | E | NA |
| NITROSODIETHYLAMINE, N- | 55-18-5 | 0.000045 | 0.0000079 | E | 0.00058 |  | 0.0001 | E | 0.0045 |  | 0.00079 | E | 0.058 |  |  | 0.01 | E | 0.00045 | 0.000079 | E | 0.0058 |  | 0.001 | E | NA |
| NITROSODIMETHYLAMINE, N- | 62-75-9 | 0.00014 | 0.000019 | E | 0.0018 |  | 0.00024 | E | 0.014 |  | 0.0019 | E | 0.18 |  |  | 0.024 | E | 0.0014 | 0.00019 | E | 0.018 |  | 0.0024 | E | NA |
| NITROSO-DI-N-BUTYLAMINE, N- | 924-16-3 | 0.0031 | 0.0038 | E | 0.016 |  | 0.02 | E | 0.31 |  | 0.38 | E | 1.6 |  |  | 2 | E | 0.31 | 0.38 | E | 1.6 |  | 2 | E | NA |
| NITROSODI-N-PROPYLAMINE, N- | 621-64-7 | 0.0025 | 0.00035 | E | 0.013 |  | 0.0018 | E | 0.25 |  | 0.035 | E | 1.3 |  |  | 0.18 | E | 0.025 | 0.0035 | E | 0.13 |  | 0.018 | E | NA |
| NITROSODIPHENYLAMINE, N- | 86-30-6 | 1.9 | 3 | E | 9.6 |  | 15 | E | 190 |  | 300 | E | 960 |  |  | 1,500 | E | 190 | 300 | E | 960 |  | 1,500 | E | 30 |
| NITROSO-N-ETHYLUREA, N- | 759-73-9 | 0.00079 | 0.000091 | E | 0.01 |  | 0.0012 | E | 0.079 |  | 0.0091 | E | 1 |  |  | 0.12 | E | 0.79 | 0.091 | E | 10 |  | 1.2 | E | NA |
| OCTYL PHTHALATE, DI-N- | 117-84-0 | 35 | 10,000 | C | 97 |  | 10,000 | C | 300 |  | 10,000 | C | 300 |  |  | 10,000 | C | 300 | 10,000 | C | 300 |  | 10,000 | C | 5 |
| OXAMYL (VYDATE) | 23135-22-0 | 20 | 2.6 | E | 20 |  | 2.6 | E | 2,000 |  | 260 | E | 2,000 |  |  | 260 | E | 20 | 2.6 | E | 20 |  | 2.6 | E | NA |
| PARAQUAT | 1910-42-5 | 3 | 120 | E | 3 |  | 120 | E | 300 |  | 12,000 | E | 300 |  |  | 12,000 | E | 3 | 120 | E | 3 |  | 120 | E | 15 |
| PARATHION | 56-38-2 | 0.1 | 0.59 | E | 0.29 |  | 1.7 | E | 10 |  | 59 | E | 29 |  |  | 170 | E | 0.1 | 0.59 | E | 0.29 |  | 1.7 | E | 15 |
| PCBS, TOTAL (POLYCHLORINATED BIPHENLYS) (AROCLORS) | 1336-36-3 | 0.05 | 9.8 | E | 0.05 |  | 9.8 | E | 5 |  | 980 | E | 5 |  |  | 980 | E | 0.05 | 9.8 | E | 0.05 |  | 9.8 | E | 10 |
| PCB-1016 (AROCLOR) | 12674-11-2 | 0.24 | 66 | E | 0.68 |  | 190 | E | 24 |  | 6,600 | E | 25 |  |  | 6,900 | E | 0.24 | 66 | E | 0.68 |  | 190 | E | 10 |
| PCB-1221 (AROCLOR) | 11104-28-2 | 0.033 | 0.16 | E | 0.14 |  | 0.68 | E | 3.3 |  | 16 | E | 14 |  |  | 68 | E | 0.033 | 0.16 | E | 0.14 |  | 0.68 | E | 20 |
| PCB-1232 (AROCLOR) | 11141-16-5 | 0.033 | 0.13 | E | 0.14 |  | 0.54 | E | 3.3 |  | 13 | E | 14 |  |  | 54 | E | 0.033 | 0.13 | E | 0.14 |  | 0.54 | E | 20 |
| PCB-1242 (AROCLOR) | 53469-21-9 | 0.033 | 4 | E | 0.14 |  | 17 | E | 3.3 |  | 400 | E | 10 |  |  | 1,200 | E | 0.033 | 4 | E | 0.14 |  | 17 | E | 10 |
| PCB-1248 (AROCLOR) | 12672-29-6 | 0.033 | 16 | E | 0.14 |  | 67 | E | 3.3 |  | 1,600 | E | 5.4 |  |  | 2,600 | E | 0.033 | 16 | E | 0.14 |  | 67 | E | 10 |
| PCB-1254 (AROCLOR) | 11097-69-1 | 0.069 | 140 | E | 0.19 |  | 380 | E | 5.7 |  | 10,000 | C | 5.7 |  |  | 10,000 | C | 0.069 | 140 | E | 0.19 |  | 380 | E | 5 |
| PCB-1260 (AROCLOR) | 11096-82-5 | 0.033 | 150 | E | 0.14 |  | 630 | E | 3.3 |  | 15,000 | E | 8 |  |  | 36,000 | E | 0.033 | 150 | E | 0.14 |  | 630 | E | 5 |
| PEBULATE | 1114-71-2 | [170] 2.4 | [290] 4 | E | [490] 6.8 |  | [830] 11 | E | [9,200] 240 |  | [10,000] 400 | [C] E | [9,200] 680 |  |  | [10,000] 1,100 | [C] E | [170] 2.4 | [290] 4 | E | [490] 6.8 |  | [830] 11 | E | 30 |
| PENTACHLOROBENZENE | 608-93-5 | 2.8 | 220 | E | 7.8 |  | 620 | E | 74 |  | 5,900 | E | 74 |  |  | 5,900 | E | 74 | 5,900 | E | 74 |  | 5,900 | E | 10 |
| PENTACHLOROETHANE | 76-01-7 | 0.72 | 3.5 | E | 3 |  | 15 | E | 72 |  | 350 | E | 300 |  |  | 1,500 | E | 0.72 | 3.5 | E | 3 |  | 15 | E | 20 |
| PENTACHLORO NITROBENZENE | 82-68-8 | 0.25 | 5 | E | 1 |  | 20 | E | 25 |  | 500 | E | 44 |  |  | 870 | E | 44 | 870 | E | 44 |  | 870 | E | 15 |
| PENTACHLOROPHENOL | 87-86-5 | 0.1 | 5 | E | 0.1 |  | 5 | E | 10 |  | 500 | E | 10 |  |  | 500 | E | 100 | 5,000 | E | 100 |  | 5,000 | E | 10 |
| PERFLUOROBUTANE SULFONATE (PFBS) | 375-73-5 | [1] 0.2 | N/A |  | [2.9] 0.2 |  | N/A |  | [100] 20 |  | N/A |  | [290] 20 |  |  | N/A |  | [1] 0.2 | N/A |  | [2.9] 0.2 |  | N/A |  | NA |
| PERFLUOROBUTANOIC ACID (PFBA) | 375-22-4 | 3.5 | N/A |  | 9.7 |  | N/A |  | 350 |  | N/A |  | 970 |  |  | N/A |  | 3.5 | N/A |  | 9.7 |  | N/A |  | NA |
| PERFLUOROHEXANE SULFONATE (PFHxS) | 355-46-4 | 0.001 | N/A |  | 0.001 |  | N/A |  | 0.1 |  | N/A |  | 0.1 |  |  | N/A |  | 0.001 | N/A |  | 0.001 |  | N/A |  | NA |
| PERFLUOROHEXANOIC ACID (PFHxA) | 307-24-4 | 1.7 | N/A |  | 4.9 |  | N/A |  | 170 |  | N/A |  | 490 |  |  | N/A |  | 1.7 | N/A |  | 4.9 |  | N/A |  | NA |
| PERFLUORONONANOIC ACID (PFNA) | 375-95-1 | 0.001 | N/A |  | 0.001 |  | N/A |  | 0.1 |  | N/A |  | 0.1 |  |  | N/A |  | 0.001 | N/A |  | 0.001 |  | N/A |  | NA |
| PERFLUOROOCTANE SULFONATE (PFOS) | 1763-23-1 | [0.007] 0.0018 0.0004 | N/A |  | [0.007] 0.0018 0.0004 |  | N/A |  | [0.7] 0.18 0.04 |  | N/A |  | [0.7] 0.18 0.04 |  |  | N/A |  | [0.007] 0.0018 0.0004 | N/A |  | [0.007] 0.0018 0.0004 |  | N/A |  | NA |
| PERFLUOROOCTANOIC ACID (PFOA) | 335-67-1 | [0.007] 0.0014 0.0004 | N/A |  | [0.007] 0.0014 0.0004 |  | N/A |  | [0.7] 0.14 0.04 |  | N/A |  | [0.7] 0.14 0.04 |  |  | N/A |  | [0.007] 0.0014 0.0004 | N/A |  | [0.007] 0.0014 0.0004 |  | N/A |  | NA |
| PHENACETIN | 62-44-2 | 30 | 12 | E | 120 |  | 46 | E | 3,000 |  | 1,200 | E | 12,000 |  |  | 4,600 | E | 30,000 | 12,000 | E | 76,000 |  | 29,000 | E | NA |
| PHENANTHRENE | 85-01-8 | 110 | 10,000 | E | 110 |  | 10,000 | E | 110 |  | 10,000 | E | 110 |  |  | 10,000 | E | 110 | 10,000 | E | 110 |  | 10,000 | E | 10 |
| PHENOL | 108-95-2 | 200 | 33 | E | 200 |  | 33 | E | 20,000 |  | 3,300 | E | 20,000 |  |  | 3,300 | E | 20,000 | 3,300 | E | 20,000 |  | 3,300 | E | NA |
| PHENYL MERCAPTAN | 108-98-5 | 3.5 | 5.3 | E | 9.7 |  | 15 | E | 350 |  | 530 | E | 970 |  |  | 1,500 | E | 3.5 | 5.3 | E | 9.7 |  | 15 | E | 30 |
| PHENYLENEDIAMINE, M- | 108-45-2 | 21 | 3 | E | 58 |  | 8.2 | E | 2,100 |  | 300 | E | 5,800 |  |  | 820 | E | 21,000 | 3,000 | E | 58,000 |  | 8,200 | E | NA |
| PHENYLPHENOL, 2- | 90-43-7 | 34 | 490 | E | 140 |  | 2,000 | E | 3,400 |  | 49,000 | E | 14,000 |  |  | 190,000 | C | 34,000 | 190,000 | C | 70,000 |  | 190,000 | C | 15 |
| PHORATE | 298-02-2 | [0.69] 0.59 | [1.5] 1.3 | E | [1.9] 1.7 |  | [4.1] 3.6 | E | [69] 59 |  | [150] 130 | E | [190] 170 |  |  | [410] 360 | E | [0.69] 0.59 | [1.5] 1.3 | E | [1.9] 1.7 |  | [4.1] 3.6 | E | 30 |
| PHTHALIC ANHYDRIDE | 85-44-9 | 4.2 | 1.3 | E | 18 |  | 5.6 | E | 420 |  | 130 | E | 1,800 |  |  | 560 | E | 420 | 130 | E | 1,800 |  | 560 | C | NA |
| PICLORAM | 1918-02-1 | 50 | 7.4 | E | 50 |  | 7.4 | E | 5,000 |  | 740 | E | 5,000 |  |  | 740 | E | 50 | 7.4 | E | 50 |  | 7.4 | E | NA |
| POTASSIUM PERFLUOROBUTANE SULFONATE | 29420-49-3 | 0.2 | N/A |  | 0.2 |  | N/A |  | 20 |  | N/A |  | 20 |  |  | N/A |  | 0.2 | N/A |  | 0.2 |  | N/A |  | NA |
| PROMETON | 1610-18-0 | 40 | 39 | E | 40 |  | 39 | E | 4,000 |  | 3,900 | E | 4,000 |  |  | 3,900 | E | 40 | 39 | E | 40 |  | 39 | E | NA |
| PRONAMIDE | 23950-58-5 | 260 | 160 | E | 730 |  | 450 | E | 1,500 |  | 920 | E | 1,500 |  |  | 920 | E | 260 | 160 | E | 730 |  | 450 | E | NA |
| PROPACHLOR | 1918-16-7 | 0.01 | 0.0046 | E | 0.01 |  | 0.0046 | E | 1 |  | 0.46 | E | 1 |  |  | 0.46 | E | 1 | 0.46 | E | 1 |  | 0.46 | E | NA |
| PROPANIL | 709-98-8 | 17 | 8.7 | E | 49 |  | 25 | E | 1,700 |  | 870 | E | 4,900 |  |  | 2,500 | E | 17 | 8.7 | E | 49 |  | 25 | E | NA |
| PROPANOL, 2- (ISOPROPYL ALCOHOL) | 67-63-0 | 42 | 7.3 | E | 180 |  | 31 | E | 4,200 |  | 730 | E | 10,000 |  |  | 3,100 | E | 42 | 7.3 | E | 180 |  | 31 | E | NA |
| PROPAZINE | 139-40-2 | 1 | 0.5 | E | 1 |  | 0.5 | E | 100 |  | 50 | E | 100 |  |  | 50 | E | 1 | 0.5 | E | 1 |  | 0.5 | E | NA |
| PROPHAM | 122-42-9 | 10 | 2.4 | E | 10 |  | 2.4 | E | 1,000 |  | 240 | E | 1,000 |  |  | 240 | E | 10 | 2.4 | E | 10 |  | 2.4 | E | NA |
| PROPYLBENZENE, N- | 103-65-1 | 210 | 400 | E | 880 |  | 1,700 | E | 5,200 |  | 9,900 | E | 5,200 |  |  | 9,900 | E | 210 | 400 | E | 880 |  | 1,700 | E | 30 |
| PROPYLENE OXIDE | 75-56-9 | 0.27 | 0.047 | E | 1.1 |  | 0.19 | E | 27 |  | 4.7 | E | 110 |  |  | 19 | E | 0.27 | 0.047 | E | 1.1 |  | 0.19 | E | NA |
| PYRENE | 129-00-0 | 13 | 2,200 | E | 13 |  | 2,200 | E | 13 |  | 2,200 | E | 13 |  |  | 2,200 | E | 13 | 2,200 | E | 13 |  | 2,200 | E | 10 |
| PYRETHRUM | 8003-34-7 | 35 | 4.4 | E | 35 |  | 4.4 | E | 35 |  | 4.4 | E | 35 |  |  | 4.4 | E | 35 | 4.4 | E | 35 |  | 4.4 | E | NA |
| PYRIDINE | 110-86-1 | [3.4] 3.5 | 0.39 | E | 9.7 |  | 1.1 | E | 350 |  | 39 | E | 970 |  |  | 110 | E | 35 | 3.9 | E | 97 |  | 11 | E | NA |
| QUINOLINE | 91-22-5 | 0.022 | 0.074 | E | 0.091 |  | 0.31 | E | 2.2 |  | 7.4 | E | 9.1 |  |  | 31 | E | 22 | 74 | E | 91 |  | 310 | E | 20 |
| QUIZALOFOP (ASSURE) | 76578-14-8 | 30 | 47 | E | 30 |  | 47 | E | 30 |  | 47 | E | 30 |  |  | 47 | E | 30 | 47 | E | 30 |  | 47 | E | 30 |
| RDX | 121-82-4 | 0.2 | 0.057 | E | 0.2 |  | 0.057 | E | 20 |  | 5.7 | E | 20 |  |  | 5.7 | E | 0.2 | 0.057 | E | 0.2 |  | 0.057 | E | NA |
| RESORCINOL | 108-46-3 | 6,900 | 800 | E | 19,000 |  | 2,200 | E | 190,000 |  | 80,000 | E | 190,000 |  |  | 190,000 | C | 6,900 | 800 | E | 19,000 |  | 2,200 | E | NA |
| RONNEL | 299-84-3 | 170 | 270 | E | 490 |  | 760 | E | 4,000 |  | 6,200 | E | 4,000 |  |  | 6,200 | E | 170 | 270 | E | 490 |  | 760 | E | 30 |
| SIMAZINE | 122-34-9 | 0.4 | 0.15 | E | 0.4 |  | 0.15 | E | 40 |  | 15 | E | 40 |  |  | 15 | E | 0.4 | 0.15 | E | 0.4 |  | 0.15 | E | NA |
| STRYCHNINE | 57-24-9 | 1 | 0.81 | E | 2.9 |  | 2.4 | E | 100 |  | 81 | E | 290 |  |  | 240 | E | 1,000 | 810 | E | 2,900 |  | 2,400 | E | NA |
| STYRENE | 100-42-5 | 10 | 24 | E | 10 |  | 24 | E | 1,000 |  | 2,400 | E | 1,000 |  |  | 2,400 | E | 1,000 | 2,400 | E | 1,000 |  | 2,400 | E | 30 |
| TEBUTHIURON | 34014-18-1 | 50 | 83 | E | 50 |  | 83 | E | 5,000 |  | 8,300 | E | 5,000 |  |  | 8,300 | E | 50 | 83 | E | 50 |  | 83 | E | 30 |
| TERBACIL | 5902-51-2 | 9 | 2.2 | E | 9 |  | 2.2 | E | 900 |  | 220 | E | 900 |  |  | 220 | E | 9 | 2.2 | E | 9 |  | 2.2 | E | NA |
| TERBUFOS | 13071-79-9 | 0.04 | 0.055 | E | 0.04 |  | 0.055 | E | 4 |  | 5.5 | E | 4 |  |  | 5.5 | E | 0.04 | 0.055 | E | 0.04 |  | 0.055 | E | 30 |
| TETRACHLOROBENZENE, 1,2,4,5- | 95-94-3 | [1] 0.1 | [4.6] 0.46 | E | [2.9] 0.29 |  | [13] 1.3 | E | [58] 10 |  | [270] 46 | E | [58] 29 |  |  | [270] 130 | E | 58 | 270 | E | 58 |  | 270 | E | 20 |
| TETRACHLORODIBENZO-P-DIOXIN, 2,3,7,8- (TCDD) | 1746-01-6 | 0.000003 | 0.032 | E | 0.000003 |  | 0.032 | E | 0.0003 |  | 3.2 | E | 0.0003 |  |  | 3.2 | E | 0.0019 | 20 | E | 0.0019 |  | 20 | E | 5 |
| TETRACHLOROETHANE, 1,1,1,2- | 630-20-6 | 7 | 18 | E | 7 |  | 18 | E | 700 |  | 1,800 | E | 700 |  |  | 1,800 | E | 700 | 1,800 | E | 700 |  | 1,800 | E | 30 |
| TETRACHLOROETHANE, 1,1,2,2- | 79-34-5 | 0.084 | 0.026 | E | 0.43 |  | 0.13 | E | 8.4 |  | 2.6 | E | 43 |  |  | 13 | E | 8.4 | 2.6 | E | 43 |  | 13 | E | NA |
| TETRACHLOROETHYLENE (PCE) | 127-18-4 | 0.5 | 0.43 | E | 0.5 |  | 0.43 | E | 50 |  | 43 | E | 50 |  |  | 43 | E | 5 | 4.3 | E | 5 |  | 4.3 | E | NA |
| TETRACHLOROPHENOL, 2,3,4,6- | 58-90-2 | 100 | 1,600 | E | 290 |  | 4,500 | E | 10,000 |  | 160,000 | E | 18,000 |  |  | 190,000 | C | 18,000 | 190,000 | C | 18,000 |  | 190,000 | C | 15 |
| TETRAETHYL LEAD | 78-00-2 | 0.00035 | 0.0043 | E | 0.00097 |  | 0.012 | E | 0.035 |  | 0.43 | E | 0.097 |  |  | 1.2 | E | 0.35 | 4.3 | E | 0.97 |  | 12 | E | 15 |
| TETRAETHYLDITHIO PYROPHOSPHATE | 3689-24-5 | 1.7 | 2.5 | E | 4.9 |  | 7.3 | E | 170 |  | 250 | E | 490 |  |  | 730 | E | 1.7 | 2.5 | E | 4.9 |  | 7.3 | E | 30 |
| TETRAHYDROFURAN | 109-99-9 | 2.5 | 0.55 | E | 13 |  | 2.8 | E | 250 |  | 55 | E | 1,300 |  |  | 280 | E | 2.5 | 0.55 | E | 13 |  | 2.8 | E | NA |
| THIOFANOX | 39196-18-4 | 1 | 0.11 | E | 2.9 |  | 0.32 | E | 100 |  | 11 | E | 290 |  |  | 32 | E | 1 | 0.11 | E | 2.9 |  | 0.32 | E | NA |
| THIRAM | 137-26-8 | 52 | 140 | E | 150 |  | 390 | E | 3,000 |  | 7,800 | E | 3,000 |  |  | 7,800 | E | 52 | 140 | E | 150 |  | 390 | E | 20 |
| TOLUENE | 108-88-3 | 100 | 44 | E | 100 |  | 44 | E | 10,000 |  | 4,400 | E | 10,000 |  |  | 4,400 | E | 10,000 | 4,400 | E | 10,000 |  | 4,400 | E | NA |
| TOLUIDINE, M- | 108-44-1 | 4.1 | 1.9 | E | 17 |  | 7.8 | E | 410 |  | 190 | E | 1,700 |  |  | 780 | E | 4.1 | 1.9 | E | 17 |  | 7.8 | E | NA |
| TOLUIDINE, O- | 95-53-4 | 4.1 | 4.7 | E | 17 |  | 19 | E | 410 |  | 470 | E | 1,700 |  |  | 1,900 | E | 4,100 | 4,700 | E | 10,000 |  | 10,000 | C | NA |
| TOLUIDINE, P- | 106-49-0 | 2.2 | 2 | E | 9.1 |  | 8.3 | E | 220 |  | 200 | E | 910 |  |  | 830 | E | 2.2 | 2 | E | 9.1 |  | 8.3 | E | NA |
| TOXAPHENE | 8001-35-2 | 0.3 | 1.2 | E | 0.3 |  | 1.2 | E | 30 |  | 120 | E | 30 |  |  | 120 | E | 0.3 | 1.2 | E | 0.3 |  | 1.2 | E | 20 |
| TRIALLATE | 2303-17-5 | [0.091] 0.91 | [0.47] 4.7 | E | [0.38] 3.8 |  | [1.9] 19 | E | [9.1] 91 |  | [47] 470 | E | [38] 380 |  |  | [190] 1,900 | E | [0.091] 0.91 | [0.47] 4.7 | E | [0.38] 3.8 |  | [1.9] 19 | E | 15 |
| TRIBROMOMETHANE (BROMOFORM) (THM) | 75-25-2 | 8 | 3.5 | E | 8 |  | 3.5 | E | 800 |  | 350 | E | 800 |  |  | 350 | E | 800 | 350 | E | 800 |  | 350 | E | NA |
| TRICHLORO-1,2,2-TRIFLUOROETHANE, 1,1,2- | 76-13-1 | 1,100 | 3,400 | E | 4,400 |  | 10,000 | C | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | 20 |
| TRICHLOROACETIC ACID (HAA) | 76-03-9 | 6 | 0.97 | E | 6 |  | 0.97 | E | 600 |  | 97 | E | 600 |  |  | 97 | E | 6 | 0.97 | E | 6 |  | 0.97 | E | NA |
| TRICHLOROBENZENE, 1,2,4- | 120-82-1 | 7 | 27 | E | 7 |  | 27 | E | 700 |  | 2,700 | E | 700 |  |  | 2,700 | E | 700 | 2,700 | E | 700 |  | 2,700 | E | 20 |
| TRICHLOROBENZENE, 1,3,5- | 108-70-3 | 4 | 31 | E | 4 |  | 31 | E | 400 |  | 3,100 | E | 400 |  |  | 3,100 | E | 4 | 31 | E | 4 |  | 31 | E | 15 |
| TRICHLOROETHANE, 1,1,1- | 71-55-6 | 20 | 7.2 | E | 20 |  | 7.2 | E | 2,000 |  | 720 | E | 2,000 |  |  | 720 | E | 200 | 72 | E | 200 |  | 72 | E | NA |
| TRICHLOROETHANE, 1,1,2- | 79-00-5 | 0.5 | 0.15 | E | 0.5 |  | 0.15 | E | 50 |  | 15 | E | 50 |  |  | 15 | E | 5 | 1.5 | E | 5 |  | 1.5 | E | NA |
| TRICHLOROETHYLENE (TCE) | 79-01-6 | 0.5 | 0.17 | E | 0.5 |  | 0.17 | E | 50 |  | 17 | E | 50 |  |  | 17 | E | 5 | 1.7 | E | 5 |  | 1.7 | E | NA |
| TRICHLOROPHENOL, 2,4,5- | 95-95-4 | 350 | 2,100 | E | 970 |  | 5,900 | E | 35,000 |  | 190,000 | C | 97,000 |  |  | 190,000 | C | 100,000 | 190,000 | C | 100,000 |  | 190,000 | C | 15 |
| TRICHLOROPHENOL, 2,4,6- | 88-06-2 | 3.5 | 10 | E | 9.7 |  | 28 | E | 350 |  | 1,000 | E | 970 |  |  | 2,800 | E | 3,500 | 10,000 | E | 9,700 |  | 28,000 | E | 20 |
| TRICHLOROPHENOXY ACETIC ACID, 2,4,5- (2,4,5-T) | 93-76-5 | 7 | 1.5 | E | 7 |  | 1.5 | E | 700 |  | 150 | E | 700 |  |  | 150 | E | 7,000 | 1,500 | E | 7,000 |  | 1,500 | E | NA |
| TRICHLOROPHENOXY PROPIONIC ACID, 2,4,5- (2,4,5-TP)(SILVEX) | 93-72-1 | 5 | 22 | E | 5 |  | 22 | E | 500 |  | 2,200 | E | 500 |  |  | 2,200 | E | 5 | 22 | E | 5 |  | 22 | E | 20 |
| TRICHLOROPROPANE, 1,1,2- | 598-77-6 | 17 | 2.9 | E | 49 |  | 8.4 | E | 1,700 |  | 290 | E | 4,900 |  |  | 840 | E | 17 | 2.9 | E | 49 |  | 8.4 | E | NA |
| TRICHLOROPROPANE, 1,2,3- | 96-18-4 | [4] 0.00071 | [3.2] 0.00058 | E | [4] 0.0091 |  | [3.2] 0.0074 | E | [400] 0.071 |  | [320] 0.058 | E | [400] 0.91 |  |  | [320] 0.74 | E | [400] 0.071 | [320] 0.058 | E | [400] 0.91 |  | [320] 0.74 | E | NA |
| TRICHLOROPROPENE, 1,2,3- | 96-19-5 | 0.063 | 0.037 | E | 0.26 |  | 0.15 | E | 6.3 |  | 3.7 | E | 26 |  |  | 15 | E | 0.063 | 0.037 | E | 0.26 |  | 0.15 | E | NA |
| TRIETHYLAMINE | 121-44-8 | 1.5 | 0.36 | E | 6.2 |  | 1.5 | E | 150 |  | 36 | E | 620 |  |  | 150 | E | 1.5 | 0.36 | E | 6.2 |  | 1.5 | E | NA |
| TRIETHYLENE GLYCOL | 112-27-6 | 6,900 | 870 | E | 10,000 |  | 2,400 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 6,900 | 870 | E | 10,000 |  | 2,400 | E | NA |
| TRIFLURALIN | 1582-09-8 | 1 | 1.9 | E | 1 |  | 1.9 | E | 100 |  | 190 | E | 100 |  |  | 190 | E | 1 | 1.9 | E | 1 |  | 1.9 | E | 30 |
| TRIMETHYLBENZENE, 1,3,4- (TRIMETHYLBENZENE, 1,2,4-) | 95-63-6 | 13 | 73 | E | 53 |  | 300 | E | 1,300 |  | 7,300 | E | 5,300 |  |  | 10,000 | C | 1,300 | 7,300 | E | 5,300 |  | 10,000 | C | 15 |
| TRIMETHYLBENZENE, 1,3,5- | 108-67-8 | 13 | 23 | E | 53 |  | 93 | E | 1,300 |  | 2,300 | E | 4,900 |  |  | 8,600 | E | 13 | 23 | E | 53 |  | 93 | E | 30 |
| TRINITROGLYCEROL (NITROGLYCERIN) | 55-63-0 | 0.5 | 0.2 | E | 0.5 |  | 0.2 | E | 50 |  | 20 | E | 50 |  |  | 20 | E | 50 | 20 | E | 50 |  | 20 | E | NA |
| TRINITROTOLUENE, 2,4,6- | 118-96-7 | 0.2 | 0.023 | E | 0.2 |  | 0.023 | E | 20 |  | 2.3 | E | 20 |  |  | 2.3 | E | 0.2 | 0.023 | E | 0.2 |  | 0.023 | E | NA |
| VINYL ACETATE | 108-05-4 | 42 | 5 | E | 180 |  | 21 | E | 4,200 |  | 500 | E | 10,000 |  |  | 2,100 | E | 42 | 5 | E | 180 |  | 21 | E | NA |
| VINYL BROMIDE (BROMOETHENE) | 593-60-2 | [0.15] 0.33 | [0.073] 0.16 | E | [0.78] 1.7 |  | [0.38] 0.83 | E | [15] 33 |  | [7.3] 16 | E | [78] 170 |  |  | [38] 83 | E | [1.5] 3.3 | [0.73] 1.6 | E | [7.8] 17 |  | [3.8] 8.3 | E | NA |
| VINYL CHLORIDE | 75-01-4 | 0.2 | 0.027 | E | 0.2 |  | 0.027 | E | 20 |  | 2.7 | E | 20 |  |  | 2.7 | E | 2 | 0.27 | E | 2 |  | 0.27 | E | NA |
| WARFARIN | 81-81-2 | 1 | 2.4 | E | 2.9 |  | 6.9 | E | 100 |  | 240 | E | 290 |  |  | 690 | E | 1,000 | 2,400 | E | 1,700 |  | 4,100 | E | 30 |
| XYLENES (TOTAL) | 1330-20-7 | 1,000 | 990 | E | 1,000 |  | 990 | E | 10,000 |  | 10,000 | C | 10,000 |  |  | 10,000 | C | 10,000 | 10,000 | C | 10,000 |  | 10,000 | C | NA |
| ZINEB | 12122-67-7 | 170 | 27 | E | 490 |  | 78 | E | 1,000 |  | 160 | E | 1,000 |  |  | 160 | E | 170 | 27 | E | 490 |  | 78 | E | NA |