- TITLE 25. ENVIRONMENTAL PROTECTION

- PART I.  DEPARTMENT OF ENVIRONMENTAL PROTECTION

- SUBPART D.  ENVIRONMENTAL HEALTH AND SAFETY

- ARTICLE VI.  GENERAL HEALTH AND SAFETY

- CHAPTER 250.  ADMINISTRATION OF LAND RECYCLING PROGRAM

- Subchapter A.  GENERAL PROVISIONS

- § 250.5. Public notice by applicant.

- * * * * *

- (f)  Documented proof of the receipt of municipal notices and the publication of newspaper notices that are required pursuant to sections 302(e), 303(h), 304(n), and 305(c) of the Act shall be submitted at the same time the NIR, plan, or report is submitted to the Department. The Department will disapprove any NIR, plan, or report that is submitted without the documented proof of the required notices.

- § 250.7. Fees.

- * * * * *

- (c)  The Department may waive the fee for resubmission of a plan or report if the resubmission is related to correcting minor administrative or technical deficiencies. The fee waiver is limited to one time for each plan or report to correct administrative deficiencies if the administrative deficiencies are corrected within 15 days of notice of the deficiencies from the Department, and one time for each plan or report to correct technical deficiencies if the technical deficiencies are corrected within 60 days of notice of the deficiencies from the Department.

- Subchapter C.  STATEWIDE HEALTH STANDARDS

- § 250.301. Scope.

- * * * * *

(b)  This subchapter sets forth generic Statewide health standards for regulated substances determined by the EPA to be mutagens. Tables 1—4 contain Statewide health standards based upon the methodology for mutagens in §§  250.306 and 250.307 (relating to ingestion numeric values; and inhalation numeric values) for the following substances classified as mutagens:

| Regulated Substance | CAS Number |
| --- | --- |
| Acrylamide Benzo[a]anthracene | 79-06-1 56-55-3 |
| Benzidine | 92-87-5 |
| Benzo[a]pyrene | 50-32-8 |
| Benzo[b]fluoranthene | 205-99-2 |
| Benzo[k]fluoranthene | 207-08-9 |
| Chromium (VI) Chrysene | 18540-29-9 218-01-9 |
| Dibenzo[a,h]anthracene | 53-70-3 |
| Dibromo-3-chloropropane, 1,2- | 96-12-8 |
| Dichloromethane Indeno[1,2,3-cd]pyrene | 75-09-2 193-39-5 |
| Methylene bis(2-chloroaniline), 4,4'- | 101-14-4 |
| Nitrosodiethylamine, N- | 55-18-5 |
| Nitrosodimethylamine, N- | 62-75-9 |
| Nitroso-N-ethylurea, N- | 759-73-9 |
| Trichloroethylene (TCE) | 79-01-6 |
| Trichloropropane, 1,2,3- Vinyl chloride | 96-18-4 75-01-4 |

* * * * *

- § 250.304. MSCs for groundwater.

* * * * *

(c)  The MSCs for regulated substances contained in groundwater in aquifers used or currently planned to be used for drinking water or for agricultural purposes is the MCL as established by the Department or the EPA [as established] in § 109.202 (relating to state MCLs, MRDLs and treatment technique requirements) and Health Advisory Levels (HALs) set forth in Drinking Water Standards and Health Advisories, EPA Office of Water Publication No. [EPA 822-R-09-011 (October, 2009)] EPA 822-S-12-001 (April, 2012). New or revised MCLs or HALs promulgated by the Department or the EPA shall become effective immediately for any demonstration of attainment completed after the date the new or revised MCLs or HALs become effective.

(1)  For a regulated substance where no MCL has been established, the MSC is the lifetime HAL [for that compound.] unless the HAL is not protective for use under this section.  A HAL is not protective for use under this section when both a lifetime HAL concentration and a 10-4 Cancer Risk concentration have been published, and the lifetime HAL concentration is higher than the 10-4 Cancer Risk concentration.

(2)  For a regulated substance where [neither an] no MCL [nor a lifetime HAL] has been established, and the HAL is not protective for use under this section, as described in paragraph (1), the MSC is the lowest concentration calculated using the appropriate residential and nonresidential exposure assumptions, and the equations in §§ 250.306 and 250.307 (relating to ingestion numeric values[;] and inhalation numeric values). [New or revised MCLs or HALs promulgated by the Department or the EPA shall become effective immediately for any demonstration of attainment completed after the date the new or revised MCLs or HALs become effective.]

* * * * *

(g)  The references referred to in subsection (f) are:

* * * * *

(12)  Yalkowsky, S.H. and R.M. Dannenfelser. 1992. Aquasol Database of Aqueous Solubility. Version 5.  College of Pharmacy, University of Arizona - Tucson, AZ. PC Version.

(13) Estimate from Log Kow.

(14)  Bennett, S.R., J.M. Bane, P.J. Benford, and R.L. Pyatt.1984. Environmental Hazards of Chemical Agent Simulants.  CRDC-TR-84055, Aberdeen Proving Ground, Md.

(15)  Munro, N.B. et al.  1999.  The Sources, Fate, and Toxicity of Chemical Warfare Agent Degradation Products. Environ. Health Perspect. 107(12): 933-4.

(16)  Monteil-Rivera, F., C. Groom, and J. Hawari. 2003. Sorption and Degradation of Octahydro-1,3,5,7-Tetranitro-1,3,5,7-Tetrazocine in Soil. Environ. Sci. Technol. 37:3878-3884.

(17)  Seidell, A. 1941. Solubilities of Organic Compounds.  New York, NY. D. Van Nostrand Co. Inc.

#### § 250.306. Ingestion numeric values.

#### * * * * *

(b)  For a regulated substance which is a carcinogen, the ingestion numeric value for that substance was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the following equations:

(1)  For regulated substances not identified as a mutagen in Section 250.301(b) (relating to scope):

MSC =      TR x ATc x 365 days/year

CSFo x Abs x EF x IFadj x CF

(2)  For regulated substances identified as a mutagen, except for vinyl chloride and trichloroethylene, in Section 250.301(b):

MSC =         TR x ATc x 365 days/year

CSFo x Abs x EF x AIFadj x CF

(3)  For vinyl chloride:

MSC =                                                                 TR

[CSFo x Abs x EF x IFadj x CF / (ATc x 365 days/year)] + (CSFo x Abs x IRc x CF/BWc)

(4)  For trichloroethylene:

MSC =                              TR x ATc x 365 days/yr

(CSFok x AIFadj + CSFol x IFadj) x Abs x EF x CF

(c)  For a regulated substance that has both an oral reference dose and an oral cancer slope factor, the ingestion numeric value is the lower of the two numbers as calculated by the equations in subsections (a) and (b).

(d)  The default exposure assumptions used to calculate the ingestion numeric values are as follows:

| Term |  | Residential |  | Nonresidential |
| --- | --- | --- | --- | --- |
|  |  | Systemic1 | Carcinogens2,6 | (Onsite Worker) |
| THQ | Target Hazard Quotient | 1 | N/A | 1 |
| RfDo | Oral Reference Dose (mg/kg-day) | Chemical-specific | N/A | Chemical-specific |
| BW | Body Weight (kg) |  | N/A |  |
|  | Soil | 15 |  | 70 |
|  | Groundwater | 70 |  | 70 |
| ATnc | Averaging Time for systemic toxicants (yr) |  |  |  |
|  | Soil | 6 | N/A | 25 |
|  | Groundwater | 30 | N/A | 25 |
| Abs | Absorption (unitless)3 | 1 | 1 | 1 |
| EF | Exposure Frequency (d/yr) |  |  |  |
|  | Soil | 250 | 250 | 180 |
|  | Groundwater | 350 | 350 | 250 |
| ED | Exposure Duration (yr) |  |  |  |
|  | Soil | 6 | N/A | 25 |
|  | Groundwater | 30 | N/A | 25 |
| IngR | Ingestion Rate |  |  |  |
|  | Soil (mg/day) | 100 | N/A | 50 |
|  | GW (L/day) | 2 | N/A | 1 |
| CF | Conversion Factor |  |  |  |
|  | Soil (kg/mg) | 1 x 10-6 | 1 x 10-6 | 1 x 10-6 |
|  | GW (unitless) | 1 | 1 | 1 |
| TR | Target Risk | N/A | 1 x 10-5 | 1 x 10-5 |
| CSFo | Oral Cancer Slope Factor (mg/kg-day)-1 | N/A | Chemical-specific | Chemical-specific |
| ATc | Averaging Time for carcinogens (yr) | N/A | 70 | 70 |
| IFadj4 | Ingestion Factor | N/A |  |  |
|  | Soil (mg-yr/kg-day) |  | 57.1 | 17.9 |
|  | GW (L-yr/kg day) |  | 1.1 | 0.4 |
| AIFadj5 | Combined Age-Dependent Adjustment Factor and Ingestion Factor | N/A |  | N/A |
|  | Soil (mg-yr/kg- day) |  | 245 |  |
|  | GW (L-yr/kg-day) |  | 3.39 |  |
| CSFok | TCE oral cancer slope factor for kidney cancer (mg/kg/day)-1 |  | 9.3 x 10-3 |  |
| CSFol | TCE oral cancer slope factor for non-Hodgkin lymphoma and liver cancer (mg/kg/day)-1 |  | 3.7 x 10-2 |  |

* * * * *

§ 250.307. Inhalation numeric values.

* * * * *

(b)  For a regulated substance which is a carcinogen, the following apply:

- For a volatile compound, the numeric value for inhalation from soil was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the following equation using TF for volatiles:

MSC =          TR x ATc x 365 days/year x 24 hr/day x TF

IUR x ET x EF x ED x CF

- For a regulated substance attached to particulates, the numeric value for inhalation from soil was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the equation in paragraph (1) using TF for particulates.

- For a regulated substance identified in Section 250.301(b) (relating to scope) as a mutagen, except for vinyl chloride and trichloroethylene, the numeric value for inhalation from soil was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the following equation using the TF for volatiles or particulates:

MSC =		 TR x ATc x 365 days/year x 24 hr/day x TF

IUR x ET x EF x AED x CF

- For vinyl chloride, the numeric value for inhalation from soil was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the following equation using the TF for volatiles or particulates:

MSC =                                                             TR

[IURxETxEFxEDxCF / (ATc x365days/yr x 24hr/d x TF)] + (IUR x CF x TF)

- For trichloroethylene, the numeric value for inhalation from soil was calculated using the appropriate residential or nonresidential exposure assumptions from subsection (d) according to the following equation using the TF for volatiles:

MSC =	TR x ATc x 365 days/yr x 24 hr/day x TF

(IURk x  AED + IURl x ED) x ET x EF x CF

(c)  For a regulated substance which is both a systemic toxicant and a carcinogen, the inhalation numeric value is the lower of the two numbers as calculated by the equations in subsections (a) and (b).

(d)  The default exposure assumptions used to calculate the inhalation numeric values for soil are as follows:

| Term |  | Residential |  | Nonresidential |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Systemic1 | Carcinogens2 | (Onsite Worker) |  |
| THQ | Target Hazard Quotient | 1 | N/A | 1 |  |
| RfCi | Inhal. Reference Concentration (mg/m3) | Chemical-specific | N/A | Chemical-specific |  |
| ATnc | Averaging Time for systemic toxicants (yr) | 30 | N/A | 25 |  |
| TF | Transport Factor (mg/kg)/(mg/m3) |  |  |  |  |
|  | Volatilization3 | Chemical-specific | Chemical-specific | Chemical-specific |  |
|  | Particulate4 | 1 x 1010 | 1 x 1010 | 1 x 1010 |  |
| ET | Exposure Time (hr/day) | 24 | 24 |  | 8 |
| EF | Exposure Frequency5 (d/yr) | 250 | 250 |  | 180 |
| ED | Exposure Duration (yr) | 30 | [N/A] 30 |  | 25 |
| CF | Conversion Factor | [1000 μg/mg] N/A | 1000 μg/mg |  | 1000 μg/mg |
| TR | Target Risk | N/A | 1 x 10-5 |  | 1 x 10-5 |
| IUR | Inhalation Unit Risk (μg/m3)-1 | N/A | Chemical-specific |  | Chemical-specific |
| ATc | Averaging Time for carcinogens (yr) | N/A | 70 |  | 70 |
| AED | Combined Age-Dependent Adjustment Factor and Exposure Duration (yr)6 | N/A | 76 |  | N/A |
| IURk | TCE inhalation unit risk for kidney cancer (ug/m3)-1 |  | 1 x 10-6 |  |  |
| IURl | TCE inhalation unit risk for both non-Hodgkin lymphoma and liver cancer (ug/m3)-1 |  | 3 x 10-6 |  |  |

* * * * *

(g)  For a regulated substance which is a carcinogen and is a volatile compound, the numeric value for the inhalation of volatiles from groundwater shall be calculated by using the appropriate residential or nonresidential exposure assumptions from subsection (h) according to the following equations:

- For regulated substances not identified as a mutagen in Section 250.301(b):

MSC =	    	   TR x ATc x 365 days/year

IUR x ET x EF x ED x TF x CF

- For regulated substances identified as a mutagen, except for vinyl chloride and trichloroethylene, in Section 250.301(b):

MSC = 	TR x ATc x 365 days/year x 24 hr/day

IUR x ET x EF x AED x TF x CF

- For vinyl chloride:

MSC =							TR

[(IUR x ET x EF x ED x TF x CF) / (ATc x 365 days/year x 24 hr/day)] +

(IUR x TF x CF)

- For trichloroethylene:

MSC =		TR x ATc x365 days/yr x 24 hr/day

(IURk x AED + IURl x ED) x ET x EF x TF x CF

(h)  The default exposure assumptions used to calculate the inhalation numeric values for the inhalation of volatiles from groundwater are as follows:

| Term |  | Residential |  | Nonresidential |
| --- | --- | --- | --- | --- |
|  |  | Systemic1 | Carcinogens2 | (Onsite Worker) |
| THQ | Target Hazard Quotient | 1 | N/A | 1 |
| RfCi | Inhal. Reference Concentration (mg/m3) | Chemical-specific | N/A | Chemical-specific |
| ATnc | Averaging Time for systemic toxicants (yr) | 30 | N/A | 25 |
| ET | Exposure Time (hr/day) | 24 | 24 | 8 |
| EF | Exposure Frequency (d/yr) | 350 | 350 | 250 |
| ED | Exposure Duration (yr) | 30 | 30 | 25 |
| TF | Transfer Factor (L/m3)3 | 0.5 | 0.5 | 0.5 |
| CF | Conversion Factor | N/A | 1,000 μg/mg | 1,000 μg/mg |
| TR | Target Risk | N/A | 1 x 10-5 | 1 x 10-5 |
| IUR | Inhalation Unit Risk (ug/m3)-1 | N/A | Chemical-specific | Chemical-specific |
| ATc | Averaging Time for carcinogens (yr) | N/A | 70 | 70 |
| AED | Combined Age-Dependent adjustment Factor and Exposure Duration (yr)4 | N/A | 76 | N/A |
| IURk | TCE inhalation unit risk for kidney cancer (ug/m3)-1 |  | 1 x 10-6 |  |
| IURl | TCE inhalation unit risk for both non-Hodgkin lymphoma and liver cancer ug/m3)-1 |  | 3 x 10-6 |  |

* * * * *