The United States Environmental Protection Agency (EPA) has established maximum contaminant levels (MCL) for the per- and polyfluoroalkyl substances (PFAS) compounds listed below. These MCLs will become the Statewide health standard medium-specific concentration (MSC) values for groundwater effective June 25, 2024. The new values are:

| Regulated Substance | CASRN | Used Aquifers |  |  |  |  |  |  |  | Nonuse Aquifers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TDS ≤ 2500 |  |  |  | TDS > 2500 |  |  |  |  |  |  |  |
|  |  | R |  | NR |  | R |  | NR |  | R |  | NR |  |
| HEXAFLUOROPROPYLENE OXIDE (HFPO) DIMER ACID (Gen-X)* | 13252-13-6 | 0.01 | M | 0.01 | M | 1 | M | 1 | M | 0.01 | M | 0.01 | M |
| PERFLUOROBUTANE SULFONIC ACID (PFBS) * | 375-73-5 | 2 | H | 2 | H | 200 | H | 20 | H | 2 | H | 2 | H |
| PERFLUOROHEXANE SULFONIC ACID (PFHxS) * | 108427-53-8 | 0.01 | M | 0.01 | M | 1 | M | 1 | M | 0.01 | M | 0.01 | M |
| PERFLUORONONAOIC ACID (PFNA) * | 72007-68-2 | 0.01 | M | 0.01 | M | 1 | M | 1 | M | 0.01 | M | 0.01 | M |
| PERFLUOROOCTANE SULFONATE (PFOS) | 1763-23-1 | 0.004 | M | 0.004 | M | 0.4 | M | 0.4 | M | 0.004 | M | 0.004 | M |
| PERFLUOROOCTANOIC ACID (PFOA) | 335-67-1 | 0.004 | M | 0.004 | M | 0.4 | M | 0.4 | M | 0.004 | M | 0.004 | M |

* In addition to meeting the individual MSC, if more than one of the marked compounds (Gen-X, PFBS, PFHxS, PFNA) are detected at any concentration in a sample, a Hazard Index (HI) must be calculated using the equation below. The HI MSC is met in this case by maintaining a rolling average HI of less than one for the most recent four quarters of samples utilizing the equation:

HI = CGen-X0.01+CPFBS2+CPFNA0.01+CPFHxS0.01

Where: All concentrations are in µg/L

CGen-X = concentration of Gen-X
CPFBS = concentration of PFBS
CPFNA = concentration of PFNA
CPFHxS = concentration of PFHxS

Updated: 7/10/2024