Commonwealth of Pennsylvania

State Board for Certification of Water and Wastewater Systems Operators

10/1/2005

Formulas, Conversions, and Common Scientific Units

ABC Formulas,

Conversions

& Abbreviations

DEP Dry and Liquid

##### Chemical Feed Diagrams

Unit Cancellation Steps

Units of Weight,

Volume,Time, Density,

Concentration & Flow

Formulas & Conversions

Formulas

The symbol ∏ = Pie  =  3.14

Alkalinity  =  (mL of Titrant) (Acid Normality) (50,000)

mL of Sample

Area of Circle  =  (.785) (Diameter2) or (∏) (Radius2)

Area of Cylinder Surface =  [(.785) (Diameter2)] + [(∏) (Diameter) (Height)]

Curved Surface Area of a Cylinder = 2 () (Radius) (Height)

End Surface areas of a Cylinder (both ends) = 2 () (Radius2)

Volume of a Cylinder  = () (Radius2) (Height)

Area of Rectangle  =  (Length) (Width)

Area of Triangle  =  (Base) (Height)

2

Circumference of Circle  =  (∏)  (Diameter) or (2) (∏) (Radius)

Chlorine Demand (lbs/day) = 
       lbs of Chlorine Fed/day – [(Chlorine Residual, mg/l) (Flow,  MGD) (8.34)]

Detention Time (minutes) = Volume of Tank (gallons)

Influent Flow (gpm)

Discharge  =  Volume

Time

Dosage, lbs/day  =  (mg/L) (8.34) (MGD)

Efficiency, %  =  (Mass In – Mass Out) (100%)    May substitute weight for mass

Mass In

Feed Rate, lbs/day =  (Plant Capacity, MGD) (Dosage, mg/L) (8.34 lbs/gal)

Filter Backwash rate =  Flow (gpm)

Filter surface area (ft2)

Food/Microorganism Ratio  =     Influent BOD, lbs/day

Aeration System MLVSS, lbs

Gallons/Capita/Day  =  Gallons Per Day

Population

Hardness  =  (mL of Titrant) (1,000)  (for 0.2 N EDTA)

mL of Sample

Horsepower (hp):
	theoretical hp = (Flow, gpm) (Total Water Head, ft)

3960

brake hp =    theoretical hp
                                  pump efficiency

Hydraulic Surface Loading Rate (gpd/ft2) = Flow Rate (gpd)
                                                                       Surface Area (ft2)

###### Loading rate (lbs/day) = (Concentration, mg/l) (Flow, MGD) (8.34)

###### Mean Cell Residence Time (MCRT):
                  (lbs of Suspended Solids in Aeration System)                     
                  (lbs of Suspended Solids Wasted/Day + lbs of Suspended Solids Lost in Effluent/Day

Organic Loading Rate  =  Organic Load, lbs BOD/day

Volume in 1000 ft3

Oxygen Uptake  =  Oxygen Usage (mg/L)

Time (min)

Population Equivalent  =  (Flow, MGD) (BOD, mg/L) (8.34 lbs/gal)

0.18 lbs BOD/day/person

Reduction in Flow, %  =  (Original Flow - Reduced Flow) (100%)

Original Flow

Slope  =  Drop or Rise

Distance

Sludge Volume Index  =  (Settleable Solids, %) (10,000)

MLSS, mg/L

Solids Applied (liquid), lbs/day  =  (Flow, MGD) (Concentration, mg/l) (8.34 lbs/gal)

Solids Loading, lbs/day/sq ft  =  Solids Applied, lbs/day

Surface Area, sq ft

Solids, mg/L  =  (Dry Solids, grams) (1,000,000)

mL of Sample

Surface Loading Rate (GPD/ft2) –   Flow Rate, GPD
                                                           Surface Area, ft2

Velocity  =  Flow  or  Distance

Area	Time

Volatile Solids, %  =  (Dry Solids – Ash Solids) (100%)

Dry Solids

Volume of Rectangular Tank (ft3)  =  (Length) (Width) (Height)

Volume of Cone (ft3)    =  (1/3) (.785) (Diameter2) (Height)

Volume of Cylinder (ft3)     =  (.785) (Diameter2) (Height) or

() (Radius2 ) (Height)

Waste Milliequivalents  =  (mL) (Normality)

Waste Normality  =  (Titrant Volume) (Titrant Normality)          Note:  Volumes are in same units

Sample Volume

Weir Overflow Rate  =         Flow (gpd)

Weir Length, (ft)

Conversion Factors:

| 1 acre = 43,560 square feet 1 cubic foot = 7.47 Gallons 1 foot = 0.305 meters 1 gallon = 3.79 liters 1 gallon = 8.34 pounds 1 grain per gallon = 17.1mg/L 1 mg/l = 1 ppm | 1 horsepower = 0.746 kilowatts 1 million gallons per day = 694 gallons per minute 1 pound = 0.454 kilograms 1 pound per square inch = 2.31 feet of water Degrees Celsius = (Degrees Fahrenheit – 32) (5/9) Degrees Farenheit = (Degrees Celsius) * 1.8 + 32 1 Ft of water column = 0.43 psi |
| --- | --- |

Abbreviations:

BOD		Biochemical Oxygen Demand

ft		feet

gpd		gallons per day

gpg		grains per gallon

gpm		gallons per minute

lbs		pounds

mg/L		milligrams per Liter

MGD		million gallons per day

mL		milliliter

MLSS		mixed liquor suspended solids

MLVSS	mixed liquor volatile suspended solids

ppm                parts per million

Chemical

Product

Product Feed

(lbs/day)

C

o

n

v

e

r

s

i

o

n

s

Plant Flow

(gal/min)

(x)

3.785

()

Plant Flow

(L/min)

Dry Chemical

Dosage

(lbs/gal)

Dry Chemical

Dosage

(mg/L)

(x)

315

()

Chemical

Product

Product Feed

(mg/min)

(x)

120,000

()

gal (Plant)

min

lbs (Dry)

gal (Plant)

L (Plant)

min

mg (Dry)

L (Plant)

mg (Dry)

min

lbs (Dry)

day

























CHEMICAL FEED CALCULATION DIAGRAM

DRY FEED

Procedure:  Fill in known data; put a question mark (?) for the value of the unknown data; convert all data to the units on the side where the (?) was placed and fill in the values; use unit cancellation to solve for the unknown.

English Units

Metric Units

C

o

n

v

e

r

s

i

o

n

s

CHEMICAL FEED CALCULATION DIAGRAM

LIQUID FEED

English Units

Metric Units

Chemical

Product

SG=Density (g/mL)

Chemical

Product

Density (lbs/gal)

g

mL (Product)

lbs

gal (Product)

(x)

(x)

Calculate

Product Strength

(          ) %

100

(          ) %

100

Chemical Product Strength

(g/mL)

Chemical Product Strength

(lbs/gal)

g (Dry)

mL (Product)

()

8.34

(x)

lbs (Dry)

gal (Product)

Product Feed

Flow (gal/hr)

(x)

63

()

Product Feed

Flow (mL/min)

mL (Product)

min

gal (Product)

hr

(x)

3.785

()

L (Plant)

min

gal (Plant)

min

Plant Flow

(gal/min)

Plant Flow

(L/min)

(x)

120,000

()

Dry Chemical

Dosage

(lbs/gal)

Dry Chemical

Dosage

(mg/L)

mg (Dry)

L (Plant)

lbs (Dry)

gal (Plant)

Procedure:  Fill in known data; put a question mark (?) for the value of the unknown data; convert all data to the units on the side where the (?) was placed and fill in the values; use unit cancellation to solve for the unknown.

# Unit Cancellation Steps

Step 1:	List unknown data as a question mark (?) including units, followed by an equals sign (=).

Sept 2:	Place data with same numerator unit to the right of the equal sign followed by a multiplication sign.

Step 3:	To cancel unwanted denominator unit, next place data with same numerator unit.

Step 4:	Continue to place data into equation to systematically cancel all unwanted units until only the unknown units remain.

Step 5:	Do the math (multiply all numerator values, multiply all denominator values; then divide numerator by the denominator.)

lbs

gal

Example:     ? lbs  =  1                x    1 g   x   3785 mL  =   8.34 lbs

gal     454 g	               mL	   1 	   	        gal

Helpful Hints:

Numerator

Denominator

Vertical format:  5 gal  =  5 gal

1

1 g  =  1000 mg	is written:	    1 g 	OR	1000 mg

1000 mg	  	   1 g

“per” means divided by:	Example:   5 gpm  =  5 gal

min

Inverting:	5 gal  =  1 min

min	     5 gal

| ## UNITS OF WEIGHT |  |
| --- | --- |
| - English | - Metric |
| - pound - lb | - gram - g - milligram - mg - kilogram - kg |
| - CONVERSIONS |  |
| - Metric/Metric | - Metric/English |
| - 1000 mg = 1 g or 1000 mg/g - 1000 gm = 1 kg or 1000 g/kg | - 1 lb = 454 g or 454 g/lb - 1 kg = 2.2 lbs or 2.2 lbs/kg |

| #### UNITS OF VOLUME |  |  |  |
| --- | --- | --- | --- |
| - English |  | - Metric |  |
| - gallon - gal - million gallon - Mgal - cubic feet - cu ft |  | - liter - L - milliliter - mL |  |
| - CONVERSIONS |  |  |  |
| - Metric/Metric | - Metric/English |  | - English/English |
| - 1000 mL = 1 liter - or 1000 mL/L | - 1 gal = 3.785 L or 3.785 L/gal - 1 gal = 3785 mL or 3785 mL/gal |  | - 7.48 gal = 1 cu ft or 7.48 gal/cu ft |

| - UNITS OF TIME |  |  |
| --- | --- | --- |
| - day - day - hour - hr | - minute - min - second - sec |  |
| - CONVERSIONS |  |  |
| - 1 day = 24 hr or 24 hr/day - 1 hr = 60 min or 60 min/hr |  | - 1 min = 60 sec or 60 sec/min - 1 day = 1440 min or 1440 min/day |

| - UNITS OF DENSITY |  |
| --- | --- |
| - English | - Metric |
| - lbs/gal - lbs/cu ft | - kg/L - g/mL |
| - THE DENSITY OF WATER |  |
| - English | - Metric/Metric |
| - 8.34 lbs/gal - 62.4 lbs/cu ft | - 1 kg/L - 1 g/mL |

| ### UNITS OF CONCENTRATION |  |
| --- | --- |
| - English | - Metric |
| - lbs/gal | - mg/L |
| - CONVERSION |  |
| - 1 lb/gal = 120,000 mg/L |  |

| - UNITS OF FLOW |  |
| --- | --- |
| - English | - Metric |
| - gallons per minute - gal/min - GPM - gallons per day - gal/day - GPD - million gallons per day - Mgal/day - MGD - cubic feet per second - cu ft/sec - CFS | - milliliters per minute - mL/min |
| - CONVERSIONS |  |
| - English/English | - English/Metric |
| - 1 MGD = 694 GPM or 694 GPM/MGD - 1 MGD = 1.55 CFS or 1.55 CFS/MGD | - 1 gal/day = 2.63 mL/min |